3GPP TSG-RAN WG4 Meeting #83												      R4- 1705622

Hangzhou, China, 15 - 19 May, 2017

Source: 	Huawei, Hisilicon

Title: 	Study TDD L-Band Tx BTS in the band 1427-1432MHz to protect EESS & propose a preferred TDD band plan.

Agenda Item:	9.6

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new TDD band covering 1427 – 1518 MHz.

At the last 3GPP RAN4 meeting#82-bis in Spokane, US, March 3 to 6 2017, it was decided to align the TDD frequency arrangement on the SDL frequency arrangement. And, in the contribution “R4-1704103”, it is proposed to study the possible uses of the 1427-1432MHz band.

Thus, this contribution studies the best options to use the 1427-1432MHz band and to protect the EESS in the 1400-1427MHz band.

# Discussion

## Background

The frequency band 1427-1518 MHz was identified for IMT in WRC-15, named as L-band.

Based on the approval at the 3GPP RAN4 meeting#82-bis in Spokane, US, March 3 to 6 2017, the TDD L-Band arrangement is aligned on the SDL L-Band arrangement. Thus, there is only a TDD L-band challenging arrangements, as follow:

Figure 1 - TDD Arrangement (a)

### The objective of SI or Core part WI or Testing part WI

The objectives of this work item is the following:

- Study the feasible channelling arrangement(s) of L-band (1427 – 1518 MHz) considering the above ITU options and relevant protection requirements of EESS below 1 427 MHz and MSS above 1518 MHz, considering the results of relevant ongoing studies, including:

  - TDD: 90 MHz (1427 – 1517 MHz), considering 1 MHz GB with MSS at the upper edge,

  - Note: the upper edge refinement is based on results of ongoing compatibility studies with MSS. For example option of 88 MHz (1427 – 1515 MHz) considers 3 MHz GB with MSS at the upper edge.

- Specify performance requirements for the feasible plan, band numbering and core requirements, as necessary.

- Support of 3, 5, 10, 15 and 20 MHz channel bandwidths

- Support of 200 KHz and 1.4 MHz for NB-IoT and eMTC, respectively, for development of Category M1 and NB1.

## Study proposed at the last 3GPP RAN4 meeting

At the last RAN4 meeting, there has been proposed and approved (see R4-1704103 [8]):

- Option 1: specify 1 band (1427-1518MHz or 1427-1517MHz) with restricted power in the first 5MHz block

Add a note in the specification:  for BS operating in the frequency range 1427-1432 MHz output power is restricted below the output maximum power declared by the manufacturer (which ensures that unwanted emission limit below 1427 MHz are met)

Provide data on max power allowed in the first 5MHz block

- Option 2: specify the following 2 operating bands

  - Band X: 1427-1432 MHz  (only for low power operations)

  - Band Y: 1432-1518 MHz or 1432-1517 MHz

- Option 3: specify 1 band (1432-1518 MHz or 1432-1517 MHz)

- Option 4: specify the following 2 operating bands

  - Band X: 1427-1518 MHz or 1427-1517 MHz (only for low power operations)

  - Band Y: 1432-1518 MHz or 1432-1517 MHz

Companies are encouraged to provide simulation results as soon as possible

## Study about the use of 1427-1432MHz band

The study is based on the need to improve the out of band emission. The table below provides the 4 key deployment scenarios

| Calculation | Macrocell | Microcell | Picocell | Femtocell |
| --- | --- | --- | --- | --- |
|  | (Wide area BS) | (Medium range BS) | (Local area BS) | (Home area BS) |
| EIRP for each antenna of a BTS => EIRP[dBm]/MHz => Case of carrier = 5MHz | 36.01029996 | 28.01029996 | 17.01029996 | 13.01029996 |
| EESS protection level Max power in the 1400-1427MHz band => -72dBw/27MHz => objective [=> -56.32dBm/MHz => Average] | -56.31363764 | -56.31363764 | -56.31363764 | -56.31363764 |
| ACLR at +-2.5MHz | 45 | 45 | 45 | 45 |
| Rejection at +-2.5MHz | -8.989700043 | -16.98970004 | -27.98970004 | -31.98970004 |
| Additional rejection at 2.5MHz But, we have only 2MHz guard band! | 47.3239376 | 39.3239376 | 28.3239376 | 24.3239376 |

Table 1: Calculation to define the additional rejection to protect EESS and to use the band 1427-1432MHz

Based on the additional rejection calculated in the table 1, we studied 3 options:

- Low EIRP in the 1427-1432MHz band,

- Use a part of the 1427-1432MHz

- A combination of the 2 first bullets

Option 1 “low EIRP” => without guard band at 1427MHz to protect the EESS, it is a challenge to comply to the EESS protection level (-72dBW/27MHz at the antenna connector). We assume that with an EIRP of about 0dBm we will be able to comply to the EESS protection level. But, an EIRP of 0dBm is considered as a too low EIRP (too small coverage), thus this option is not realist

Option 2 “use a part of the 1427-1432MHz band” => Taking into account the studies already provided about the filters of order 10 and 12 (see R4-1704103 [8]), we assume that the 1429-1432MHz band (3MHz) can be used with a normal EIRP e.g. macro site. The frequency separation of 2MHz, between 1427-1432MHz, is needed to get the EESS protection level.

Option 3 “A combination of option 1 & 2” => As only the option 2 is realist, this option 3 is not study.

## Option(s) about the band plan

Based on the previous study and the filter performances (see R4-1704103 [8]), the largest bandwidth will be the simplest option, thus, only a band plan between 1427MHz to 1517/8MHz is proposed:

Assuming that for the first block “1427-1432MHz”, only the 1429-1432MHz band can be used with a normal EIRP e.g. Macro cell.

As in region 2 and maybe in some other countries, there is no MSS use above 1518MHz, the 1MHz block “1517-1518MHz” could be used.

# Conclusion

In the case of the TDD arrangement, the use of the LBand can be:

Proposal 1: 1427-1432MHz band:

- Use the 1429-1432MHz band with a normal EIRP e.g. Macro cell

&

- Need a frequency separation of 2MHz (1427-1429MHz) between the IMT BTS and the EESS to get the EESS protection level defined in the Resolution 750.

Proposal 2: Based on the previous proposal 1 and on the filter performances (see R4-1704103 [8]), it is proposed to have only a frequency arrangement between 1427 to 1517/8MHz:

Assuming that for the first block “1427-1432MHz”, only the 1429-1432MHz will be used with a normal EIRP e.g. Macro cell.

As in region 2 and maybe in several countries, there is no MSS use just above 1518MHz, the 1MHz block “1517-1518MHz” could be used.

# References

[1] Radiocommunication Study Group, Document 5D/XX-E,  “Proposal for the revision of Recommendation ITU-R M.1036-5 to include frequency arrangement in the band 1427-1518 MHz”, June 2016.

[2] RP-162557 “New E-UTRAN L-band plan 1427-1518MHz”, Etisalat, Huawei, HiSilicon;

[3] ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1525 MHz”;

[4] Temp SWG C 009 Draft ECC Report on L-Band

[5] Temp SWG C 008 Draft CEPT Report on L-Band

[6] TR 36.745 V14.0.0, Feasibility study on global application of LTE Band 11 and of LTE Band 21 User Equipments (UEs) (Release 14)

[7] ITU-R Report M.2292 “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”

[8] R4-1704103 “Way forward on SDL band plan”