3GPP TSG-RAN WG4 Meeting #83	            R4-1705657

Hangzhou, China, 15-19 May 2017

Agenda item:	10.3.5

Source: 	Intel Corporation

Title: 	UE sub-6 GHz ACS considerations and proposals

Document for:	Discussion

# Introduction

UE ACS for sub-6 GHz was discussed in the last RAN4 meeting [1-2]. This document proposes methodology and initial values to define sub-6 GHz UE ACS for bandwidths wider than 20 MHz, which is the revision of [1].

# Discussion on sub-6 GHz ACS

Excerpt from [3]: Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a 5G signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the center frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s)

RAN4 NR should consider the following tables as baseline for defining sub-6 GHz UE ACS values for bandwidths of 50 MHz and 100 MHz. In tables 1 to 3, the bandwidth of interfering signal is kept to be the same as the bandwidth of wanted RX signal.

Table 1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| ACS | dB | 33.0 | 33.0 | 30 | 27 | 27 | 27 |

Table 2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  | 50 MHz |  | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |  |  |
| PInterferer | dBm | REFSENS +45.5dB |  | REFSENS +45.5dB |  | REFSENS +42.5dB |  | REFSENS +39.5dB |  | REFSENS +45.5dB | REFSENS +45.5dB |
| BWInterferer | MHz | 5 |  | 5 |  | 5 |  | 5 |  | 50 | 100 |
| FInterferer (offset) | MHz | 5+0.0025 / -5-0.0025 |  | 7.5+0.0075 / -7.5-0.0075 |  | 10+0.0125 / -10-0.0125 |  | 12.5+0.0025 / -12.5-0.0025 |  | 50+0.0025 / -50-0.0025 | 100+0.0125 / -100-0.0125 |

Table 3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 Hz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -56.5 | -56.5 | -53.5 | -50.5 | -56.5 | -56.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 50 | 100 |
| FInterferer (offset) | MHz | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 | 50+0.0025 / -50-0.0025 | 100+0.0125 / -100-0.0125 |

The alternative is to keep the same interfering signal bandwidth of 5MHz, which is the same as the interfering signal bandwidth for the case of 5/10/15/20 MHz, and lower the ACS values proportionally.

Table 4: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| ACS | dB | 33.0 | 33.0 | 30 | 27 | 19 | 13 |

Table 5: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  | 50 MHz |  | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |  |  |
| PInterferer | dBm | REFSENS +45.5dB |  | REFSENS +45.5dB |  | REFSENS +42.5dB |  | REFSENS +39.5dB |  | REFSENS +31.5dB | REFSENS +25.5dB |
| BWInterferer | MHz | 5 |  | 5 |  | 5 |  | 5 |  | 5 | 5 |
| FInterferer (offset) | MHz | 5+0.0025 / -5-0.0025 |  | 7.5+0.0075 / -7.5-0.0075 |  | 10+0.0125 / -10-0.0125 |  | 12.5+0.0025 / -12.5-0.0025 |  | 27.5+0.0025 / -27.5-0.0025 | 55.5+0.0075 / -55.5-0.0075 |

Table 6: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 Hz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -56.5 | -56.5 | -53.5 | -50.5 | -42.5 | -36.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 | 27.5+0.0025 / -27.5-0.0025 | 55.5+0.0075 / -55.5-0.0075 |

# Conclusion

Proposal:  RAN4 should study further the following 2 possibilities and agree on UE ACS definition for sub-6 GHz (50MHz and 100MHz).

- Table 1 to 3 in this document without relaxation on ACS and set the interfering signal bandwidth the same as the wanted signal bandwidth

- Table 4 to 6 in this document with relaxation on ACS and keep the interfering signal bandwidth of 5 MHz

# References

- R4-1703174: "UE sub-6 GHz ACS considerations and proposals", Intel Corporation

- R4-1703560, “Conducted UE ACS below for frequency range 1”, Nokia, Alcatel-Lucent Shanghai Bell

- 3GPP TS 36.101 V14.2.1 (2017-01)