3GPP TSG-RAN WG4 Meeting #83 												     R4- 1705766

Hangzhou, China, 15 - 19 May, 2017

Source: 	Etisalat, Huawei, Hisilicon

Title: 	TP for TR: A-MPR summary for MSS protection for TDD operation in L-band

Agenda Item:	9.6

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new TDD band covering 1427 – 1518 MHz.

This contribution is about the update of the TR to add the A-MPR studies to protect EESS in the band 1400-1427MHz.

This contribution is based on the study from the contribution R4-1705373 “A-MPR implementation to comply to “TDD L-Band UE emission level to protect MSS above 1518MHz””

# Discussion

## Background

The frequency band 1427-1518 MHz was identified for IMT in WRC-15, named as L-band.

Based on the approval at the 3GPP RAN4 meeting#82-bis in Spokane, US, March 3 to 6 2017, the TDD L-Band arrangement is aligned on the SDL L-Band arrangement. Thus, there is only a TDD L-band challenging arrangements, as follow:

Figure 1 - TDD Arrangement (a)

### The objective of SI or Core part WI or Testing part WI

The objectives of this work item is the following:

- Study the feasible channelling arrangement(s) of L-band (1427 – 1518 MHz) considering the above ITU options and relevant protection requirements of EESS below 1 427 MHz and MSS above 1518 MHz, considering the results of relevant ongoing studies, including:

  - TDD: 90 MHz (1427 – 1517 MHz), considering 1 MHz GB with MSS at the upper edge,

  - Note: the upper edge refinement is based on results of ongoing compatibility studies with MSS. For example option of 88 MHz (1427 – 1515 MHz) considers 3 MHz GB with MSS at the upper edge.

- Specify performance requirements for the feasible plan, band numbering and core requirements, as necessary.

- Support of 3, 5, 10, 15 and 20 MHz channel bandwidths

- Support of 200 KHz and 1.4 MHz for NB-IoT and eMTC, respectively, for development of Category M1 and NB1.

## TDD LTE => A-MPR study to protect MSS

According to the R4-1705373 “A-MPR implementation to comply to “TDD L-Band UE emission level to protect MSS above 1518MHz””, the text below is a summary of the A-MPR study

.

# Text proposal for TR

<Unchanged sections omitted>

# 2 References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Release specifications".

[3]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[4]	R4-1705373 “A-MPR implementation to comply to “TDD L-Band UE emission level to protect MSS above 1518MHz””

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

<Unchanged sections omitted>

# 5.2.2 	Tx UE - MSS protection

# 5.2.2-1 For First 3 MHz of TDD L-Band (1514 – 1517 MHz)

Figure 2.4-1 Simulation results for first 3 MHz at 1514-1517MHz [4]

Table 2.4-1: A-MPR scheme for the first 3 MHz case

| Parameters | Region A |  | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| RBstart | 0 – 3 |  | 4 – 7 | 8-10 | 11-14 |
| LCRB [RBs] | 8-11 | ≥ 12 | ≥ 8 | ≥ 3 | ≥ 1 |
| A-MPR [dB] | ≤ 3 | ≤ 5 | ≤ 6 | ≤ 3 | ≤ 3 |
| Note: RB allocations other than indicated in this table, UE is permitted to transmit power with A-MPR = 0. |  |  |  |  |  |

# 5.2.2-1 For First 5 MHz of TDD L-Band (1512 – 1517 MHz)

- First 5 MHz of TDD L-Band (1512 – 1517 MHz) to protect the MSS frequency range 1518-1559MHz. the MSS protection level is -30MHz/MHz with 1MHz separation frequency between IMT/LTE and MSS

Figure 2.4-3 shows the simulation result of power back off relative to 23dBm meeting the emission limit of -30dBm/MHz in 1512-1517MHz band.

Figure 2.4-3 Simulation results for first 5 MHz at 1512-1517MHz [4]

Table 2.4-2: A-MPR scheme for the first 5 MHz case

| Parameters | Region A |  | Region B |  | Region C |  | Region D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBstart | 0 – 5 |  | 6 – 11 |  | 12-16 |  | 17-24 |  |
| LCRB [RBs] | 12-16 | ≥ 17 | 9-12 | ≥ 13 | 5-8 | ≥ 9 | 1-6 | ≥ 7 |
| A-MPR [dB] | ≤ 5 | ≤ 9 | ≤ 6 | ≤ 10 | ≤ 4 | ≤ 9 | ≤ 4 | ≤ 6 |
| Note: RB allocations other than indicated in this table, UE is permitted to transmit power with A-MPR = 0. |  |  |  |  |  |  |  |  |

# 5.2.2-1 For First 10 MHz of TDD L-Band (1507 – 1517 MHz)

Figure 2.4-5 Simulation results for first 10 MHz at 1507-1517MHz [4]

Table 2.4-3: A-MPR scheme for the first 10 MHz case

| Parameters | Region A | Region B |  | Region C |  | Region D |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RBstart | 0 – 22 | 23 – 27 |  | 28-35 |  | 36-49 |  |
| LCRB [RBs] | ≥ 20 | 15-19 | ≥ 20 | 3-14 | ≥ 15 | 1-9 | ≥ 10 |
| A-MPR [dB] | ≤ 4 | ≤ 4 | ≤ 9 | ≤ 4 | ≤ 10 | ≤ 3 | ≤ 8 |
| Note: RB allocations other than indicated in this table, UE is permitted to transmit power with A-MPR = 0. |  |  |  |  |  |  |  |

# 5.2.2-1 For First 15 MHz of TDD L-Band (1502 – 1517 MHz)

Figure 2.4-7 Simulation results for first 15 MHz at 1502-1517MHz [4]

Table 2.4-4: A-MPR scheme for the first 15 MHz case

| Parameters | Region A | Region B |  | Region C |  | Region D |
| --- | --- | --- | --- | --- | --- | --- |
| RBstart | 0 – 22 | 23 – 48 |  | 49-60 |  | 61-74 |
| LCRB [RBs] | ≥ 36 | 9-23 | ≥ 24 | 1-14 | ≥ 15 | ≥ 1 |
| A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 5 | ≤ 3 | ≤ 9 | ≤ 4 |
| Note: RB allocations other than indicated in this table, UE is permitted to transmit power with A-MPR = 0. |  |  |  |  |  |  |

# 5.2.2-1 For First 20 MHz of TDD L-Band (1497 – 1517 MHz)

Figure 2.4-9 Simulation results for first 20 MHz at 1497-1517MHz [4]

Table 2.4-5: A-MPR scheme for the first 20 MHz case

| Parameters | Region A |  | Region B | Region C |  | Region D |  | Region E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 20 |  | 21 – 40 | 41 – 60 |  | 61 – 80 |  | 81 – 99 |  |
| LCRB [RBs] | 24-47 | ≥ 48 | ≥ 40 | 15-29 | ≥ 30 | 1-17 | ≥18 | 1-14 | ≥ 15 |
| A-MPR [dB] | ≤ 2 | ≤ 4 | ≤ 3 | ≤2 | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 3 | ≤ 6 |
| Note: RB allocations other than indicated in this table, UE is permitted to transmit power with A-MPR = 0. |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

# Conclusion

Based on the supported channel bandwidths and MSS protection defined in the R4-1705373 “A-MPR implementation to comply to “TDD L-Band UE emission level to protect MSS above 1518MHz””, we propose:

Proposal: Add the text as proposed in the paragraph “Text proposal for TR” in the TR [1]

.

# References

[1] RP-162557 “New E-UTRAN L-band plan 1427-1518MHz”, Etisalat, Huawei, HiSilicon

[2] R4-1704099_WF on UE and BS requirements in FDD L-band

[3] ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1525 MHz”;

[4] ITU-R Report M.2292 “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”

[5] R4-1704098- Band plan discussion 1 5 GHz TDD

[6] TR 36.892 - 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Introduction of 1447-1467MHz Band for TD-LTE in China

[7] R4-1703068 - UE reference sensitivity

[8] R4-1703518, “Discussion on L Band FDD EU emission to protect EESS” Huawei, Hisilicon

[9] R4-1703465 - TP for TR 36.751: A-MPR summary for EESS protection for FDD operation in L-band - NTT DOCOMO, INC., Nokia, Huawei