3GPP TSG-RAN WG4 Meeting #83	R4-1705855

# Hangzhou, China, 15th-19th May, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4669 | rev | 1 | Current version: | 13.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fixed Reference Channel clarification for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2017-05-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | IMCS was initial acronym used in RAN1 when specifying NB-IoT MSC, it’s not accurate anymore and might introduce confusion. “Tx Time” was for information and is redundant as it could be determined from number of repetition and number of resource unit. It’s so better to remove this line from NB-IoT FRCs as well to avoid any confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “IMCS” from “IMCS/TBS” line and “Tx Time” line from NB-IoT FRC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New reader might be confused with those lines. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A-14, A-15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.141 CR 1027 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

# A.14	Fixed Reference Channels for NB-IOT reference sensitivity (π/2 BPSK, R=1/3)

The parameters for the reference measurement channels are specified in Table A.14-1 for reference sensitivity and repetitions sensitivity.

Table A.14-1 FRC parameters for reference sensitivity, repetitions sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 | A14-3 | A14-4 |
| --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 | 3.75 |
| Number of tone | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 | TBD | TBD |
| ITBS | 0 | 0 | 0 | 0 |
| Payload size (bits) | 32 | 32 | 32 | 32 |
| Allocated resource unit | 2 | 2 | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 96 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |

# A.15	Fixed Reference Channels for NB-IoT dynamic range (π/4 QPSK, R=2/3)

The parameters for the reference measurement channels are specified in Table A.15-1 for NB-IoT dynamic range.

# Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| ITBS | 7 | 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |

<End of change>