3GPP TSG-RAN WG4 #83 Meeting                                        	R4-1705874

Hangzhou, P. R. China, 15th – 19th May, 2017

Agenda item:	7.7.2

Source: 	LG Electronics

Title: 	TP on summary MSD levels for xDL/2UL CA band combinations with self desense problems

Document for:	Approval

1	Introduction

This contribution provides summary MSD levels for new CA band combinations with self desense problems based on proposed MSD results from interested companies.

For 3DL/2UL CA band combinations, we analyse the impact to the 3rd own receiving band by the harmonics and IMD products from the dual uplink transmission.

For 4DL/2UL CA band combinations, we analyse the impact to the 3rd and 4th own receiving bands by the harmonics and IMD products from the dual uplink transmission.

For 5DL/2UL CA band combinations, we analyse the impact to the 3rd, 4th and 5th own receiving bands by the harmonics and IMD products from the dual uplink transmission.

Based on the self- desense analysis, we propose that RAN4 need to define the MSD requirements for only 3DL/2UL CA band combinations since there was no different self- desense problems between 3DL/2UL CA and other xDL/2UL CA.

**************** Start of the TP in subcluase 5.1.5 & othes of TR36.714-00-02 *******************

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes the CA band combinations with self-interference problems for CA_3DL/2UL CA.

Table 5.1.5-1: Summary of Self-interference analysis for 2UL/3DL CA

| downlink CA configuration | uplink CA Configuration | Harmonic relation to 3rd band without uplink | intermodulation to 3rd band without uplink | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| CA_1A-3A-40A | CA_1A-3A | - | 5th order | Yes | 8.0 Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-5A-40A | CA_1A-5A | - | 4th order | - | 9.0 |
| CA_1A-3A-7A | CA_1A-3A | - | - | Yes | N/A |
|  | CA_1A-7A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
|  | CA_3A-7A | - | - |  | Table 7.3.1A-0bA for CA_1A-3A [TS36.101] |
| CA_1A-7A-8A | CA_1A-7A | - | 5th order | - | No Need The IMD5 is fairly small and the overlapped region is quite small portion |
|  | CA_1A-8A | B8 3rd harmonic |  | - | Table 7.3.1A-0a for CA_1A-7A-8A [TS36.101] |
| CA_1A-8A-40A | CA_1A-8A | - | - | - | N/A |
| CA_1A-42C | CA_1A-42A | - | 4th order | - | No impact when consider a fixed Tx-Rx separation of 190MHz in Band 1 own Rx band. |
| CA_2A-4A-5A | CA_2A-4A | - | - | - | N/A |
| CA_2A-4A-29A | CA_2A-4A |  |  |  | N/A |
| CA_2A-12B | CA_2A-12A | - | - | - | N/A |
| CA_2A-12A-30A | CA_2A-12A | - | 4th order | - | 12.0 |
| CA_3A-5A-40A | CA_3A-5A | - | - | - | N/A |
| CA_3A-7A-8A | CA_3A-7A |  | 3rd order | - | 17.9 |
|  | CA_3A-8A | B8 3rd harmonic | 2nd & 3rd order | - | Table 7.3.1A-0a for CA_3A-7A-8A [TS36.101] 29.0 |
|  | CA_7A-8A | B8 2nd harmonics | - |  | Harmonics issues already completed in 2DL/1UL CA_3A-8A |
| CA_3A-8A-40A | CA_3A-8A | - | - | - | N/A |
| CA_3A-42C | CA_3A-42A | - | - | - | Studied in 2UL/2DL CA in Rel-14. N/A |
|  | CA_42C | - | - | - | N/A |
| CA_4A-12B | CA_4A-12A | - | - | - | N/A |
| CA_4A-12A-30A | CA_4A-12A | - | - | - | N/A |
| CA_7C-28A | CA_7C | - | - | - | N/A |
|  | CA_7A-28A | - | - | - | N/A |
| CA_19A-42C | CA_19A-42A | - | 4th order | - | Studied in 2UL/2DL CA in Rel-14. Consider specific spectrum of the operator N/A |
| CA_21A-42C | CA_21A-42A | - | - | - | N/A |
| CA_1A-5A-46A | CA_1A-5A | B1 3rd harmonic B5 7th harmonics | 4th & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A & CA_5A-46A. 1.4 |
| CA_1A-7A-46A | CA_1A-7A | B1 3rd harmonic | 4th & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_1A-46A. 4.4 |
| CA_5A-7A-46A | CA_5A-7A | B5 7th harmonics | 3rd & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_5A-46A. 12.5 |
| CA_1A-3A-21A | CA_1A-3A | - | 3rd & 5th order | Yes | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-21A | - | - | - | N/A |
|  | CA_3A-21A | - | 3rd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_1A-3A-42A | CA_1A-3A | B3 2nd harmonics | 4th order | Yes | - Harmonics issues already completed in 2DL/1UL CA_3A-42A. 13.3 |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | - | - | - | N/A |
| CA_1A-19A-42A | CA_1A-19A | - | 3rd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 5th order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-42A | - | - | - | N/A |
| CA_1A-21A-42A | CA_1A-21A | - | 2nd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_1A-42A | - | 2nd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | 2nd order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
| CA_3A-5A-7A | CA_3A-5A | - | 2nd & 3rd order | - | 30.0 for 2nd IMD 19.5 for 3rd IMD |
|  | CA_3A-7A | - | 3rd order | - | 19.2 |
|  | CA_5A-7A | - | - | - | N/A |
| CA_3A-19A-21A | CA_3A-19A | - | - | - | N/A |
|  | CA_3A-21A | - | 5th order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_19A-21A | - | 5th order | - | 4.0 |
| CA_3A-19A-42A | CA_3A-19A | B3 2nd harmonics | 3rd & 5th order | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_19A-42A | - | 3rd order | - | - No MSD since the overlapped region (0.1MHz) is quite small portion. |
| CA_3A-21A-42A | CA_3A-21A | B3 2nd harmonics | - | - | - Harmonics issues already completed in 2DL/1UL CA_3A-42A |
|  | CA_3A-42A | - | 5th order | - | - No MSD due to IMD to be specified according to actural spectrum holdings. |
|  | CA_21A-42A | - | - | - | N/A |
| CA_19A-21A-42A | CA_19A-21A | - | 4th order | - | 12.9 |
|  | CA_19A-42A | - | - | - | N/A |
|  | CA_21A-42A | - | 4th order | - | 12.5 |
| CA_1A-3C | CA_1A-3A | - | - | Yes | Table 7.3.1A-0bA for CA_1A-3A |
|  | CA_3C | - | - | - | N/A |
| CA_3C-8A | CA_3C | - | - | - | N/A |
|  | CA_3A-8A | B8 2nd harmonics | 4th & 5th order |  | Harmonics issues already completed in 2DL/1UL CA_3A-8A IMD issues already covered in 2UL/2DL CA_3A-8A |
| CA_2A-4A-7A | CA_2A-4A | - | - | - | N/A |
| CA_41A-42C | CA_41A-42A | - | - | - | N/A |
| CA_41C-42A | CA_41A-42A | - | - | - | N/A |
|  | CA_41C | - | - | - | N/A |
| CA_1A-3A-28A | CA_1A-3A | - | - | Yes | Table 7.3.1A-0bA for CA_1A-3A |
|  | CA_1A-28A | - | 5th order |  | 4.3 |
|  | CA_3A-28A | B28 3rd harmonics | 4th order |  | Harmonics issues already completed in 2DL/1UL CA_1A-28A 10.7 |
| CA_1A-21A-28A | CA_1A-21A | - | - | - | N/A |
|  | CA_1A-28A | B28 2nd harmonics | - | - | No MSD due to harmonics by considering of actural spectrum holdings. |
|  | CA_21A-28A | B28 3rd harmonics | 2nd & 3rd order | - | Harmonics issues already completed in 2DL/1UL CA_1A-28A No MSD due to IMD problems by considering of actural spectrum holdings. |
| CA_1A-28A-42A | CA_1A-28A | - | 3rd order | - | 15.4 |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_28A-42A | B28 3rd harmonics | 3rd order | - | Harmonics issues already completed in 2DL/1UL CA_1A-28A 11.3 |
| CA_3A-21A-28A | CA_3A-21A | - | 5th order | - | 2.9 |
|  | CA_3A-28A | B28 2nd harmonics | - | - | No MSD due to harmonics by considering of actural spectrum holdings. |
|  | CA_21A-28A | - | - | - | N/A |
| CA_3A-28A-42A | CA_3A-28A | B3 2nd harmonics | - | - | Harmonics issues already completed in 2DL/1UL CA_3A-42A |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_28A-42A | - | - | - | N/A |
| CA_21A-28A-42A | CA_21A-28A | - | 3rd & 4th order | - | No MSD due to IMD by considering of actural spectrum holdings. |
|  | CA_21A-42A | - | 4th order | - | No MSD due to IMD by considering of actural spectrum holdings. |
|  | CA_28A-42A | B28 2nd harmonics | - | - | No MSD due to harmonics by considering of actural spectrum holdings. |
| CA_28A-42C | CA_28A-42A | - | - | - | N/A |
|  | CA_42C | - | - | - | N/A |
| CA_1A-7A-7A | CA_1A-7A | - | - | - | N/A |
| CA_3A-7A-7A | CA_3A-7A | - | 4th order |  | 4th IMD was covered in 2DL/2UL CA_3A-7A |
| CA_5A-7A-7A | CA_5A-7A | - | 5th order | - | 5th IMD was covered in 2DL/2UL CA_5A-7A |
| CA_3A-41A-42A | 41A-42A | - | 3rd & 4th order | - | - 16.3 for 3rd IMD - 7.0 for 4th IMD |
| CA_28A-41A-42A | 41A-42A | - | 2nd & 5th order | - | - 25.8 for 2nd IMD - 0.8for 5th IMD |

<Unchanged sections are omitted>

Based on the MSD test configuration as summarized in Table 5.1.5.1, 5.1.5.2 & 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3DL/2UL inter-band CA with IMD problem as shown in Table 5.1.5.4.

Table 5.1.5.4: Proposed 3DL/2UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | 3rd DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |
| CA_1A-3A-40A | CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | TDD-FDD |
|  |  | 3 | 1735 | 5 | 25 | 1830 | 5 |  |  |
|  |  | 40 |  |  |  | 2380 | 5 | 8.0 |  |
| CA_1A-5A-40A | CA_1A-5A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | TDD-FDD |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5 |  |  |
|  |  | 40 |  |  |  | 2305 | 10 | 9.0 |  |
| CA_2A-12A-30A | CA_2A-12A | 2 | 1885 | 5 | 25 | 1965 | 5 | N/A | FDD |
|  |  | 12 | 708.5 | 5 | 25 | 738.5 | 5 |  |  |
|  |  | 30 | 2308 | 5 | 25 | 2353 | 5 | 12.0 |  |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | 18.0 |  |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | 29.0 |  |
| CA_1A-5A-46A | CA_1A-5A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | TDD-FDD |
|  |  | 5 | 830 | 5 | 25 | 875 | 5 |  |  |
|  |  | 46 |  |  |  | 5560 | 20 | 1.0 |  |
| CA_1A-7A-46A | CA_1A-7A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | TDD-FDD |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |
|  |  | 46 |  |  |  | 5580 | 20 | 4.0 |  |
| CA_5A-7A-46A | CA_5A-7A | 5 | 845 | 5 | 25 | 890 | 5 | N/A | TDD-FDD |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |
|  |  | 46 |  |  |  | 5865 | 20 | 13.0 |  |
| CA_1A-3A-42A | CA_1A-3A | 1 | 1922.5 | 5 | 25 | 2112.5 | 5 | N/A | TDD-FDD |
|  |  | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 |  |  |
|  |  | 42 |  |  |  | 3425 | 5 | 13.0 |  |
| CA_3A-5A-7A | CA_3A-5A | 3 | 1780 | 10 | 50 | 1875 | 10 | N/A | FDD |
|  |  | 5 | 845 | 5 | 25 | 890 | 5 |  |  |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 30.0 |  |
|  | CA_3A-7A | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | FDD |
|  |  | 7 | 2565 | 10 | 50 | 2685 | 10 |  |  |
|  |  | 5 | 840 | 5 | 25 | 885 | 5 | 19.0 |  |
| CA_3A-19A-21A | CA_19A-21A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | FDD |
|  |  | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 |  |  |
|  |  | 3 | 1774.6 | 5 | 25 | 1869.6 | 5 | 4.0 |  |
| CA_3A-19A-42A | CA_19A-42A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | TDD-FDD |
|  |  | 42 | 3517.5 | 5 | 25 | 3517.5 | 5 |  |  |
|  |  | 3 | 1767.4 | 5 | 25 | 1862.4 | 5 | 0 |  |
| CA_19A-21A-42A | CA_19A-21A | 19 | 842.5 | 5 | 25 | 887.5 | 5 | N/A | TDD-FDD |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | 5 |  |  |
|  |  | 42 | 3508.7 | 5 | 25 | 3508.7 | 5 | 13.0 |  |
|  | CA_21A-42A | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A | TDD-FDD |
|  |  | 42 | 3500 | 5 | 25 | 3500 | 5 |  |  |
|  |  | 19 | 836.2 | 5 | 25 | 881.2 | 5 | 13.0 |  |
| CA_1A-3A-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 |  |  |
|  |  | 3 | 1723.5 | 5 | 25 | 1818.5 | 5 | 4.0 |  |
|  | CA_3A-28A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 |  |  |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 |  |
| CA_1A-28A-42A | CA_1A-28A | 1 | 1955 | 5 | 25 | 2145 | 5 | N/A | TDD-FDD |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 |  |  |
|  |  | 42 | 3425 | 5 | 25 | 3425 | 5 | 15.0 |  |
|  | CA_28A-42A | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A | TDD-FDD |
|  |  | 42 | 3560 | 5 | 25 | 3560 | 5 |  |  |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 |  |
| CA_3A-21A-28A | CA_3A-21A | 3 | 1782 | 5 | 25 | 1877 | 5 | N/A | FDD |
|  |  | 21 | 1451 | 5 | 25 | 1499 | 5 |  |  |
|  |  | 28 | 734 | 5 | 25 | 789 | 5 | 3.0 |  |
| CA_3A-41A-42A | CA_41A-42A | 41 | 2640 | 10 | 50 | 2640 | 10 | N/A | FDD-TDD |
|  |  | 42 | 3425 | 10 | 50 | 3425 | 10 |  |  |
|  |  | 3 | 1760 | 5 | 25 | 1855 | 5 | [16.0] |  |
| CA_28A-41A-42A | CA_41A-42A | 41 | 2672 | 10 | 50 | 2672 | 10 | N/A | FDD-TDD |
|  |  | 42 | 3460 | 10 | 50 | 3460 | 10 |  |  |
|  |  | 28 | 733 | 5 | 25 | 788 | 5 | [26.0] |  |

The required MSD levels are derived by taking average values of proposed MSD levels from interested companies. If there are outliers, the sources for differences will be discussed and applicability for averaging considered case by case.

<Unchanged sections are omitted>

6.12	LTE Advanced Carrier Aggregation: Band 3 and Band 7 and Band 8 with 2 ULs

6.12.1	List of specific combination issues

6.12.1.1	Channel bandwidths per operating band for CA

Table 6.12.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A-7A-8A | CA_3A-7A or CA_3A-8A or CA_7A-8A | 3 |  |  | Yes | Yes | Yes |  | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 2 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |

6.12.1.2	Co-existence studies for LTE-A UL CA_3A-7A and DL CA_3A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.12.1.2-1

Table 6.12.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 715 | 860 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 850 | 1070 | 3215 | 3430 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5920 | 6140 | 6710 | 6925 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2560 | 2855 | 5715 | 6000 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7630 | 7925 | 9210 | 9495 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1720 | 1430 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8570 | 8215 | 4640 | 4270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11710 | 12065 | 9340 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4290 | 3930 | 355 | 10 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10920 | 11280 | 10130 | 10495 |

The 4th IMD fall into the own Rx frequency of Band 7. But this impact already covered in 2UL/2DL CA in Rel-12.

The 3rd IMD will interfere own Rx frequency of Band 8. It will be analysed to slove the self- interference problem.

6.12.1.3	Co-existence studies for LTE-A UL CA_3A-8A and DL CA_3A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.12.1.3-1

Table 6.12.1.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The 2nd harmonics of Band 8, 4th & 5th IMDs fall into the own Rx frequency of Band 3. And also the 4th & 5th IMDs fall into the own Rx frequency of Band 8. But these impacts already covered in 2UL/2DL CA in Rel-12 using operator specific own spectrum allocation in this example is 1735 - 1755 / 1830 - 1850 MHz in Band 3 and 905 - 915 / 950 - 960 MHz in Band 8. Hence only 4th IMD problem in Band 3 has covered in rel-12 specification.

And also, the 3rd harmonics of Band 8 and 2nd &3rd IMDs will interfere own Rx frequency of Band 7. It will be analysed to slove the self- interference problem.

6.12.1.4	Co-existence studies for LTE-A UL CA_7A-8A and DL CA_3A-7A-8A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.12.1.4-1

Table 6.12.1.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 670 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 70 | 245 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3380 | 3170 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9400 | 9085 | 1160 | 950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5670 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

The 2nd harmonics of Band 8 fall into the own Rx frequency of Band 3. But these impacts already covered in 2DL/1UL CA_3A-8A. Also the 3rd harmonics of Band 8 impacted to the own Rx frequency of Band 7. Also the 5th IMD fall into the own Rx frequency of Band 8. However this impact already covered in 2DL/2UL CA_7A-8A. Hence there was no self-interference issues.

6.12.1.5	MSD

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-7A-8A there is interference components from 2 uplink operation which would interfere the downlink of the Band 8.

When uplink CA configurations CA_3A-8A is paired with downlink CA configuration CA_3A-7A-8A there is interference components from 2 uplink operation which would interfere the downlink of the Band 7.

And also when uplink CA configurations CA_7A-8A is paired with downlink CA configuration CA_3A-7A-8A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

Figure 6.12.1.5-1shows the RF front-end architecture for the MSD analysis. Another alternative would be common triplexer based architecture. There is a harmonic trap filter assumed in B8 path for this CA case because harmonic trap was assumed for B8+B7 1UL/2DL CA and for B3+B7+B8 1UL/3DL CA.

Figure 6.12.1.5-1 Common diplexer architecture for CA_3A-7A-8A 3DL with CA_3A-7A and CA_3A-8A 2UL CA

Table 6.12.1.5-1shows the MSD analysis for IMD3 interference landing on band 8 DL in CA_3A-7A-8A 3DL with CA_3A-7A 2UL CA.

Table 6.12.1.5-1: MSD analysis for IMD3 of CA_3A-7A-8A 3DL with CA_3A-7A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 0.62 | 21.6 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |
| R4-165757 by LGE | B3 | IMD3 | 2*fB3 - fB7 | 1735 | 5 | 25 | B8 | 940.0 | 5 | 1.2 | 14.1 |
|  | B7 |  |  | 2530 | 10 | 50 |  |  |  |  |  |

Table 6.12.1.5-2, 6.12.1.5-3, 6.12.1.5-4 are the MSD analyses for IMD2, IMD3 and the overlapped IMD2+IMD3 interference landing on band 7 DL in CA_3A-7A-8A 3DL with CA_3A-8A 2UL CA. Note that for the overlapped IMD2+IMD3 case, the B8 third harmonic will also fall on top of the B7 DL block, but the impact of the third harmonic is negligible because it is more than 20dB below the MSD due to IMD2 and IMD3. Thus “H3” is not included in the MSD table.

Table 6.12.1.5-2: MSD analysis for IMD2 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 32.0 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |
| R4-165757 by LGE | B3 | IMD2 | fB3 + fB8 | 1735 | 5 | 25 | B7 | 2635.0 | 10 | 1.0 | 25.1 |
|  | B8 |  |  | 900 | 5 | 25 |  |  |  |  |  |

Table 6.12.1.5-3: MSD analysis for IMD3 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 0 | 22.1 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |
| R4-165757 by LGE | B3 | IMD3 | 2*fB3 - fB8 | 1772.5 | 5 | 25 | B7 | 2635.0 | 10 | 1.8 | 12.9 |
|  | B8 |  |  | 910 | 5 | 25 |  |  |  |  |  |

Table 6.12.1.5-4: MSD analysis for overlapped IMD2+IMD3 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL

| Reference | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | CF [dB] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-168860 by Huawei, Hisilicon | B3 | IMD2+ IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1780 | 5 | 25 | B7 | 2670 | 10 | 0 | 32.5 |
|  | B8 |  |  | 890 | 5 | 25 |  |  |  |  |  |

For this combination, there exists an additional scenario where IMD2 and IMD3 from B3+B8 both hit on top of B7 DL but they do not overlap. The MSD analyses are shown in table 6.12.1.5-5. Note that this is an example calculation with UL RB allocations wider than in respective single band RFESENS and these IMD’s don’t include CF’s that will be fairly significant in this case.

Table 6.12.1.5-5: MSD analysis for non-overlapped IMD2+IMD3 of CA_3A-7A-8A 3DL with CA_3A-8A 2UL from R4-168800 (proposed by Huawei, Hisilicon)

| DL CA | UL CA | IMD |  | UL Fc [MHz] | UL BW [MHz] | UL [RB] | DL Band | IMD center [MHz] | DL BW [MHz] | MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3+7+8 | B3 | IMD2 | fB3 + fB8 | 1762.5 | 15 | 75 | B7 | 2650.0 | 20 | 29.0 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB8 | 1762.5 | 15 | 75 | B7 | 2650.0 | 20 | 19.2 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |
|  | B3 | IMD2+ IMD3 | fB3 + fB8, 2*fB3 - fB8 | 1762.5 | 15 | 75 | B7 | 2650 | 20 | 29.5 |
|  | B8 |  |  | 900 | 10 | 50 |  |  |  |  |

When considering which test point should be defined in the standard, it was agreed to define the IMD3 case for the CA_3A-7A-8A 3DL paired with CA_3A-7A 2UL CA, and define the overlapped IMD2 + IMD3 case for the CA_3A-7A-8A 3DL paired with CA_3A-8A 2UL CA. Table 6.12.1.5-6 shows the merged MSD result from the interested companies, and table 6.12.1.5-7 is the MSD summery for this combination.

Table 6.12.1.5-6: merged MSD results from interested companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Avg. MSD [dB] | Proposed MSD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B8 | B3 | IMD3 | 2*fB3 - fB7 | 14.1 | 21.6 | 17.9 | 18.0 |
|  | B7 |  |  |  |  |  |  |
|  | B3 | IMD2+IMD3 | fB3 + fB8, 2*fB3 - fB8 | 25.5 | 32.5 | 29.0 | 29.0 |
|  | B8 |  |  |  |  |  |  |

Table 6.12.1.5-7: MSD summary for CA_3A-7A-8A 3DL/2UL CA

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | 3rd DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | 18.0 |  |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | 29.0 |  |

6.12.1.6	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

<Unchanged sections are omitted>

6.48	LTE Advanced Carrier Aggregation: Band 3 and Band 41 and Band 42 with 2 ULs

6.48.1	List of specific combination issues

6.48.1.1	Channel bandwidths per operating band for CA

Table 6.48.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_3A_41A-42A | CA_41A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

6.48.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_3A_41A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.48.1.2-1

Table 6.48.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 3rd & 4th IMDs impact to the 3rd own Rx frequency of Band 3.

It will be analysed to slove the self- interference problem.

6.48.1.3	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_3A_41A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 3.

The merged MSD level was provided from interested companies as below Table 6.48.1.3-1.

Table 6.48.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Qualcomm | Avg. MSD [dB] | Proposed MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B41+B42 | B41 | IMD3 | ¦2*fB41 -fB42¦ | 15.8 | 16.8 | 16.3 | 16.0 |
|  | B42 |  |  |  |  |  |  |
|  | B41 | IMD4 | ¦2*fB42-2*f41¦ | 5.8 | 8.2 | 7.0 | 7.0 |
|  | B42 |  |  |  |  |  |  |

6.48.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

6.49	LTE Advanced Carrier Aggregation: Band 28 and Band 41 and Band 42 with 2 ULs

6.49.1	List of specific combination issues

6.49.1.1	Channel bandwidths per operating band for CA

Table 6.49.1.1-1: CA configurations under study

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_28A-41A-42A | CA_41A-42A | 28 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |

6.49.1.2	Co-existence studies for LTE-A UL CA_41A-42A and DL CA_28A_41A-42A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.49.1.2-1

Table 6.49.1.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3400 | 3600 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6800 | 7200 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 10200 | 10800 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 710 | 1104 | 5896 | 6290 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1392 | 1980 | 4110 | 4704 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8392 | 8980 | 9296 | 9890 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3888 | 4670 | 7510 | 8304 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10888 | 11670 | 12696 | 13490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2208 | 1420 | 11792 | 12580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11904 | 10910 | 7360 | 6384 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 16096 | 17090 | 13384 | 14360 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5808 | 4820 | 1270 | 288 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15192 | 16180 | 14288 | 15270 |

From the harmonics/IMDs analysis table, there were 2nd & 5th IMDs impact to the 3rd own Rx frequency of Band 28.

It will be analysed to slove the self- interference problem.

6.49.1.3	MSD

When uplink CA configurations CA_41A-42A is paired with downlink CA configuration CA_28A_41A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 28.

The merged MSD level was provided from interested companies as below Table 6.49.1.3-1.

Table 6.49.1.3-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Qualcomm | Avg. MSD [dB] | Proposed MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B28+B41+B42 | B41 | IMD2 | ¦fB42 -fB41¦ | 29.3 | 22.2 | 25.8 | [26.0] |
|  | B42 |  |  |  |  |  |  |
|  | B41 | IMD5 | ¦2*fB42 -3*fB41¦ | 0.4 | 1.2 | 0.8 | [1.0] |
|  | B42 |  |  |  |  |  |  |

6.49.1.4	∆TIB and ∆RIB values

[Editor Note] It will be updated in future

<End of TP>