3GPP TSG-RAN4 Meeting #83	R4-1705921

# Hangzhou, China, 15-19 May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.977 | CR | 0060 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Channel Model Validation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETS-Lindgren Europe, MVG Industries |  |  |  |  |  |  |  |  |  |
| XSource to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-MIMO_OTA |  |  |  |  |  | Date: |  |  | 2017-03-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | To add system providers’ SCME UMi validation results |  |  |  |  |  |  |  |  |
| Summary of change: |  | Validation results for PDP, Temporal correlation, Spatial correlation, and cross polarization are presented |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR requirements won’t be fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2 to 8.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged content omitted >>

## 8.4	Channel Model validation results

### 8.4.1 	Scope

Clauses 8.4.2-6 contain the validation results of channel models defined in Clause 8.2 for companies using methods as described in Clauses 6.3.1.1 , 6.3.1.2 and 6.3.1.3.  These results are based on three different types of channel emulators and setup vendors, and all three sets of results are included here for comparison.

### 8.4.2	Power Delay Profile (PDP)

# The power delay profiles of the SCME UMi channel model specified in Clause 8.2 have been measured according to the procedures in 8.3.2.1.  Figure 8.4.1-1 below illustrates the measured results for Band 13, and Band 7.

# (a)  (b)

# (c)  (d)

# (e)  (f)

Figure 8.4.2-1: For MPAC system 1, CE 1 (a) and CE 2 (b) PDP verification measurement for Band 13; for MPAC system 1, CE 1 (c) CE 2 (d) PDP verification measurement for Band 7; For MPAC system 2, PDP verification measurements for Band 13 (e) and Band 7 (f).

Table 8.4.2-1 below summarizes the PDP verification results.

# Table 8.4.2-1: Summary of PDP verification results at Band 13, and Band 7 from MPAC system providers

| MPAC System 1 and CE 1 Band 13 |  |  |  |  |  |  |  |  |  |  | MPAC System 1 and CE 2 Band 13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cluster | Delay (ns) |  |  |  | Power (dB) |  |  |  | Delay (ns) |  |  |  |  |  |  | Power (dB) |  |  |  |
|  | Theory | Meas | Delta | Theory | Meas | Delta | Theory | Meas | Delta | Theory | Meas | Delta |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 205 | 201 | 4 | -2.70 | -3.00 | 0.30 | 205 | 204 | 1 | -2.70 | -3.06 | 0.36 |
| 3 | 285 | 280 | 5 | -1.30 | -1.10 | 0.20 | 285 | 284 | 1 | -1.30 | -1.25 | 0.05 |
| 4 | 660 | 659 | 1 | -4.30 | -4.26 | 0.04 | 660 | 660 | 0 | -4.30 | -4.39 | 0.09 |
| 5 | 805 | 800 | 5 | -6.00 | -6.00 | 0.00 | 805 | 804 | 1 | -6.00 | -5.83 | 0.17 |
| 6 | 925 | 920 | 5 | -8.40 | -8.58 | 0.18 | 925 | 925 | 0 | -8.40 | -8.39 | 0.01 |
| MPAC System 1 and CE 1 Band 7 |  |  |  |  |  |  |  |  |  |  | MPAC System 1 and CE 2 Band 7 |  |  |  |  |  |  |  |
| Cluster | Delay (ns) |  |  |  | Power (dB) |  |  |  | Delay (ns) |  |  |  |  | Power (dB) |  |  |  |  |  |
|  | Theory | Meas | Delta | Theory |  | Meas | Delta |  |  | Theory |  | Meas | Delta |  | Theory |  | Meas | Delta |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| 2 | 205 | 200 | 5 | -2.70 | -2.94 | 0.24 | 205 | 201 | 4 | -2.70 | -3.17 | 0.47 |
| 3 | 285 | 285 | 0 | -1.02 | -1.12 | 0.10 | 285 | 287 | 2 | -1.30 | -1.18 | 0.12 |
| 4 | 660 | 656 | 4 | -4.30 | -4.59 | 0.29 | 660 | 658 | 2 | -4.30 | -4.42 | 0.12 |
| 5 | 805 | 805 | 0 | -6.00 | -6.01 | 0.01 | 805 | 807 | 2 | -6.00 | -6.01 | 0.01 |
| 6 | 925 | 923 | 2 | -8.40 |  | -8.43 |  | 0.03 |  | 925 |  | 926 | 1 |  | -8.40 |  | -8.68 | 0.28 |  |
| MPAC System 2 and CE 1 Band 7 |  |  |  |  |  |  |  |  |  |  | MPAC System 2 and CE 1 Band 7 |  |  |  |  |  |  |  |
| Cluster | Delay (ns) |  |  |  | Power (dB) |  |  |  | Delay (ns) |  |  |  |  | Power (dB) |  |  |  |  |  |
|  | Theory | Meas | Delta | Theory |  | Meas | Delta |  |  | Theory |  | Meas | Delta |  | Theory |  | Meas | Delta |  |
| 1 | 0 | 0 | 0 | 0 |  | 0 | 0 |  |  | 0 |  | 0 | 0 |  | 0 |  | 0 | 0 |  |
| 2 | 205 | 208 | 3 | -2.70 |  | -2.83 | 0.13 |  |  | 205 |  | 200 | 5 |  | -2.70 |  | -2.81 | 0.11 |  |
| 3 | 285 | 290 | 5 | -1.30 |  | -1.29 | 0.00 |  |  | 285 |  | 289 | 4 |  | -1.30 |  | -1.23 | 0.07 |  |
| 4 | 660 | 668 | 8 | -4.30 |  | -4.41 | 0.11 |  |  | 660 |  | 665 | 5 |  | -4.30 |  | -4.44 | 0.14 |  |
| 5 | 805 | 812 | 7 | -6.00 |  | -5.73 | 0.27 |  |  | 805 |  | 810 | 5 |  | -6.00 |  | -5.85 | 0.15 |  |
| 6 | 925 | 928 | 3 | -8.40 |  | -8.29 | 0.11 |  |  | 925 |  | 930 | 5 |  | -8.40 |  | -8.27 | 0.13 |  |

The results for the RTS method with geometric implementation are shown in Figures 8.4.2-2:

Figure 8.4.2-2: For Band 13, SCMe UMa and UMi PDP verification measurement
 for the RTS method with geometric implementation

The results for the RTS method with correlation implementation (Jake’s Doppler spectrum) are shown in Figures 8.4.2-3:

Figure 8.4.2-3: For Band 13, SCMe UMa and UMi PDP verification measurement
 for the RTS method with correlation implementation (Jake's Doppler spectrum)

The summarized results can be found in Tables 8.4.2-2 and 8.4.2-3:

Table 8.4.2-2: Summary of PDP verification results at Band 13 for the RTS method with geometric implementation

| SCME Urban Macro |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cluster | Delay (ns) |  |  | Power (dB) |  |  |
|  | Theory | Measured | Delta (ns) | Theory | Measured | Delta (dB) |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 255 | 256 | 1 | -1.7 | -2 | -0.3 |
| 3 | 360 | 360 | 0 | -2.2 | -2.2 | 0 |
| 4 | 1040 | 1040 | 0 | -5.2 | -5.2 | 0 |
| 5 | 2730 | 2728 | -2 | -9.1 | -9.4 | -0.3 |
| 6 | 4600 | 4600 | 0 | -12.5 | -12.5 | 0 |
| SCME Urban Micro |  |  |  |  |  |  |
| Cluster | Delay (ns) |  |  | Power (dB) |  |  |
|  | Theory | Measured | Delta (ns) | Theory | Measured | Delta (dB) |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 205 | 208 | 3 | -2.7 | -2.6 | 0.1 |
| 3 | 285 | 288 | 3 | -1.3 | -1.2 | 0.1 |
| 4 | 660 | 664 | 4 | -4.3 | -4.5 | -0.2 |
| 5 | 805 | 808 | 3 | -6 | -6.1 | -0.1 |
| 6 | 925 | 928 | 3 | -8.4 | -8.3 | 0.1 |

Table 8.4.2-3: Summary of PDP verification results at Band 13 for the RTS method  with correlation implementation (Jake's Doppler spectrum)

| SCME Urban Macro |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cluster | Delay (ns) |  |  | Power (dB) |  |  |
|  | Theory | Measured | Delta (ns) | Theory | Measured | Delta (dB) |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 255 | 256 | 1 | -1.7 | -1.6 | 0.1 |
| 3 | 360 | 360 | 0 | -2.2 | -2.1 | 0.1 |
| 4 | 1040 | 1040 | 0 | -5.2 | -5.2 | 0 |
| 5 | 2730 | 2728 | -2 | -9.1 | -9.7 | -0.6 |
| 6 | 4600 | 4600 | 0 | -12.5 | -12.3 | 0.2 |
| SCME Urban Micro |  |  |  |  |  |  |
| Cluster | Delay (ns) |  |  | Power (dB) |  |  |
|  | Theory | Measured | Delta (ns) | Theory | Measured | Delta (dB) |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 205 | 208 | 3 | -2.7 | -2.9 | 0.2 |
| 3 | 285 | 288 | 3 | -1.3 | -1.4 | -0.1 |
| 4 | 660 | 664 | 4 | -4.3 | -4.9 | -0.6 |
| 5 | 805 | 808 | 3 | -6 | -6 | 0 |
| 6 | 925 | 928 | 3 | -8.4 | -8.3 | 0.1 |

### 8.4.3.	Doppler / Temporal Correlation

The Doppler spread and temporal correlation of the SCME UMi channel model defined in clause 8.2 has been characterized according to clause 8.3.2.2.  Figure 8.4.3-1 below illustrates the measured results for Band 13 and Band 7.

# (a)  (b)

# (c)  (d)

# (e)  (f)

# Figure 8.4.3-1: For MPAC system 1, CE 1 (a) and CE 2 (b) temporal correlation measurements for Band 13; for MPAC system 1, CE 1 (c) CE 2 (d) temporal correlation measurements for Band 7; For MPAC system 2, temporal correlation measurements for Band 13 (e) and Band 7 (f)

# Figure 8.4.3-2: Void

The temporal correlation results for the RTS method using the geometric implementation are shown in Figure 8.4.3-3:

Figure 8.4.3-3: Band 13 temporal correlation measurements of SCMe UMa and UMi
 for the RTS method with geometric implementation

The temporal correlation results for the radiated two-stage method using correlation implementation of SCME (Jake's Doppler spectrum) are shown in Figure 8.4.3-4:

Figure 8.4.3-4: Band 13 Temporal correlation measurements for the radiated two-stage method using correlation implementation of SCME UMa and UMi with with correlation implementation (Jake's Doppler spectrum)

### 8.4.4	Spatial correlation

The spatial correlation properties of the SCME UMi channel model defined in clause 8.2 have been characterized according to clause 8.3.2.3. Figure 8.4.4-1 below illustrates the measured results for Band 13 and Band 7.

# (a)  (b)

# (c)  (d)

# (e)  (f)

# Figure 8.4.4-1: For MPAC system 1, CE 1 (a) and CE 2 (b) spatial correlation measurements for Band 13; for MPAC system 1, CE 1 (c) CE 2 (d) spatial correlation measurements for Band 7; For MPAC system 2, spatial correlation measurements for Band 13 (e) and Band 7 (f)

Figure 8.4.4-2: Void

The spatial correlation results for the RTS method using correlation implementation of SCME (Jake's Doppler spectrum) are shown in Figure 8.4.4-3.

Figure 8.4.4-3: Band 13 spatial correlation measurements for the radiated two-stage method using correlation implementation of SCME UMa and UMi (Jake's Doppler spectrum)

The spatial correlation results for the RTS method using geometric implementation of SCME are shown in Figure 8.4.4-4.

Figure 8.4.4-4: Band 13 spatial correlation measurements for the RTS method using geometric implementation of SCME UMa and UMi

### 8.4.5	Cross polarization

The cross polarization properties of the SCME UMi channel model defined in clause 8.2 have been characterized according to clause 8.3.2.3.  The measured results shown in Table 8.4.5-1 below are reported considering the antenna gain difference of the reference antennas.

# Table 8.4.5-1: Summary of cross polarization verification results for Band 13, and Band 7 for the UMi channel model

|  |  | MPAC System 1, CE 1, Band 13 |  |  |  |  |  |  |  | MPAC System 1, CE 1, Band 7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel Model |  |  | Theory |  |  | V/H ratio |  | Deviation |  |  | Theory |  | V/H ratio |  | Deviation |  |
| UMi |  |  | 0.74 |  |  | 1.07 |  | 0.33 |  |  | 0.74 |  | 0.14 |  | 0.60 |  |
|  | MPAC System 1, CE 2, Band 13 |  |  |  |  |  |  |  | MPAC System 1, CE 2, Band 7 |  |  |  |  |  |  |
| Channel Model |  |  |  | Theory | V/H ratio |  | Deviation |  |  | Theory |  | V/H ratio |  | Deviation |  |  |
| UMi |  |  | 0.74 |  |  | 0.77 |  | 0.03 |  |  | 0.74 |  | 0.97 |  | 0.23 |  |
|  |  | MPAC System 2, CE 1, Band 13 |  |  |  |  |  |  |  | MPAC System 2, CE 1, Band 7 |  |  |  |  |  |  |
| Channel Model |  | Theory |  |  | V/H ratio |  | Deviation |  |  | Theory |  | V/H ratio |  | Deviation |  |  |
| UMi |  |  | 0.74 |  |  | 0.47 |  | 0.27 |  |  | 0.74 |  | 0.67 |  | 0.07 |  |

The cross-polarization results for the RTSmethod using geometric implementation of SCME are shown in Table 8.4.5.2:

Table 8.4.5-2: Summary of cross polarization verification results for Band 13 with geometric implementation

| Channel model |  |  | V/H ratio | Theory | Deviation |
| --- | --- | --- | --- | --- | --- |
| UMa MC |  |  | 8.1 | 8.13 | 0.03 |
| UMi MC |  |  | 0.8 | 0.83 | 0.03 |

The cross-polarization results for the RTS method using correlation implementation of SCME (Jake's Doppler spectrum) are shown in Table 8.4.5.3:

Table 8.4.5-3: Summary of cross polarization verification results for Band 13 with correlation implementation (Jake’s Doppler spectrum)

| Channel model | V/H ratio | Theory | Deviation |
| --- | --- | --- | --- |
| UMa | 7.9 | 8.13 | -0.23 |
| UMi | 0.8 | 0.83 | -0.03 |

### 8.4.6	Summary

The summary of the channel model validation activity is provided in Table 8.4.6-1 below.

# Table 8.4.6-1: Summary of channel model validation results

| Item | Parameter | Result | Tolerances | Comments |
| --- | --- | --- | --- | --- |
| 1 | Power delay profile | See 8.4.2 | FFS (Note) |  |
| 2 | Doppler / Temporal Correlation | See 8.4.3 | FFS (Note) |  |
| 3 | Spatial Correlation | See 8.4.4 | FFS (Note) |  |
| 4 | Cross Polarization | See 8.4.5 | FFS (Note) |  |
| NOTE: Further investigation of channel model validation metrics and their corresponding tolerances is on-going within the framework of measurement uncertainty budget development |  |  |  |  |

## << End of changes >>