3GPP TSG-RAN WG4 Meeting #83	R4-1705971

# Hangzhou, China, 15 – 19 May, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 4978 | rev | 1 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FS3-only sTAG support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  |  | Date: |  | 2017-05-19 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | FS3-only sTAG support is not included in the specification yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of FS3-only sTAG in eLAA and the definition of a reliable timing reference cell is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 7	Timing and signalling characteristics

## 7.1	UE transmit timing

### 7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the connected eNode B. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. The UE shall be configured with a pTAG containing the PCell. The pTAG may also contain up to four SCells, if configured. The UE capable of supporting multiple timing advances [2] may also be configured with one or two serving cells with uplink in one or two sTAG and pTAG.

The other downlink SCell(s), if configured, will be contained in either the pTAG or the sTAG(s). In pTAG, UE shall use the PCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. When the UE capable of supporting multiple timing advance [2] is configured with one or two sTAG(s), the UE shall use an activated SCell from the sTAG for deriving the UE transmit timing for cells in the sTAG. UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements. The requirements in clause 7 apply to all TAGs.

The UE capable of supporting dual connectivity shall be configured with one pTAG and may also be configured with one psTAG. The pTAG shall contain the PCell and may also contain one SCell, if configured. The psTAG shall contain the PSCell and may also contain one SCell, if configured. In pTAG, UE shall use the PCell as the reference cell for deriving the UE transmit timing for pTAG, and in psTAG, UE shall use the PSCell as the reference cell for deriving the UE transmit timing for psTAG. UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements. The requirements in clause 7 apply to both TAGs.

The UE supporting carrier aggregation with FS3 SCells shall be configured with one pTAG and may also be configured with one or two sTAG(s). The pTAG shall contain the PCell and may also contain up to four FS3 SCells, if configured. In pTAG, UE shall use the PCell as the reference cell for deriving the UE transmit timing for FS3 SCells. When the UE capable of supporting multiple timing advance [2] is configured with one or two sTAG(s) with at least one FS3 SCell in which case the pTAG may or may not contain any FS3 SCell, the UE shall use an activated SCell from the sTAG for deriving the UE transmit timing for cells in the sTAG. If an sTAG contains at least one non-FS3 SCell, the UE shall use a non-FS3 SCell as the reference cell for deriving the UE transmit timing for the cells in the sTAG. If an sTAG contains only FS3 cells, the UE shall use one of the FS3 cells for deriving the UE transmit timing for the cells in the sTAG. An FS3 cell, which is used for deriving the UE transmit timing for the cells in the same sTAG, shall fulfil all conditions specified in section 7.1.1.1. UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements. The requirements in clause 7 apply to pTAG.

#### 7.1.1.1	Reliability of FS3 cell as timing reference

An FS3 cell shall be considered as a reliable timing reference for other cells in the same sTAG, if:

- [3] DL subframes are available within [1.28] s before the uplink transmission, when the UE is not using DRX or using DRX with DRX cycle length not longer than [1.28] s, or

- [3] DL subframes are available within [1.28] s before the uplink transmission within the same DRX ON period, when the UE is using DRX with DRX cycle length longer than [1.28] s.

The UE shall monitor all FS3 cells in the sTAG containing only FS3 cells to keep track of a reliable timing reference cell for deriving the UE transmit timing, and choose a reliable FS3 cell as timing reference cell. If an FS3 cell used as timing reference for other cells in the sTAG turns unreliable, the UE shall choose another reliable FS3 cell as timing reference cell.