3GPP TSG-RAN WG4 Meeting #83 	R4-1706025

Hangzhou, China, 15 - 19 May, 2017

Agenda item:	7.26

Source: 	Intel Corporation

Title: 	Meeting minutes for measurement gap enhancement WI

Document for:	Approval

# Reply LS to RAN2 on NCSG application

RAN4 has discussed the RAN2 LS and reached the following agreement:

Agreement:

NCSG is also applicable to non-CA case.

NCSG can be configured, for the inter-frequency/inter-RAT measurement when a UE is capable of performing such measurement without gap, but needs interruption, and:

- RAN4 considers NCSG as per-UE gap pattern for non-CA cases or when per-CC gap is not configured.

Agreement:

- Per-CC test cases

  - Case 1: Parallel measurement with identical gap configured

    - Test setup:

      - 1 Pcell and 1 activated SCell (f1 and f2)

      - # of frequency layers to be monitored: 4 (f3/4/5)

      - Test metric: measurement delay vs. N_freq_effective scaled measurement delay

    - On how to choose f1/2/3/4/5/6 to guarantee N_freq_effective is less than N_freq

      - Option 1: Manufacturer declared f1/2/3/4/5/6 and related test setup

      - Option 2: Pre-determined f1/2/3/4/5/6 and test setup to strive to reach the object but not guaranteed

  - Case 2: Measurement gap configured only for some of CC

    - Test setup:

      - 1 Pcell and 1 activated SCell (f1 and f2).

        - No gap is configured for one of them, e.g. measurement gap for f1 and NCSG for f2

      - # of frequency layers to be monitored: 1 (f3)

      - Test metric: measurement delay  and Ack/Nack loss on f2

    - On how to choose f1/2/3 to guarantee one of serving cells configured with no measurement gap

      - Option 1: Manufacturer declared f1/2/3 and related test setup

      - Option 2: Pre-determined f1/2/3 and test setup to strive to reach the object but not guaranteed

E///: for case 1, increase number of activated SCell to increase the chance of parallel measurements. Manufacture is at the best position to know the architecture and measurement capability.

QCOM: slightly prefer to option 1 even though concerning on manufacturer to determine the setup.

R&S: concern on number of layers, SNR and Doppler variation from TE vendor perspective

Anritsu: better to limit the number of layers.

E///, QCOM, Intel: parallel measurement feature may be triggered depending on # of layers

Intel: can we leave # of layers as part of manufacturer declaration with per-determined upper bound?

HW: how about to have the signaling test only

Agreements:

    - Further study the following options in coming meeting under TEI agenda

      - Option 1: manufacturer declaration based test, or

      - Option 2: signaling/function test only

      - Others options are not precluded

# Shorten MGL test cases

- How to make sure PSS/SSS of the target cells are within the measurement window (2ms)

- R4-1705844	CR on reduced MGL test case for FDD-FDD, Intel

- R4-1705845	CR on reduced MGL test case for FDD-FDD, Intel

Nokia: only specify that it is an async network and PSS/SSS of the target cell should be within the measurement window (2ms)

QCOM: frame boundary difference is 500us.

Agreement: frame boundary difference is 500us.

# NCSG test cases

- On how to address Ack/Nack missing rate impact due to PDCCH detection error

  - R4-1704718	CR on NCSG for FDD-FDD inter-frequency measurements, Intel

  - R4-1704720	CR on NCSG for TDD-TDD inter-frequency measurements, Intel

  - R4-1705754	Introduction of test case for NCSG

Agreement: serving cell AWGN and SNR=16dB, 0% ack/nack missing rate, target cell SNR is the same as existing test case

# Burst gap test cases

- R4-1705753	Introduction of test case for Burst gap, Qualcomm

- Agreement: serving cell AWGN and SNR=16dB, 0% ack/nack missing rate, target cell SNR is the same as existing test case

- Interruption requirement for NCSG-based inter-frequency/inter-RAT measurement, regarding the CR

- R4-1705843  Introduction of interruption control for Network Controlled Short Gap pattern for inter-frequency measurements
36.133     CR-4951  rev  Cat: B (Rel-14) v14.3.0
 Source: Nokia, Alcatel-Lucent Shanghai Bell

# UE capability

  - Per-CC gap: per-UE optional feature

  - Shorter MGL: per-UE optional feature

  - NCSG: per-UE optional feature

  - Burst gap: per-UE optional feature