3GPP TSG-RAN4 Meeting #83	R4-1706027

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.171 | CR | 0013 | rev | 1 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | InDoPos: New WLAN delay test case in nominal conditions with LTE FDD and TDD (Rel-14) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_LTE_iPos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2017-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | WLAN test requirements are not defined and they are part of the UTRA_LTE_iPos_enh2 WI. In the last R4 meetings, it has been agreed that RSSI accuracy will not be tested, but that test cases from reporting time are required (WP in R4-1704295). |  |  |  |  |  |  |  |  |
| Summary of change: |  | WLAN test requirements for nominal conditions have been added. The test cases evaluate the ability of a UE to identify and report WLAN AP within the maximum response time under nominal conditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WLAN test requirements will remain undefined and the work item cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.571-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

# A.3	UE Measurement Procedures

< Unchanged sections omitted >

### A.3.X	WLAN Access Point Identifcation and Reporting Delay

#### A.3.X.1	General

The purpose of this section is to verify the UE ability to identify and report WLAN APs under different conditions. The UE shall send wlan-MeasurementInformation IE including WLAN measurements for each AP indicating at least wlan-AP-Identifier (BSSID and SSID), RSSI and apChannelFrequency as stated in [3].

The tests in this section as, as stated in Clause 7, statistical in nature and the UE shall meet the corresponding requirement for at least 90% of the reported cases. The test cases are defined for E-UTRA testing. The test cases are assuming an LTE connection between the SS and the UE and the measurements are performed in RRC_CONNECTED state.

The tests assume that the beacon signals from multiple WLAN APs are available at the UE with a periodicity of at least 102.4 ms. The beacon signals from different APs shall be received at different time slots or in non-overlapping frequency channels. Non overlapping frequency channels shall be at least 25 MHz appart in the WLAN 2.4 GHz band and at least 20 MHz appart in the WLAN 5 GHz band.

#### A.3.X.2	LTE-FDD: WLAN AP Identification and reporting delay under nominal conditions test

##### A.3.X.2.1 Test purpose and Environment

The purpose of this test is to verify the requirements in Clause 7.3 for WLAN AP measurements.

In this test, there are cell1 (E-UTRAN FDD) and 12 WLAN APs transmitting beacon signals at least every 102.3 ms. The APs are transmitting in 3 non-overlapping frequency channels in the same WLAN Frequency Band. There are 4 APs in every channel. The test consists of two successive time periods, with duration of T1 and T2, respectively. WLAN-RequestLocationInformation message shall be provided to the UE during T1. WLAN Access Points only transmit signal during T2.

Table A.3.X.2.1-1: General WLAN AP test parameters 
for WLAN AP Identification and reporting delay under nominal conditions test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Number of Access Points | N/A | 12 | AP1- AP12 |
| Time Slot 1 | ms | 1 | AP1, AP2, AP3 |
| Time Slot 2 | ms | 1 | AP4, AP5, AP6 |
| Time Slot 3 | ms | 1 | AP7, AP8, AP9 |
| Time Slot 4 | ms | 1 | AP10, AP11, AP12 |
| Beacon Interval | ms | [20] | [In order to ensure that the UE is in passive scan mode] |
| T1 | s | 5 | During this time the WLAN signal is not transmitted |
| T2 | s | 35 | UE shall report WLAN measurement information within 30s |

Table A.3.X.2.1-2: E-UTRAN TDD Cell specific and WLAN AP specific test parameters 
for WLAN AP Identification and reporting delay under nominal conditions test

| Parameter | Unit | Cell 1 |  | AP 1,4,7,10 |  | AP 2,5,8,11 |  | AP 3,6,9,12 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | N/A |  | N/A |  | N/A |  |
| WLAN Channel Number |  | N/A |  | 1 |  | 2 |  | 3 |  |
| BWchannel |  | 10MHz |  | 20 MHz |  | 20 MHz |  | 20 MHz |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 FDD |  | N/A |  | N/A |  | N/A |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 FDD |  | N/A |  | N/A |  | N/A |  |
| OCNG Patterns |  | OP.1 FDD |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA | dB | 0 |  | N/A |  | N/A |  | N/A |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Noc1Note 2 | dBm/15 KHz | -98 |  | N/A |  | N/A |  | N/A |  |
| Noc2Note 3 | dBm/20 MHz | N/A |  | [-75] |  | [-75] |  | [-75] |  |
| Ês/Noc1 | dB | 3 | 3 | N/A |  | N/A |  | N/A |  |
| Ês/Iot Note 4 | dB | 3 | 3 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -65.5 | -65.5 |  |  |  |  |  |  |
| WLAN Received Power Level | dBm/20 MHz | N/A | N/A | - inf | -60 | - inf | -60 | - inf | -60 |
| WLAN SNRNote 4 | db | N/A |  | [15] |  | [15] |  | [15] |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  | - |  | - |  | - |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over the bandwidth and time and shall be modelled as AWGN of appropriate power for Noc2 to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP, Io and WLAN SNR have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |

##### A.3.X.2.2 Test Requirements

The WLAN Response Time shall fulfil the requirements in section 4.3 and the WLAN AP report shall fulfil the requirements in section 7.3.

#### A.3.X.3	LTE-TDD: WLAN AP Identification and reporting delay under nominal conditions test

##### A.3.X.3.1 Test purpose and Environment

The purpose of this test is to verify the requirements in Clause 7.3 for WLAN AP measurements.

In this test, there are cell1 (E-UTRAN TDD) and 12 WLAN APs transmitting beacon signals at least every 102.3 ms. The APs are transmitting in 3 non-overlapping frequency channels in the same WLAN Frequency Band. There are 4 APs in every channel. The test consists of two successive time periods, with duration of T1 and T2, respectively. WLAN-RequestLocationInformation message shall be provided to the UE during T1. WLAN Access Points only transmit signal during T2.

Table A.3.X.3.1-1: General WLAN AP test parameters 
for WLAN AP Identification and reporting delay under nominal conditions test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Number of Access Points | N/A | 12 | AP1- AP12 |
| Time Slot 1 | ms | 1 | AP1, AP2, AP3 |
| Time Slot 2 | ms | 1 | AP4, AP5, AP6 |
| Time Slot 3 | ms | 1 | AP7, AP8, AP9 |
| Time Slot 4 | ms | 1 | AP10, AP11, AP12 |
| Beacon Interval | ms | [20] | [In order to ensure that the UE is in passive scan mode] |
| T1 | s | 5 | During this time the WLAN signal is not transmitted |
| T2 | s | 35 | UE shall report WLAN measurement information within 30s |

Table A.3.X.3.1-2: E-UTRAN TDD Cell specific and WLAN AP specific test parameters 
for WLAN AP Identification and reporting delay under nominal conditions test

| Parameter | Unit | Cell 1 |  | AP 1,4,7,10 |  | AP 2,5,8,11 |  | AP 3,6,9,12 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | N/A |  | N/A |  | N/A |  |
| WLAN Channel Number |  | N/A |  | 1 |  | 2 |  | 3 |  |
| BWchannel |  | 10MHz |  | 20 MHz |  | 20 MHz |  | 20 MHz |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 TDD |  | N/A |  | N/A |  | N/A |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 TDD |  | N/A |  | N/A |  | N/A |  |
| OCNG Patterns |  | OP.1 TDD |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA | dB | 0 |  | N/A |  | N/A |  | N/A |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Noc1Note 2 | dBm/15 KHz | -98 |  | N/A |  | N/A |  | N/A |  |
| Noc2Note 3 | dBm/20 MHz | N/A |  | [-75] |  | [-75] |  | [-75] |  |
| Ês/Noc1 | dB | 3 | 3 | N/A |  | N/A |  | N/A |  |
| Ês/Iot Note 4 | dB | 3 | 3 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -65.5 | -65.5 |  |  |  |  |  |  |
| WLAN Received Power Level | dBm/20 MHz | N/A | N/A | - inf | -60 | - inf | -60 | - inf | -60 |
| WLAN SNR Note 4 | db | N/A |  | [15] |  | [15] |  | [15] |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  | - |  | - |  | - |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over the bandwidth and time and shall be modelled as AWGN of appropriate power for Noc2 to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP, Io and WLAN SNR have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |

##### A.3.X.3.2 Test Requirements

The WLAN Response Time shall fulfil the requirements in section 4.3 and the WLAN AP report shall fulfil the requirements in section 7.3.

< End of changes >