3GPP TSG-RAN WG4 Meeting #83	                  	R4-1706139

Hangzhou, China, 15 - 19 May, 2017

Source: 	Huawei

Title:	TP to TR 37.843: Consideration of missing aspects for core and conformance Tx spurious requirement OTA

Agenda Item:	9.12.2.5	(Other Transmitter Requirements)

Document for:	Approval

# Introduction

During previous RAN4#82bis meeting, discussion on the Rx spurious emissions was initiated in [2] and [3]. During offline discussions, it was decided to wait with certain proposals from [3] until the next meeting, in order to combine them with the emissions scaling aspects, loss factor consideration, etc.

In this contribution we are providing TP to TR 37.843, capturing missing aspects for core and conformance Tx spurious requirement OTA, like loss factor consideration for spurious region, SM.329 limits, emissions scaling, fundamental range for OTA, initial consideration of the MU for TRP for conformance.

# References

TR 37.843, v.0.3.0 	Active Antenna System (AAS) Base Station (BS) radiated requirements

R4-1703451			Receiver spurious emissions requirements for eAAS, RAN4#82bis, NTT DOCOMO, INC.

R4-1703945			Rx spurious requirement OTA, RAN4#82bis, Huawei

# TP to the TR 37.843

In this section, TP to the TR 37.843 [1] is proposed for approval.

- ------------------------------ Modified section ------------------------------

### 5.6.6 	Spurious emission

#### 5.6.6.1 	Background information on the conducted requirement

According to TS 37.105 [3], the transmitter spurious emission limits for the hybrid AAS BS operating bands apply in the frequency range from 9 kHz to 12.75 GHz, with consideration of the following exceptions:

- Unwanted emissions: the spurious requirements apply at frequencies within the above specified frequency range with the exception of the UTRA spectrum emission mask (SEM) and E-UTRA operating band unwanted emissions (OBUE) ranges as specified below:

  - UTRA TDD BS 1.28 Mcps option as specified in TS 25.105 [20]: from 4 MHz below the lowest frequency of each operating band to 4 MHz above the highest frequency of each operating band.

  - UTRA TDD BS 3.84 Mcps option as specified in TS 25.105: from more than 12.5 MHz under the lowest carrier frequency used or more than 12.5 MHz above the highest carrier frequency used.

  - UTRA TDD BS 7.68 Mcps option as specified in TS 25.105: from more than 25 MHz under the lowest carrier frequency used or more than 25 MHz above the highest carrier frequency used.

  - UTRA FDD BS as specified in TS 25.104 [19]: from 12.5 MHz below the lowest carrier frequency used up to 12.5 MHz above the highest carrier frequency used.

  - E-UTRA BS as specified in TS 36.104 [8]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

  - MSR BS as specified in TS 37.104 [7]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

- Upper frequency limit of the spurious range: in order to comply with the SM.329 recommendation [18], in case of spurious emissions requirement for certain operating bands, the upper limit of the spurious range is extended beyond 12.75 GHz to the limit of 5th harmonic (but not higher than 26 GHz). For more details on the upper limit setting in those cases, refer to SM.329.

For AAS BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, this exclusion applies for each supported operating band.

#### 5.6.6.2	Transmitter spurious emissions OTA requirement

The metric used to capture transmitter spurious emissions OTA is total radiated power (TRP).

The emission levels and the measurement bandwidths for the transmitter spurious emissions OTA requirement are  based on the conducted requirements specified in TS 37.105 [3]. In case of OTA AAS BS requirement, emission levels are subject to fixed scaling, as described in sub-clause 5.6.1.2.

The core requirement for the transmitter spurious emissions OTA is applied over 30 MHz to 12.75 GHz range, with consideration of the SEM and OBUE exceptions, and the operating band specific upper limit extensions of the spurious range, as defined in subclause 5.6.6.1.

Transmitter spurious emissions such as co-location requirements or regional requirements are FFS.

#### 5.6.6.3 	Transmitter spurious emissions OTA conformance requirement

In order that the requirement is measurable the total radiated power as defined in subclause 5.1.1 is approximated as the sum of the EIRP at a number of discrete directions around the sphere as follows:

, where EIRPe is the filtered mean emission power in the spurious domain  and p1 and p2 denotes two orthogonal polarizations.

An approximation of TRP measurement can be achieved by sampling discrete EIRP measurements surrounding the AAS BS. The use of discrete sampling grid for the TRP approximation is motivated by the observation that in the spurious domain there will be no coherent beam forming, as the AAS BS are designed to form beams based on superposition of signals from individual transceivers in TRXU array in their in-band frequency ranges, while in the spurious domain those signals are expected to be sufficiently uncorrelated.

Conformance requirement for the TX spurious requirement OTA will be composed as a single requirement together with the EMC RE requirement OTA, as there is no possibility to separate those requirements in OTA test setup. The resulting test requirement will be subject to single test setup and single measurement procedure. Definition of the conformance requirement will be subject to  TRP MU, where the MU definition is FFS.  For more details on the EMC RE requirement, refer to clause 8.

- ------------------------------ End of modified sections ------------------------------