TSG-RAN Working Group 4 (Radio) #84 meeting	R4-17 07071

Berlin, Germany, 21 - 25 August, 2017

Source:	Samsung

Title:	MPR evaluation results for sub 6GHz

Agenda item:	9.4.3.1.2

Document for:	Discussion

# Background

In RAN4 NR AdHoc2, MPR evaluation assumptions are agreed in [1]. There are 8 test cased defined for calibration.

| Waveform type | Modulation order | Channel BW | SCS | RB allocation | remark |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | QPSK, 16QAM | 20MHz | 15kHz | 100RB3 | Maximum DFT-s-OFDM allocation |
| DFT-s-OFDM | QPSK, 16QAM | 20MHz | 15kHz | 18RB0 | Channel edge (LTE 0dB MPR for QPSK) |
| DFT-s-OFDM | QPSK, 16QAM | 20MHz | 15kHz | 18RB18 | Inside channel |
| DFT-s-OFDM | QPSK, 16QAM | 100MHz | 30kHz | 270RB1 | Maximum channel BW |
| CP-OFDM | QPSK, 16&64QAM | 20MHz | 15kHz | 106RB0 | Allocation for spectrum utilisation |
| CP-OFDM | QPSK, 16&64QAM | 20MHz | 15kHz | 18RB0 | Channel Edge |
| CP-OFDM | QPSK, 16&64QAM | 20MHz | 15kHz | 18RB18 | Inside Channel |
| CP-OFDM | QPSK, 16&64QAM | 100MHz | 30kHz | 273RB0 | Allocation for spectrum utilisation & max CH BW |
| Companies are encouraged to report reduced MPR and negative MPR cases Companies are encouraged to report which criteria is limiting and when applicable which transmitter impairments drives the limitation Companies are encouraged to provide their view of Pi/2 BPSK use at sub-6GHz |  |  |  |  |  |

This contribution provides initial MPR simulation results for sub 6GHz frequency range for calibration.

# Discussion on simulation model

Our simulation model includes windowing at baseband for spectrum confinement. Hann window is used with overlapping length equal to 2% of symbol length. PN model applies a CMOS model in [2], down-scaled to 5GHz carrier frequency.

PA model uses the polynomial model provided in [3]. Some modifications on PA model are done to cover a wide simulation range. PA output is saturated at 29dBm. Otherwise, a dip from polynomial would show up. We also linearize AMAM and AMPM below Pin = -20dBm.

Figure 1. Left: modified AMAM response based on [3], Right: modified AMPM response based on [3].

The frontloaded DMRS is located at Symbol 0 and modelled with two ports using comb structure, i.e., every of the other RE is used for one DMRS port.

Figure 2. sub6 NR frame structure.

For EVM measurement, EVM window is scaled-up based on 1.4MHz LTE setting. Frequency domain linear interpolation is performed for channel estimation to recover REs used for the other port.

# Simulation results

Our initial simulation results for PC3 RF requirements are provided in Table 1. The RF requirements that are limiting PA output power are highlighted in red. Some observations are

Observation 1: 	All 256QAM simulations are EVM limited.

Observation 2: 	For channel edge transmission, both ACLR and SEM / spurious (except 256QAM) are limiting PA output power.

Observation 3: 	For full-BW transmission, ACLR (except 256QAM) is limiting PA output power.

Table 1. sub6 MPR simulation results

| Test Case | Waveform | SCS [KHz] | CHBW [MHz] | Mod | RB allocation [start RB, RB num] | ACLR feasible Pin (<=) [dBm] | EVM feasible Pin (<=)[dBm] | SEM& Spurious feasible Pin (<=) [dBm] | Pout [dBm] | MPR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | DFT-s-OFDM | 15 | 20 | QPSK | [0,100] | -1.1 | 0 | -1 | 25.6 | 0.4 |
|  |  |  |  | 16QAM |  | -1.6 | 0 | -1.4 | 25 | 1 |
|  |  |  |  | 64QAM |  | -1.7 | -1 | -1.5 | 24.9 | 1.1 |
|  |  |  |  | 256QAM |  | -1.7 | -9.1 | -1.5 | 17.2 | 8.8 |
| Case 2 | DFT-s-OFDM | 15 | 20 | QPSK | [0,18] | -0.2 | 0 | -0.5 | 26 | 0 |
|  |  |  |  | 16QAM |  | -0.7 | 0 | -0.9 | 25.5 | 0.5 |
|  |  |  |  | 64QAM |  | -0.8 | -0.9 | -1 | 25.4 | 0.6 |
|  |  |  |  | 256QAM |  | -0.9 | -9 | -1 | 17 | 9 |
| Case3 | DFT-s-OFDM | 15 | 20 | QPSK | [18,18] | 0 | 0 | 0 | 26.6 | -0.6 |
|  |  |  |  | 16QAM |  | 0 | 0 | 0 | 26.4 | -0.4 |
|  |  |  |  | 64QAM |  | 0 | -1.2 | 0 | 25.5 | 0.5 |
|  |  |  |  | 256QAM |  | 0 | -9.3 | 0 | 17 | 9 |
| Case 4 | DFT-s-OFDM | 30 | 100 | QPSK | [1,270] | -1.5 | 0 | 0 | 25.2 | 0.8 |
|  |  |  |  | 16QAM |  | -1.9 | -0.1 | -1.6 | 24.8 | 1.2 |
|  |  |  |  | 64QAM |  | -1.9 | -1.3 | -1.7 | 24.7 | 1.3 |
|  |  |  |  | 256QAM |  | -2 | -9.6 | -1.7 | 16.6 | 9.4 |
| Case 5 | OFDM | 15 | 20 | QPSK | [0,106] | -3.1 | -0.2 | -2.3 | 23.6 | 2.4 |
|  |  |  |  | 16QAM |  | -3.1 | -1.5 | -2.3 | 23.6 | 2.4 |
|  |  |  |  | 64QAM |  | -3.1 | -2.8 | -2.3 | 23.6 | 2.4 |
|  |  |  |  | 256QAM |  | -3.1 | -11.5 | -2.3 | 14.7 | 11.3 |
| Case 6 | OFDM | 15 | 20 | QPSK | [0,18] | -2.2 | -0.2 | -2.1 | 24.2 | 1.8 |
|  |  |  |  | 16QAM |  | -2.2 | -1.4 | -2.1 | 24.2 | 1.8 |
|  |  |  |  | 64QAM |  | -2.2 | -2.6 | -2.1 | 23.7 | 2.3 |
|  |  |  |  | 256QAM |  | -2.2 | -11.4 | -2.1 | 14.5 | 11.5 |
| Case 7 | OFDM | 15 | 20 | QPSK | [18,18] | 0 | -0.5 | 0 | 25.7 | 0.3 |
|  |  |  |  | 16QAM |  | 0 | -1.8 | 0 | 24.8 | 1.2 |
|  |  |  |  | 64QAM |  | 0 | -3 | 0 | 23.8 | 2.2 |
|  |  |  |  | 256QAM |  | 0 | -11.8 | 0 | 14.5 | 11.5 |
| Case 8 | OFDM | 30 | 100 | QPSK | [0,273] | -3.2 | -0.1 | -0.3 | 23.4 | 2.6 |
|  |  |  |  | 16QAM |  | -3.2 | -1.4 | -0.3 | 23.4 | 2.6 |
|  |  |  |  | 64QAM |  | -3.2 | -2.8 | -0.3 | 23.4 | 2.6 |
|  |  |  |  | 256QAM |  | -3.2 | -11.2 | -0.3 | 14.9 | 11.1 |

# References

[1]	R4-1706980, “WF on MPR evaluation assumption”, NTT DOCOMO, INC.

[2]	R4-1703528, “Phase noise model for above 6 GHz”, Huawei, Hisilicon

[3]	R4-164542, “Response LS on realistic power amplifier model for NR waveform evaluation”, Nokia

# Appendix: simulated operation curves

- Test case 1

- Test case 2

- Test case 3

- Test case 4

- Test case 5

- Test case 6

- Test case 7

- Test case 8