3GPP TSG-RAN WG4 Meeting #84	R4-1707237

Berlin, Germany, 21 – 25 Aug, 2017

Source:	Mediatek Inc

Title:	TP for TR36.715-03-01: the support of CA_1A-1A-7A_1UL_BCS0

Agenda item:	8.3.2

Document for:	Approval

1.  Introduction

CA_1A-1A-7A_1UL_BCS0 was approved in 3DL/1UL basket WID in the RAN-P [1]. This paper is to provide BCS information, co-existence studies and Tib/Rib TP for the completion of this combination.

2.  Reference

[1]      RP-171126 LTE Advanced inter-band CA Rel-15 for 3DL/1UL, RAN #76.

[2]      TR36.852-12-c20 LTE Advanced inter-band Carrier Aggregation (CA) (2DL/1UL)

Annex: TP for TR36.715-03-01 (va.b.c)

[Unchanged portions skipped]

CA_1A-1A-7A_1UL_BCS0

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-1A-7A | 1 | See the CA_1A-1A Bandwidth combination set 0 in the Table below |  |  |  |  |  | 60 | 0 |
|  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |

# Table 5.x.1-2: Supported E-UTRA bandwidths of CA_1A-1A configuration

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_1A-1A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

5.x.2	Co-existence studies

For single UL the 2nd and 3rd harmonic analysis for B1 and B7 is already done in the 2DL/1UL[2]. As shown in the table 5.x.2-1, none of the products falls into the B1 or B7 UE receiving bands. For UE coexistence the single UL is already considered for these bands in TS 36.101.

Table 5.x.2-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |
| 7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 |

### 5.x.3	ΔTIB and ΔRIB values

The UE architecture is same as that of CA_1A-7A. Same values of TIB,c and ΔRIB,c can be re-used.

Table 5.x.3-1: ΔTIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-1A-7A | 1 | 0.5 |
|  | 7 | 0.6 |

Table 5.x.3-2: ΔRIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-1A-7A | 1 | 0 |
|  | 7 | 0 |

5.x.4	REFSENS requirements

REFSENS remain unchanged. No further MSD is needed.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"