3GPP TSG-RAN WG4 Meeting #84	R4-1707241

Berlin, Germany, 21 – 25 Aug, 2017

Source:	Mediatek Inc

Title:	TP for TR36.715-03-01: the support of CA_3A-7A-20A_1UL_BCS1

Agenda item:	8.3.2

Document for:	Approval

1.  Introduction

CA_3A-7A-20A_1UL_BCS1 was approved in 3DL/1UL basket WID in the RAN-P [1]. This paper is to provide TP for the completion of this combination.

Since the intention to introduce CA_3A-7A-20A_1UL_BCS1 is to add 5MHz BW support in Band 7, the essential study results, including co-existence, Tib/Rib, REFSENS, remain unchanged.

2.  Reference

[1]      RP-171126 LTE Advanced inter-band CA Rel-15 for 3DL/1UL, RAN #76

Annex: TP for TR36.715-03-01 (va.b.c)

[Unchanged portions skipped]

CA_3A-7A-20A_1UL_BCS1

5.x.1	Channel bandwidths per operating band for CA

A new bandwidth combination set 1 is introduced for CA_3A-7A-20A in Release 15.

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_3A-7A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2	Co-existence studies

Co-existence already studied when BCS0 for CA_3A-7A-20A was introduced and no further studies needed.

### 5.x.3	ΔTIB and ΔRIB values

Insertion loss values already defined when BCS0 for CA_3A-7A-20A was introduced.

5.x.4	REFSENS requirements

REFSENS remain unchanged. No further MSD is needed.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"