3GPP TSG-RAN WG4 Meeting #84	R4-1707361

# Berlin, Germany, 21 - 25 August, 2017

Agenda item:	6.2.4.2

Title: 	Simulation results for MPDCCH performance for non BL UE in CE Mode A

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

The existing Rel-13 requirements for UE supporting CE are applicable to 1 Rx. In RAN4# 83 it was agreed that new requirements shall be defined for UE supporting CE with 2/4 Rx antenna. Based on the agreements captured in the [1] companies were requested to provide ideal simulation results for the following test cases with 2/4 Rx antenna.

| Test cases | Reference in TS36.101 |
| --- | --- |
| MPDCCH CE Mode A | 8.11.2.1.1 / 8.11.2.2.1 |
| MPDCCH CE Mode B | 8.11.2.1.2 / 8.11.2.2.2 |
| PDSCH TM6 | 8.11.1.1.1 / 8.11.1.2.1 |
| PDSCH TM9 | 8.11.1.1.2 / 8.11.1.2.2 |
| PDSCH TM2 | 8.11.1.1.3 / 8.11.1.2.3 |

In this contribution we provide ideal simulation results for MPDCCH for CE Mode A in FDD and TDD duplex modes.

# MPDCCH Simulation Results – 2 Rx Antenna

In this section we provide the simulation results for MPDCCH in CE Mode A with 2 Rx antenna in FDD and TDD modes. Based on the agreements captured in [1], the simulation assumptions are the same as that for 1Rx in TS36.101 sections 8.11.2.1.1 and 8.11.2.2.1 except the MPDCCH repetition level. The simulation parameters are listed in Appendix A.

Figure 1: MPDCCH CE Mode A in FDD - 2 Rx

Observation 1: With 2 Rx in CE Mode A, repetition level of 8 achieves 1% MPDCCH error rate at -8.2 dB SNR, which is close to performance of 1Rx with repetition level of 32

Proposal 1: Set the repetition level for CE Mode A, 2Rx as 8 in FDD

Figure 2: MPDCCH CE Mode A in TDD – 2Rx

Observation 2: In TDD with 2 Rx in CE Mode A, repetition level of 8 achieves 1% MPDCCH error rate at -8.8 dB and repetition level of 4 achieves 1% MPDCCH error rate at -6.3 dB

Proposal 2: Set the repetition level for CE Mode A, 2Rx as 8 or 4 in TDD

Table 1: Simulation Results for MPDCCH - 2Rx, CE Mode A

| Duplex Type | BW | Aggregation level | Repetition Level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| FDD | 10 MHz | 16 ECCE | 16 | R.81 FDD | EPA5 | 2 x 2 Low | 1 | -10.3 |
|  |  |  | 8 | R.81 FDD | EPA5 | 2 x 2 Low | 1 | -8.2 |
|  |  |  | 4 | R.81 FDD | EPA5 | 2 x 2 Low | 1 | -5.4 |
| TDD | 10 MHZ | 16 ECCE | 8 | R.81 TDD | EPA5 | 2 x 2 Low | 1 | -8.8 |
|  |  |  | 4 | R.81 TDD | EPA5 | 2 x 2 Low | 1 | -6.3 |
|  |  |  | 2 | R.81 TDD | EPA5 | 2 x 2 Low | 1 | -3.6 |

# MPDCCH Simulation Results – 4 Rx Antenna

In this section we provide the simulation results for MPDCCH in CE Mode A with 4 Rx antenna in FDD and TDD modes. The simulation parameters are listed in Appendix A.

Figure 3: MPDCCH CE Mode A in FDD - 4 Rx

Observation 3: With 4 Rx in CE Mode A, repetition level of 2 achieves 1% MPDCCH error rate at -8.2 dB SNR, which is close to performance of 1Rx with repetition level of 32

Proposal 3: Set the repetition level for CE Mode A, 4Rx as 2 in FDD

Figure 4: MPDCCH CE Mode A in TDD - 4 Rx

Observation 4: In TDD with 4 Rx in CE Mode A, repetition level of 2 achieves 1% MPDCCH error rate at -8.3 dB SNR, which is close to performance of 1Rx with repetition level of 16

Proposal 4: Set the repetition level for CE Mode A, 4Rx as 2 in TDD

Table 2: Simulation Results for MPDCCH - 4Rx, CE Mode A

| Duplex Type | BW | Aggregation Level | Repetition Level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| FDD | 10 MHz | 16 ECCE | 4 | R.81 FDD | EPA5 | 2 x 4 Low | 1 | -10.0 |
|  |  |  | 2 | R.81 FDD | EPA5 | 2 x 4 Low | 1 | -8.2 |
|  |  |  | 1 | R.81 FDD | EPA5 | 2 x 4 Low | 1 | -5.4 |
| TDD | 10 MHZ | 16 ECCE | 4 | R.81 TDD | EPA5 | 2 x 4 Low | 1 | -10.8 |
|  |  |  | 2 | R.81 TDD | EPA5 | 2 x 4 Low | 1 | -8.3 |
|  |  |  | 1 | R.81 TDD | EPA5 | 2 x 4 Low | 1 | -5.9 |

# Conclusions

Based on the simulation results presented the following are recommended for UE supporting CE with 2/4 Rx for CE Mode A:

Proposal 1: Set the repetition level for CE Mode A, 2Rx as 8 in FDD

Proposal 2: Set the repetition level for CE Mode A, 2Rx as 8 or 4 in TDD

Proposal 3: Set the repetition level for CE Mode A, 4Rx as 2 in FDD

Proposal 4: Set the repetition level for CE Mode A, 4Rx as 2 in TDD

# References

- R4-1706196  “WF on requirements for Rel-13 Non-BL UE”, Huawei, HiSilicon, Ericsson, Intel Corporation, Qualcomm

# Appendix A

Simulation assumptions for CE Mode A are listed below.

Table 2: Simulation assumptions for MPDCCH for FDD

| Parameter |  | Unit | CE Mode A |
| --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  | δ | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 4 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 |
| Transmission type |  |  | Distributed |
| Frequency hopping |  |  | Disabled |
| Number of frequency hopping narrowbands |  |  | N/A |
| 1.5Frequency hopping offset |  |  | N/A |
| Frequency hopping inverval |  | ms | 4 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  | 1.5 |
| Maximum number of repetitions |  |  | 16,8,4 for 2 Rx 4,2,1 for 4 Rx |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 |
| PDSCH TM |  |  | TM2 |
| Reference channel for PDSCH transmission |  |  | R.81 FDD |
| DCI Format |  |  | 6-1A |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 |
| Note1: For each test, DC subcarrier puncturing shall be considered. Note2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |

Table 3:  Simulation assumptions for MPDCCH for TDD

| Parameter |  | Unit | CE Mode A |
| --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 |
| Unused RE-s and PRB-s |  |  | OCNG |
| Cell ID |  |  | 0 |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 |
|  |  | dB | 0 |
|  | δ | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Cyclic prefix |  |  | Normal |
| Subframe Configuration |  |  | Non-MBSFN |
| Precoder Update Granularity |  | PRB | 1 |
|  |  | ms | 5 (Note 2) |
| Beamforming Pre-Coder |  |  | Annex B.4.4 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 |
| Transmission type |  |  | Distributed |
| Frequency hopping |  |  | Disabled |
| Number of frequency hopping narrowbands |  |  | N/A |
| Frequency hopping offset |  |  | N/A |
| Frequency hopping inverval |  | ms | 5 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 4) |  |  | 2 |
| Maximum number of repetitions |  |  | 8,4,2 for 2 Rx 4,2,1 for 4 Rx |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 |
| PDSCH TM |  |  | TM2 |
| Reference channel for PDSCH transmission |  |  | R.81 TDD |
| DCI Format |  |  | 6-1A |
| TDD UL/DL Configuration |  |  | 0 |
| TDD Special Subframe |  |  | 1 |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1000010000 |
| Note 1: For each test, DC subcarrier puncturing shall be considered. Note 2: Same precoding matrix is used for a PRB across subframes where MPDCCH is repeated. Note 3: The special subframes are not supported by MPDCCH, and are assumed as non- BL/CE DL subframes. Note 4: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. |  |  |  |