3GPP TSG-RAN WG4 Meeting #84 	R4-1707411

Berlin, Germany, 21 – 25 August, 2017

Agenda item:	10.1.1.2

Source: 	Intel Corporation

Title: 	Initial PDSCH simulation results for 8Rx with rank higher than 4

Document for:	Discussion

- Introduction

The Release 15 Study Item “Study on LTE DL 8Rx antenna ports” has been introduced and the Study Item Description is given in [1]. It is expected that the 8Rx antenna configuration would have a lot different demodulation performance compared with a smaller number of receive antennas, such as, 4Rx antenna configuration. In this contribution, we provide our initial PDSCH simulation results for 8Rx with rank higher than 4.

- Simulation assumptions and results

We first consider an existing 4Rx testcase for TM9 4-layer Spatial Multiplexing, which is specified in TS 36.101 [1] Section 8.10.1.1.9, and is also summarized and listed below:

Table 8.10.1.1.9-1: Minimum performance for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | 4 layer precoding based on WB PMI feedback |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Precoding granularity |  |  | 50 |
| PMI delay |  |  | 8 |
| Reporting interval |  |  | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.10.1.1.9-2: Minimum performance for for 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 16QAM | R.75 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 18.4 | ≥5 |

We then try to re-use the above 4Rx testcase for 8Rx configuration as a measure of comparison between 4Rx and 8Rx antenna configurations. Moreover, for the 8Rx case, we consider an 8×8 antenna configuration, and 8-layer Spatial Multiplexing. The purpose for the simulations is to compare the operating SNR range between 8-layer and 4-layer Spatial Multiplexing, i.e. the operating SNR range of 8-layer Spatial Multiplexing relative to the 4-layer scenario.

Fig.1 Throughput performance for TM9, 4-layer (4×4) and 8-layer (8×8) spatial multiplexing, EPA5, low correlation

Fig.2 Normalized throughput performance for TM9, 4-layer (4×4) and 8-layer (8×8) spatial multiplexing, EPA5, low correlation

Observation 1: 8-layer (8×8) spatial multiplexing is able to achieve double throughput compared with 4-layer (4×4) spatial multiplexing, if SNR is high enough.

Observation 2: The operating SNR range (70% of max throughput) of 8-layer (8×8) spatial multiplexing is about 1.5dB worse than 4-layer (4×4) spatial multiplexing.

Observation 3: The simulations in Fig.1 and Fig.2 consider low modulation level (16QAM) and low MIMO channel correlation. Obviously, the operating SNR would increase as the modulation level and MIMO channel correlation increase.

Proposal 1: Considering one of our parallel contributions [2] that addresses the MIMO channel correlation issue for 8Rx, it is proposed to further study the impact of higher receive antenna correlation for up to 8-layer (8×8) spatial multiplexing scenarios, especially on the operating SNR range.

Proposal 2: Since one of the promised benefits of 8Rx implementation is to boost the downlink peak data rate. One way to do that is to combine 8Rx with higher modulation levels, such as 256QAM. It would make a lot sense to investigate the operating SNR range specifically for 8-layer (8×8) spatial multiplexing combined with 256QAM.

- Conclusion

In this contribution, we provided our initial PDSCH simulation results for 8Rx.

Observation 1: 8-layer (8×8) spatial multiplexing is able to achieve double throughput compared with 4-layer (4×4) spatial multiplexing, if SNR is high enough.

Observation 2: The operating SNR range (70% of max throughput) of 8-layer (8×8) spatial multiplexing is about 1.5dB worse than 4-layer (4×4) spatial multiplexing.

Observation 3: The simulations in Fig.1 and Fig.2 consider low modulation level (16QAM) and low MIMO channel correlation. Obviously, the operating SNR would increase as the modulation level and MIMO channel correlation increase.

Proposal 1: Considering one of our parallel contributions [2] that addresses the MIMO channel correlation issue for 8Rx, it is proposed to further study the impact of higher receive antenna correlation for up to 8-layer (8×8) spatial multiplexing scenarios, especially on the operating SNR range.

Proposal 2: Since one of the promised benefits of 8Rx implementation is to boost the downlink peak data rate. One way to do that is to combine 8Rx with higher modulation levels, such as 256QAM. It would make a lot sense to investigate the operating SNR range specifically for 8-layer (8×8) spatial multiplexing combined with 256QAM.

- Reference

[1] TS 36.101 User Equipment (UE) radio transmission and reception v13.6.1 (2017-01).

[2] R4-1707409, “Initial PDSCH simulation results for 8Rx with rank lower than or equal to 4”, Intel Corporation, August 2017.