3GPP TSG-RAN WG4 Meeting #84	R4-1707477

Berlin, Germany, 21 – 25 August, 2017

Source:		Ericsson

Title:			Further discussion on OOB blocking challenges

Agenda item:		8.28.3.4

Document for:	Discussion

# Introduction

In RAN4-#83 OOB blocking was discussed in [1] and [2]. In [2] the history of the requirement of -15 dBm was explained. A review has been made as part of the work for this discussion also, but no other background than found in [2], i.e. the rationale for the OOB blocking requirement is based on a UE analysis and a proposal by Nokia in 1999 has been found. Clearly there are some strange properties of the OOB blocking requirement and when specifying OOB blocking OTA, there seems to be good reasons to think twice and not entirely base the OTA requirement on the legacy conducted requirement. This contribution further discusses aspects of the OOB blocking requirement when translating into an OTA requirement.

# Discussion

As also noted in previous discussions there are several rather odd properties of the OOB blocking requirement:

- The OOB blocking requirement is based on a limited UE analysis for frequencies 1-3 GHz and no analysis has been made for BS equipment, and not for frequencies < 1 GHz and > 3 GHz.

- The OOB blocking requirement is the same (-15 dBm) for WA, MR, and LA cases

- The OOB blocking requirement is constant over the entire frequency range (1 MHz – 12.75 GHz) of the requirement

Observation 1:

The OOB blocking requirement has several odd properties and is not suitable to translate as such to an OTA requirement.

When applying the same assumptions as [2] for the WA case, i.e. a minimum FSPL based on a minimum distance of 42 m (BS height 25 m and closest UE 35 m distant along ground at 1.5 m height), and calculating the required EIRP level by the closest possible interferer in order to reach -15 dBm received power level, the results of Figure 1 are obtained.

Figure 1:  Minimum required EIRP (dBm) of interferer to reach -15 dBm received power of victim.

At 6 GHz the FSPL is, based on the assumptions, approximately 80 dB, and the required EIRP of a hypothetical interferer would then need to be at least 65 dBm (approximately 3.2 kW). This is a very high output power level which is most likely unrealistic in real life for frequencies > 6 GHz, and consequently it is reasonable to limit the WA OOB blocking requirement to frequencies < 6 GHz. Note that no antenna gains have been taken into account in this example (seems reasonable for the victim since the analysis is for OOB), however no other losses than the FSPL are taken into account either.

Limiting the OOB blocking requirement for WA OTA AAS BS to < 6 GHz would significantly simplify the specification of an OTA test method and also align with the radiated immunity (RI) frequency range.

Observation 2:

Due to the high path loss at frequencies > 6 GHz the required EIRP output power of an interferer is  > 65 dBm in order to exceed the OOB blocking requirement of -15 dBm at the victim.

Observation 3:

Limiting the OOB blocking OTA test to frequencies < 6 GHz makes the upper frequency range aligned with the EMC radiated immunity requirement, and may thereby be possible to align.

Proposal 1:

For the WA case, the OTA OOB blocking requirement shall be limited to frequencies < 6 GHz.

# Conclusion

The following observations and proposals were made in this discussion:

Observation 1:

The OOB blocking requirement has several odd properties and is not suitable to translate as such to an OTA requirement.

Observation 2:

Due to the high path loss at frequencies > 6 GHz the required EIRP output power of an interferer is  > 65 dBm in order to exceed the OOB blocking requirement of -15 dBm at the victim.

Observation 3:

Limiting the OOB blocking OTA test to frequencies < 6 GHz makes the upper frequency range aligned with the EMC radiated immunity requirement, and may thereby be possible to align.

Proposal 1:

For the WA case, the OTA OOB blocking requirement shall be limited to frequencies < 6 GHz.

# References

R4-1705495		General OOB blocking challenges, Ericsson, RAN4-#83

R4-1705649		Receiver out of band blocking, Huawei, RAN4-#83