3GPP TSG-RAN WG4 Meeting #84				           R4-1707489

Berlin, Germany, 21th – 25th Aug, 2017

Title:	Further discussion on UE measurement capabilities across LTE and NR

Source:	CATT

Agenda Item:	9.6.2

Document for:	Discussion

- Introduction

In RAN2#98 meeting, RAN2 sent LS to RAN4 on UE measurement capability across LTE and NR. The related questions are copied as follows [1].

When a UE is configured with MR-DC: Q1: Will RAN4 specify UE requirements on; - the total number of measurable objects across LTE and NR? - the total number of configurable measurement events across LTE and NR? Q2: if the answer to Q1-a) is Yes, and if both the MN and SN separately configure a measurement object on the same carrier frequency (e.g. the MN eNB RRC configures an inter-RAT NR measurement on a given carrier and the SN gNB RRC configures an intra-RAT NR measurements on the same - serving or non-serving - carrier frequency), should it be counted as 1 or 2 measured objects? Q3: Would the answer to Q2 be dependent on differences in configuration of the measurement object? Q4: If MN and SN are to separately configure a measurement object on the same carrier frequency as in Q2, which parameters need to be configured with the same value (i.e., would need to be coordinated between the MN and SN) and which can be allowed to differ, in order to regard the two measurement object configurations from both MN and SN as one measurement object? - For example, the parameters included in E-UTRA measurement object are listed in Annex. - Any other parameters to be specified for NR, if any. Q5: In addition to Q1, will RAN4 specify additional UE requirements for which the UE requirement across inter-RATs is not the union of the one for each RAT (like the number of measurable carriers)?

In last RAN4 NR Adhoc#2 meeting, RAN4 has discussed and concluded on the first 2 question. The related replies are copied as follows [2].

Q1: Will RAN4 specify UE requirements on; - the total number of measurable objects across LTE and NR? - the total number of configurable measurement events across LTE and NR? Reply: - RAN4 specifies the minimum requirements with the total number of frequency layers. - RAN4 will further discuss how the total number of frequency layers should be specified, i.e., per RAT, across LTE and NR, and/or across all the supported RAT-s. - The terminology of measurable objects will be not used in RAN4 minimum requirements.. - RAN4 specifies the minimum requirements with the total number of reporting criteria. - RAN4 will further discuss how the total number of reporting criteria should be specified, i.e., per RAT, across LTE and NR, and/or across all the supported RAT-s. Q2: if the answer to Q1-a) is Yes, and if both the MN and SN separately configure a measurement object on the same carrier frequency (e.g. the MN eNB RRC configures an inter-RAT NR measurement on a given carrier and the SN gNB RRC configures an intra-RAT NR measurements on the same - serving or non-serving - carrier frequency), should it be counted as 1 or 2 measured objects? Reply: If the multiple measurement objects refer to the same NR carrier frequency, the UE can measure the carrier frequency with a single measurement for some of measurement object configurations. In that case, aligned with reply to question 1 on RAN4 terminology, RAN4 view is that the separately configured measurement objects on the same carrier frequency can be counted as 1 frequency layer for some of measurement object configurations. RAN4 will discuss further if there are conditions regarding differences in the measurement object configurations.

In this contribution, we further discuss the remaining questions in the LS and provide some discussion on UE measurement capability across LTE and NR.

- Discussion

In last meeting, RAN4 agreed that the UE measurement capabilities are defined as minimum number of carrier frequency layers. And RAN4 will further discuss how the total number of frequency layers and reporting criteria should be specified, i.e., per RAT, across LTE and NR, and/or across all the supported RAT-s.

In LTE, the UE measurement capabilities requirements of the total number of frequency layers and reporting criteria is specified both in per RAT and across all the supported RATs in TS36.133 section 4 and section 8 [3], e.g. UE should monitor at least 7 frequency layers for all RATs and depending on UE capability UE should monitor at least 3 EUTRA FDD frequency layers, 3 EUTRA TDD frequency layers, 3 UTRA FDD frequency layers. With the similar approach, the NR UE measurement capabilities requirements can be specified at least per RAR and across all supported RATs.

Proposal 1: The NR UE measurement capabilities requirements can be specified at least per RAR and across all supported RATs.

For NSA NR, RAN2 has been investigating that MN and SN separately configure measurement objects on the same frequency carrier, RAN4 agreed in last meeting that the UE can measure the carrier frequency with a single measurement for some of measurement object configurations. Thus in terms of UE measurement capabilities, the total number of measured carriers needs to be coordinated between the MN and SN so that it does not go beyond the UE capability. It is beneficial to coordinate MN and SN to define the UE measurement capability requirements, and this could allow UE to monitor more frequency carriers. Based on above considerations, we propose that the UE measurement capabilities requirements can be specified across LTE and NR for NSA scenarios.

Proposal 2: The UE measurement capabilities requirements can be specified across LTE and NR for NSA scenarios.

Regarding the question 4 in the LS, RAN2 asked RAN4 to identify which parameters need to be configured with the same value and which can be allowed to be different. As we discussed in last meeting, if UE L1 measurement behaviours are the same for two measurement configurations at one time, it should be counted as one measurement objective. Thus all the parameters related to the L1 measurement behaviours should be the same. In following table 1, we provide our views on whether these parameters should be the same or not.

| Parameters | Descriptions | Whether the value of parameters should be the same or not? |
| --- | --- | --- |
| E-UTRA carrier frequency | Identifies E UTRA carrier frequency for which this configuration is valid. | Shall be the same |
| Allowed measurement bandwidth | indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 | Shall be the same |
| Presence of antenna port 1 | indicate whether all the neighbouring cells use Antenna Port 1. | Shall be the same |
| Neighbour cell configuration | provide the information related to MBSFN and TDD UL/DL configuration of neighbour cells. | Shall be the same |
| Offset to the carrier frequency | Offset value applicable to the carrier frequency. | Shall be the same |
| Cell list | List of cells to add/ modify in the cell list. | Shall be the same |
| Black list | List of cells to add/ modify in the black list of cells. | Shall be the same |
| Cell for which to report CGI | Target cell to report CGI | Shall be the same |
| Measurement cycle of SCell | Measurement cycle for deactivated SCell(s) | Shall be the same Applied for CA |
| Wideband RSRQ measurements | RSRQ measurement bandwidth wider than 6 PRBs | Shall be the same |
| Cells to apply alternative TTT | Target cells to apply different TTT | Could be different |
| T312 | Short RLM timer | Could be different |
| Reduced measurement performance | Indicate to measure with reduced performance | Shall be the same Applied for IncMon |
| Measurement DS configuration | Discovery Signal configuration for measurements | Shall be the same Applied for small cell |
| White cells | Target cell to report measurements | Shall be the same |
| RMTC configuration | RSSI measurement timing configuration | Shall be the same Applied for LAA |

Proposal 3: In order to count the two measurement objects on the same carrier frequency configured from MN and SN as one measurement object, all the parameters related to the L1 measurement behaviours should be the same value.

- 3	Conclusion

In this paper, we further discuss the remaining questions in the LS and provide some discussion on UE measurement capability across LTE and NR. And we have the following proposals:

Proposal 1: The NR UE measurement capabilities requirements can be specified at least per RAR and across all supported RATs.

Proposal 2: The UE measurement capabilities requirements can be specified across LTE and NR for NSA scenarios.

Proposal 3: In order to count the two measurement objects on the same carrier frequency configured from MN and SN as one measurement object, all the parameters related to the L1 measurement behaviours should be the same value.

| Parameters | Descriptions | Whether the value of parameters should be the same or not? |
| --- | --- | --- |
| E-UTRA carrier frequency | Identifies E UTRA carrier frequency for which this configuration is valid. | Shall be the same |
| Allowed measurement bandwidth | indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 | Shall be the same |
| Presence of antenna port 1 | indicate whether all the neighbouring cells use Antenna Port 1. | Shall be the same |
| Neighbour cell configuration | provide the information related to MBSFN and TDD UL/DL configuration of neighbour cells. | Shall be the same |
| Offset to the carrier frequency | Offset value applicable to the carrier frequency. | Shall be the same |
| Cell list | List of cells to add/ modify in the cell list. | Shall be the same |
| Black list | List of cells to add/ modify in the black list of cells. | Shall be the same |
| Cell for which to report CGI | Target cell to report CGI | Shall be the same |
| Measurement cycle of SCell | Measurement cycle for deactivated SCell(s) | Shall be the same Applied for CA |
| Wideband RSRQ measurements | RSRQ measurement bandwidth wider than 6 PRBs | Shall be the same |
| Cells to apply alternative TTT | Target cells to apply different TTT | Could be different |
| T312 | Short RLM timer | Could be different |
| Reduced measurement performance | Indicate to measure with reduced performance | Shall be the same Applied for IncMon |
| Measurement DS configuration | Discovery Signal configuration for measurements | Shall be the same Applied for small cell |
| White cells | Target cell to report measurements | Shall be the same |
| RMTC configuration | RSSI measurement timing configuration | Shall be the same Applied for LAA |

- 4	Reference

[1]	R2-1706140	LS on UE measurement capabilities across LTE and NR, RAN2

[2]	R4-1706905	Reply LS on UE measurement capability across LTE and NR, RAN4

[3]	TS36.133