3GPP TSG-WG RAN4 Meeting #84 	R4-1707602

# Berlin, Germany, Date 21st  – 25th Aug, 2017

Source: 	Nokia

Title: 					NB-IoT A-MPR

Agenda Item: 			7.8.2

Document for:	Discussion

# 1	Introduction

In previous RAN meeting a WF [1] was agreed to Evaluate and, if appropriate, specify new network signalling value(s) for NB-IoT additional SEM for guard-band deployment in Rel-14.

In this contribution we present simulation results for potential A-MPR to assist UEs to meet the A-SEM and have a proposal for A-SEM.

# 2	Discussion

We found that the combined spectrum emission mask given in [2] is incorrect. There is a 7.5 kHz error in the lowest frequency offset that corresponds to the edge of the LTE UL SEM. Fig 1 illustrates the involved bandwidths and frequency gaps. The corrected SEM is given in Table 1.

Figure 1: Bandwidths and frequency gaps that determine the combined SEM

Table 1: Additional NB-IoT spectrum emission mask for upper guard 
band deployment with 105 kHz gap between RBs of LTE and NB-IoT

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 205 | -15.7 | 30 kHz |
| 205-221.4 | -18 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Our simulation showed that no A-MPR is necessary to meet the A-SEM in Table 1.

Some plots in Figure 2..

Figure 2: Spectrum detail of 3.75 kHz NB-IoT pi/2-BPSK signal with a tone at the upper channel edge and 12 tone QPSK-signal

# 3	Conclusion

In this contribution we have presented an A-SEM proposal for the priority case when NB-IoT carrier is deployed in the right-hand side guard-band of a 10 MHz LTE carrier with a gap of 105 kHz between the NB-IoT RB and the LTE edge RB. For this case we have run simulations that indicate that no A-MPR is necessary.

# 4	References

[1] R4-1705868, WF on NB-IoT additional spectrum emission mask, Neul, Huawei, HiSilicon, RAN4#83

[2] R4-1705579, “NB-IoT Additional Spectrum Emission Mask,” Neul, Huawei, HiSilicon , RAN4#83, Hangzhou, May 2017