3GPP TSG-RAN WG4 Meeting #84                                                R4-1707635

Berlin, Germany, 21-25 Aug 2017

Source:		Ericsson

Title:			Inter-site BS synchronization requirements for 200 µs CP (1.25 kHz)

Agenda item:		7.16.4

Document for:	Discussion

# 1 Introduction

It was decided to capture MBMS synchronization requirements as requirements on eNB-eNB synchronization [1] for FeMBMS, MBSFN areas with 200 µs CP and 1.25 kHz subcarrier spacing.  In this contribution, we forward some observations and propose a requirement.

# 2 Discussion

MBSFN relies on synchronized transmission of frames from several eNB in an MBSFN area. The canonical example with two eNB is shown below:

Rx

Ry

Figure 1: MBSFN dimensioning case

The MBSFN case is the dimensioning case. The requirement dimensioning relation is the same for all MBSFN cases of CP and can be described as follows:

ΔTBS+|Rx-Ry|c+ΔTChannel<TCP

Where:

- ΔTBS is the network eNB synchronization uncertainty, R is the cell radius, c is the speed of light, TChannel  is the radio channel excess delay and TCP is the Channel Prefix.

- The differences in signal arrival time at the UE between the sites contributing to MBSFN cells |R_x-R_y | adds 10 µs per 3 km.

- ΔTChannel  is the delay spread of the channel.

- TCP is 200 µs for FeMBMS MBFSN with 1.25 kHz subcarrier and 16.67 µs otherwise.

In [2,3] Huawei analyzed the MBSFN case with 16.67 µs and arrived that  ΔTBS = 11 µs was sufficient. (In the end, the requirement was settled at 5 µs).

If we reuse the analysis in [2] and set the FeMBMS requirement at ΔTBS= 10 µs (MBSFN Cell Phase Synchronization) we would still have 190 µs to handle the differences in signal arrival time + the delay spread of the channel.

Proposal: Define Cell Phase Synchronization requirement to be 10 µs, for 200 µs CP for MBFSN with 1.25 kHz subcarrier.

# 3 Conclusion

Proposal: Define Cell Phase Synchronization requirement to be 10 µs, for 200 µs CP for MBFSN with 1.25 kHz subcarrier.

# 4 References

- Draft_MeetingReport_RAN_76_170608_eom.zip.

- R4-167774, Further discussion on requirements of inter-cell synchronization for MBMS, Huawei, HiSilicon.

- R4-1701346, CR on Introducing inter-cell synchronization requirements for MBMS R14, Huawei, HiSilicon.

- R4-1705035, Consideration on RRM measurement requirements for eMBMS enhancements, Huawei, HiSilicon