3GPP TSG-RAN WG4 Meeting #84	R4-1707656

Berlin, Germany, 21 – 25 August 2017

Agenda Item:	6.2.4.2

Source:	Ericsson

Title:	Simulation result for Non-BL CE UE MPDCCH demodulation

Document for:	Discussion

# 1	Introduction

RAN4#83 agreed with the way forward on requirements for Rel-13 Non-BL CE UEs:

- Define new UE demodulation and CQI requirements for non-BL UE supporting coverage enhancements with 2Rx/4Rx using full antenna connection (i.e., Option 2) from Rel-13.

  - UE demodulation requirements

    - Reuse setup for category M1 (except repetition numbers) specified for Rel-13 Cat-M1 UE requirements.

    - Repetition numbers would be adjusted to meet considering the target SNR for CE Mode A (-6dB) and CE mode B (-15dB).

      - Option 1: Adopt a half and a quarter of repetition number for category M1 to define requirements for other categories with 2 Rx and 4Rx respectively.

      - Other options are not precluded.

This contribution presents our simulation results for MPDCCH considering 2Rx/4Rx UE supporting coverage enhancements.

# 2	Simulation results

## 2.1	CE Mode A

Figure 1 shows the simulation results of MPDSCH CE Mode A according to the parameters specified in Rel-13 TS36.101 8.11.2. For 1Rx case, the MPDCCH repetition numbers for FDD and TDD are set to 32 and 16, respectively.

Figure 1	Simulation results of MPDCCH CE Mode A for 1Rx/2Rx/4Rx.

For 2Rx UE, it is observed from the simulation results that the repetition number 4 can achieve MPDCCH BLER=1% around SNR=-6dB for both FDD and TDD. For 4Rx UE, it is observed that the repetition number 1 can achieve MPDCCH BLER=1% around -6dB for both FDD and TDD.

We therefore propose to set the repetition numbers for MPDCCH CE Mode A as follows:

|  | FDD | TDD |
| --- | --- | --- |
| 1Rx (Rel-13) | 32 | 16 |
| 2Rx | 4 | 4 |
| 4Rx | 1 | 1 |

## 2.2	CE Mode B

Figure 2 shows the simulation results of MPDSCH CE Mode B according to the parameters specified in Rel-13 TS36.101 8.11.2. For 1Rx case, the MPDCCH repetition numbers for FDD and TDD are set to 64 and 32, respectively.

Figure 2	Simulation results of MPDCCH CE Mode B for 1Rx/2Rx/4Rx.

For 2Rx UE, it is observed from the simulation results that the repetition numbers 32 and 16 can achieve MPDCCH BLER=1% around SNR=-15dB for FDD and TDD, respectively. For 4Rx UE, it is observed that the repetition numbers 8 and 4 can achieve MPDCCH BLER=1% around -15dB for FDD and TDD, respectively.

We therefore propose to set the repetition numbers for MPDCCH CE Mode B as follows:

|  | FDD | TDD |
| --- | --- | --- |
| 1Rx (Rel-13) | 64 | 32 |
| 2Rx | 32 | 16 |
| 4Rx | 8 | 4 |

# 3	Conclusion

Proposal: Set the MPDCCH repetition numbers for non-BL CE UE as follows:

|  | CE Mode A |  | CE Mode B |  |
| --- | --- | --- | --- | --- |
|  | FDD | TDD | FDD | TDD |
| 1Rx (Rel-13) | 32 | 16 | 64 | 32 |
| 2Rx | 4 | 4 | 32 | 16 |
| 4Rx | 1 | 1 | 8 | 4 |

# 4	References

R4-1706196, “WF on requirements for Rel-13 Non-BL UE”, Huawei, HiSilicon, Ericsson, Intel Corporation, Qualcomm.