3GPP TSG-RAN WG4 Meeting #84	R4-1707673

Berlin, Germany, 21 – 25 August 2017

Agenda Item:	7.9.2

Source:	Ericsson, Anritsu, Sony

Title:	FRC tables for UE Cat-M2 RF requirements

Document for:	Discussion

# 1	Introduction

RAN4#83 completed the Rel-14 FeMTC RF core part. The remaining issue in RAN4 is to specify the FRC table corresponding Cat-M2 UE. In this contribution, we propose the UE Cat-M2 FRC tables by reusing the Cat-M1 FRC tables.

First, we highlight the difference of UE Cat-M2 CE Mode A from Cat-M1:

- UE Cat-M2 supports wider bandwidth PDSCH transmission up to 4 narrowbands.

  - For 1.4MHz system bandwidth, 1 narrowband, i.e., 6PRBs, can be allocated.

  - For 3MHz system bandwidth, up to 2 narrowbands, i.e. 12PRBs, can be allocated.

  - For 5MHz/10MHz/15MHz/20MHz system bandwidth, up to 4 narrowbands, i.e., 24PRBs, can be allocated.

- PUSCH

  - Maximum UL TBS: 6,968 bits (See TS 36.306)

  - TBS range:  (See TS 36.213 8.6.1)

  - PRB allocation (DCI format 6-0A)

    - For allocating 2 or more narrowbands, UL resource allocation type 4 is used.

      - Possible to allocate 9, 12, 15, 18, 21, and 24PRB.

    - For allocating 1 narrowband, UE resource allocation type 0 is used

      - Possible to allocate 1, 2, 3, 4, 5, and 6PRBs.

- PDSCH

  - Maximum DL TBS: 4,008 bits (See TS 36.306)

  - TBS range:  (See TS 36.213 7.1.7.2)

  - PRB allocation (DCI format 6-1A)

    - For allocating 2 or more narrowbands, first indicate the number of narrowbands (2, 3, or 4), and the resource allocation within each used narrowband will be identical.

      - This means the total number of allocated PRBs are given by:
  (PRBs in each narrowband) x (number of narrowbands)

    - For allocating 1 narrowband, same as Rel-13.

      - Possible to allocate 1, 2, 3, 4, 5, and 6PRBs.

# 2	Proposal for FRC tables

## 2.1	UL FRC tables

For UL FRC tables, we reuse FRC for Cat-M1 and extend the PRB sizes to 12 for 3MHz channel bandwidth and 24 for 5MHz/10MHz/15MHz/20MHz channel bandwidth. The payload sizes are adjusted accordingly to keep the same target coding rate.

## A.2.2	Reference measurement channels for FDD

Table A.2.2.1.1-1x Reference Channels for QPSK with full/maximum RB allocation for UE UL category M2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 12 | 24 | 24 | 24 | 24 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 600 | 1224 | 2472 | 2472 | 2472 | 2472 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (NOTE 1) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame | Bits | 1728 | 3456 | 6912 | 6912 | 6912 | 6912 |
| Total symbols per Sub-Frame |  | 864 | 1728 | 3456 | 3456 | 3456 | 3456 |
| UE UL Category |  | M2 | M2 | M2 | M2 | M2 | M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |

Table A.2.2.1.2-1x: Reference Channels for 16-QAM with maximum RB allocation for UE UL category M2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 12 | 24 | 24 | 24 | 24 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size | Bits | 1032 | 2088 | 4264 | 4264 | 4264 | 4264 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame | Bits | 3456 | 6912 | 13824 | 13824 | 13824 | 13824 |
| Total symbols per Sub-Frame |  | 864 | 1728 | 3456 | 3456 | 3456 | 3456 |
| UE Category |  | M2 | M2 | M2 | M2 | M2 | M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |

Table A.2.2.2.1-1x: Reference Channels for QPSK with partial RB allocation for UE UL category M2

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Trans-port block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | M2 |
|  | 1.4 - 20 | 2 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | M2 |
|  | 1.4 - 20 | 3 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | M2 |
|  | 1.4 - 20 | 4 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | M2 |
|  | 1.4 - 20 | 5 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | M2 |
|  | 3-20 | 6 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | M2 |
|  | 3-20 | 9 | 12 | QPSK | 1/3 | 776 | 24 | 1 | 2592 | 1296 | M2 |
|  | 3-20 | 12 | 12 | QPSK | 1/3 | 1032 | 24 | 1 | 3456 | 1728 | M2 |
|  | 5-20 | 15 | 12 | QPSK | 1/3 | 1320 | 24 | 1 | 4320 | 2160 | M2 |
|  | 5-20 | 18 | 12 | QPSK | 1/3 | 1864 | 24 | 1 | 5184 | 2592 | M2 |
|  | 5-20 | 21 | 12 | QPSK | 1/3 | 2216 | 24 | 1 | 6068 | 3024 | M2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD UE, the uplink subframes are scheduled at the 5th, 6th and 7th subframes every 10ms for the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2.2-1x Reference Channels for 16-QAM with partial RB allocation for UE UL category M2

| Parameter | Ch BW | Allocated RBs | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame | Total symbols per Sub-Frame | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 12 | 16QAM | 1/2 | 256 | 24 | 1 | 576 | 144 | M2 |
|  | 1.4 - 20 | 2 | 12 | 16QAM | 1/2 | 552 | 24 | 1 | 1152 | 288 | M2 |
|  | 1.4 - 20 | 3 | 12 | 16QAM | 1/2 | 840 | 24 | 1 | 1728 | 432 | M2 |
|  | 1.4 - 20 | 4 | 12 | 16QAM | 1/2 | 1128 | 24 | 1 | 2304 | 576 | M2 |
|  | 1.4 - 20 | 5 | 12 | 16QAM | 1/2 | 1416 | 24 | 1 | 2880 | 720 | M2 |
|  | 3 - 20 | 6 | 12 | 16QAM | 1/2 | 1736 | 24 | 1 | 3456 | 864 | M2 |
|  | 3 - 20 | 9 | 12 | 16QAM | 1/2 | 2600 | 24 | 1 | 5184 | 1296 | M2 |
|  | 3 - 20 | 12 | 12 | 16QAM | 1/2 | 3496 | 24 | 1 | 6912 | 1728 | M2 |
|  | 5 - 20 | 15 | 12 | 16QAM | 1/2 | 4264 | 24 | 1 | 8640 | 2160 | M2 |
|  | 5 - 20 | 18 | 12 | 16QAM | 1/2 | 5160 | 24 | 1 | 10368 | 2592 | M2 |
|  | 5 - 20 | 21 | 12 | 16QAM | 1/2 | 5992 | 24 | 1 | 12096 | 3024 | M2 |
|  | 5 - 20 | 24 | 12 | 16QAM | 1/2 | 6968 | 24 | 2 | 13824 | 3456 | M2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 2: For HD-FDD UE, the uplink subframes are scheduled at the 4th, 5th and 6th subframes every 10ms for the channel bandwidth 5MHz/10MHz/15MHz/20MHz. For HD-FDD , the uplink subframes are scheduled at the 5th, 6th, and 7th subframes every 10msfor the channel bandwidth 1.4MHz/3MHz. Information bit payload is available if uplink subframe is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

## A.2.3	Reference measurement channels for TDD

Table A.2.3.1.1-1x Reference Channels for QPSK with full/maximum RB allocation for UE UL category M2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 12 | 24 | 24 | 24 | 24 |
| Uplink-Downlink Configuration (Note 2) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 | Bits | 600 | 1224 | 2472 | 2472 | 2472 | 2472 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 | Bits | 1728 | 3456 | 6912 | 6912 | 6912 | 6912 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 |  | 864 | 1728 | 3456 | 3456 | 3456 | 3456 |
| UE UL Category |  | M2 | M2 | M2 | M2 | M2 | M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 |  |  |  |  |  |  |  |

Table A.2.3.1.2-1x: Reference Channels for 16-QAM with maximum RB allocation for UE UL category M2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 12 | 24 | 24 | 24 | 24 |
| Uplink-Downlink Configuration (Note 2) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 | Bits | 1032 | 2088 | 4264 | 4264 | 4264 | 4264 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (Note 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 | Bits | 3456 | 6912 | 13824 | 13824 | 13824 | 13824 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,3,7,8 |  | 864 | 1728 | 3456 | 3456 | 3456 | 3456 |
| UE Category |  | M2 | M2 | M2 | M2 | M2 | M2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211[4] |  |  |  |  |  |  |  |

Table A.2.3.2.1-1x: Reference Channels for QPSK with partial RB allocation for UE UL category M2

| Parameter | Ch BW | Allocated RBs | UDL Configuration (Note 2) | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size for Sub-Frame 2, 3, 7, 8 | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 3, 7, 8 | Total symbols per Sub-Frame for Sub-Frame 2, 3, 7, 8 | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 1 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | M2 |
|  | 1.4 - 20 | 2 | 1 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | M2 |
|  | 1.4 - 20 | 3 | 1 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | M2 |
|  | 1.4 - 20 | 4 | 1 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | M2 |
|  | 1.4 - 20 | 5 | 1 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | M2 |
|  | 3-20 | 6 | 1 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | M2 |
|  | 3-20 | 9 | 1 | 12 | QPSK | 1/3 | 776 | 24 | 1 | 2592 | 1296 | M2 |
|  | 3-20 | 12 | 1 | 12 | QPSK | 1/3 | 1032 | 24 | 1 | 3456 | 1728 | M2 |
|  | 5-20 | 15 | 1 | 12 | QPSK | 1/3 | 1320 | 24 | 1 | 4320 | 2160 | M2 |
|  | 5-20 | 18 | 1 | 12 | QPSK | 1/3 | 1864 | 24 | 1 | 5184 | 2592 | M2 |
|  | 5-20 | 21 | 1 | 12 | QPSK | 1/3 | 2216 | 24 | 1 | 6068 | 3024 | M2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: As per Table 4.2-2 in TS 36.211 [4]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2.2-1x: Reference Channels for 16QAM with partial RB allocation UE UL category M2

| Parameter | Ch BW | Allocated RBs | UDL Configuration (Note 2) | DFT-OFDM Symbols per Sub-Frame | Mod’n | Target Coding rate | Payload size for Sub-Frame 2, 3, 7, 8 | Transport block CRC | Number of code blocks per Sub-Frame (Note 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 3, 7, 8 | Total symbols per Sub-Frame for Sub-Frame 2, 3, 7, 8 | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 1 | 12 | 16QAM | 1/2 | 256 | 24 | 1 | 576 | 144 | M2 |
|  | 1.4 - 20 | 2 | 1 | 12 | 16QAM | 1/2 | 552 | 24 | 1 | 1152 | 288 | M2 |
|  | 1.4 - 20 | 3 | 1 | 12 | 16QAM | 1/2 | 840 | 24 | 1 | 1728 | 432 | M2 |
|  | 1.4 - 20 | 4 | 1 | 12 | 16QAM | 1/2 | 1128 | 24 | 1 | 2304 | 576 | M2 |
|  | 1.4 - 20 | 5 | 1 | 12 | 16QAM | 1/2 | 1416 | 24 | 1 | 2880 | 720 | M2 |
|  | 3 - 20 | 6 | 1 | 12 | 16QAM | 1/2 | 1736 | 24 | 1 | 3456 | 864 | M2 |
|  | 3 - 20 | 9 | 1 | 12 | 16QAM | 1/2 | 2600 | 24 | 1 | 5184 | 1296 | M2 |
|  | 3 - 20 | 12 | 1 | 12 | 16QAM | 1/2 | 3496 | 24 | 1 | 6912 | 1728 | M2 |
|  | 5 - 20 | 15 | 1 | 12 | 16QAM | 1/2 | 4264 | 24 | 1 | 8640 | 2160 | M2 |
|  | 5 - 20 | 18 | 1 | 12 | 16QAM | 1/2 | 5160 | 24 | 1 | 10368 | 2592 | M2 |
|  | 5 - 20 | 21 | 1 | 12 | 16QAM | 1/2 | 5992 | 24 | 1 | 12096 | 3024 | M2 |
|  | 5 - 20 | 24 | 1 | 12 | 16QAM | 1/2 | 6968 | 24 | 2 | 13824 | 3456 | M2 |
| Note 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 2: As per Table 4.2-2 in TS 36.211 [4]. |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.1	DL FRC tables

For DL FRC tables used for REFSENS and maximum input level test, we reuse FRC for Cat-M2, considering the following rules:

- 1 CRS antenna port.

- The number of PRBs for 3MHz system BW are 12. It consists of 2 narrowbands and each narrowband contains 6 PRBs.

- Considering the simultaneous transmission of MPDCCH and PDSCH in wideband, we propose to assign 2PRB for MPDCCH. According to the PRB assignment for PDSCH specified in TS36.212, each narrowband should have the same PRB allocation. This means 4 PRBs are assigned for PDSCH for each narrowband in the wideband. The figure below illustrates an example of MPDCCH and PDSCH allocation.

  - For 1.4MHz channel BW, 1 narrowband is assigned and 4 PRBs are allocated for PDSCH

  - For 3MHz channel BW, 2 narrowbands are assigned ad 4 PRBs in each narrowband are allocated for PDSCH. In total 8 PRBs are allocated for PDSCH.

  - For 5MHz/10MHz/15MHz/20MHz channel BW, 4 narrowbands are assigned ad 4 PRBs in each narrowband are allocated for PDSCH. In total 16 PRBs are allocated for PDSCH.

- The payload sizes are adjusted accordingly to keep the same target coding rate.

## A.3.2	Reference measurement channel for receiver characteristics

Table A.3.2-1x Fixed Reference Channel for Receiver Requirements (FDD and HD-FDD) – for CAT-M2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks (Note 6) |  | 4 | 8 | 16 | 16 | 16 | 16 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated subframes per Radio Frame |  | 2 | 2 | 8 | 8 | 8 | 8 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Number of HARQ Processes | Processes | 8 | 8 | 8 | 8 | 8 | 8 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,8 | Bits | 256 | 680 | 1384 | 1384 | 1384 | 1384 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A | N/A | 1384 | 1384 | 1384 | 1384 |
| For Sub-Frame 4 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 6 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,8 | Bits | 1 | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A | N/A | 1 | 1 | 1 | 1 |
| For Sub-Frame 4 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 6 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,8 | Bits | 912 | 2016 | 4032 | 4416 | 4416 | 4416 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A | N/A | 4032 | 4416 | 4416 | 4416 |
| For Sub-Frame 4 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 6 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| Max. Throughput averaged over 1 frame for FDD | kbps | 51.2 | 136.0 | 1107.2 | 1107.2 | 1107.2 | 1107.2 |
| Max. Throughput averaged over 1 frames for HD-FDD | kbps | 25.6 | 68.0 | 415.2 | 415.2 | 415.2 | 415.2 |
| UE DL Category |  | M2 | M2 | M2 | M2 | M2 | M2 |
| Note 1: 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10MHz channel BW. 3 symbols allocated to PDCCH for 5 MHz and 3 MHz. 4 symbols allocated to PDCCH for 1.4 MHz Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211. Note 3: The scheduled narrowband other than 1.4MHz and 3MHz channel bandwidth avoids the centre of the channel where some REs of the same PRBs are occupied by PBCH and synchronization signals. Note 4: For HD-FDD UE, PDSCH are scheduled at the 3rd subframe every 1 radio frame for 1.4MHz and 3MHz channel bandwidth. For other channel bandwidth, PDSCH are scheduled at the 0th, 1st and 2nd subframes every 1 radio frame. Information bit payload is available if downlink subframe is scheduled. The corresponding MPDCCH is scheduled 2 subframes before the corresponding PDSCH transmission. Note 5: 2 resource blocks allocated to MPDCCH. Note 6: 4 resource blocks in each narrowband allocated to PDSCH. |  |  |  |  |  |  |  |

Table A.3.2-2x Fixed Reference Channel for Receiver Requirements (TDD) – for CAT-M2

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel Bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks (Note 8) |  | 4 | 8 | 16 | 16 | 16 | 16 |
| Uplink-Downlink Configuration (Note 5) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated subframes per Radio Frame (D) |  | 3 | 3 | 3 | 3 | 3 | 3 |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmission |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload per Sub-Frame | Bits |  |  |  |  |  |  |
| For Sub-Frame 4, 9 |  | 256 | 680 | 1384 | 1384 | 1384 | 1384 |
| For Sub-Frame 1, 6 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 256 | 680 | 1384 | 1384 | 1384 | 1384 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (Note 4) |  |  |  |  |  |  |  |
| For Sub-Frame 4, 9 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frame 1, 6 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Binary Channel Bits Per Sub-Frame | Bits |  |  |  |  |  |  |
| For Sub-Frame 4, 9 |  | 912 | 2016 | 4032 | 4416 | 4416 | 4416 |
| For Sub-Frame 1, 6 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 912 | 2016 | 4032 | 4416 | 4416 | 4416 |
| Max. Throughput averaged over 1 frame | kbps | 76.8 | 204.0 | 415.2 | 415.2 | 415.2 | 415.2 |
| UE DL Category |  | M2 | M2 | M2 | M2 | M2 | M2 |
| Note 1: For normal subframes(0,4,5,9), 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. Note 2: No data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance Note 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [4] Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: As per Table 4.2-2 in TS 36.211 [4] Note 6: For Sub-Frame 0, the scheduled narrowband avoids the centre of the channel where some REs of the same PRBs are occupied by PBCH and synchronization signals. Note 7: 2 resource blocks allocated to MPDCCH Note 8: 4 resource blocks in each narrowband allocated to PDSCH. |  |  |  |  |  |  |  |

Table A.3.2-3x Fixed Reference Channel for Maximum input level for UE DL Category M2 (FDD and HD-FDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks (Note 6) |  | 2 | 8 | 15 | 15 | 15 | 15 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated subframes per Radio Frame (Note 6) |  | 2 | 2 | 8 | 8 | 8 | 8 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 3/5 | 3/5 | 3/5 | 3/5 | 3/5 | 3/5 |
| Number of HARQ Processes | Processes | 8 | 8 | 8 | 8 | 8 | 8 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload |  |  |  |  |  |  |  |
| For Sub-Frames 3, 8 | Bits | 552 | 2280 | 4008 | 4008 | 4008 | 4008 |
| For Sub-Frames 0,1,2,5,7,9 | Bits | N/A | N/A | 4008 | 4008 | 4008 | 4008 |
| For Sub-Frame 4 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 6 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3, 8 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frames 0,1,2,5,7,9 |  | N/A | N/A | 1 | 1 | 1 | 1 |
| For Sub-Frame 4 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 6 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| Binary Channel Bits Per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3, 8 | Bits | 912 | 3648 | 6840 | 6840 | 6840 | 6840 |
| For Sub-Frames 0,1,2,5,7,9 |  | N/A | N/A | 6840 | 6840 | 6840 | 6840 |
| For Sub-Frame 4 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 6 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| Max. Throughput averaged over 1 frame for FDD | kbps | 110.4 | 456.0 | 3206.4 | 3206.4 | 3206.4 | 3206.4 |
| Max. Throughput averaged over 1 frame for HD-FDD |  | 55.2 | 228.0 | 1202.4 | 1202.4 | 1202.4 | 1202.4 |
| Note 1: 4 symbols allocated to PDCCH for all channel bandwidths. Note 2: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211. Note 3: The scheduled narrowband other than 1.4MHz and 3MHz channel bandwidth avoids the centre of the channel where some REs of the same PRBs are occupied by PBCH and synchronization signals. Note 4: For HD-FDD UE, PDSCH are scheduled at the 3rd subframe every 1 radio frame for 1.4MHz and 3MHz channel bandwidth. For other channel bandwidth, PDSCH are scheduled at the 0th, 1st, and 2nd subframes every 1 radio frame. Information bit payload is available if downlink subframe is scheduled. The corresponding MPDCCH is scheduled 2 subframes before the corresponding PDSCH transmission. Note 5: 2 resource blocks allocated to MPDCCH. Note 6: 2 resource blocks allocated to PDSCH for 1.4MHz channel bandwidth. 2 narrowbands and 4 resource blocks in each narrowband allocated to PDSCH for 3MHz channel bandwidth. For 5MHz/10MHz/15MHz/20MHz channel bandwidth, configure 3 narrowbands and 5 resource blocks in each narrowband allocated to PDSCH. |  |  |  |  |  |  |  |

Table A.3.2-4x Fixed Reference Channel for Maximum input level for UE DL Category M2 (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks (Note 7) |  | 2 | 8 | 15 | 15 | 15 | 15 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Uplink-Downlink Configuration (Note 5) |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated subframes per Radio Frame |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 3/5 | 3/5 | 3/5 | 3/5 | 3/5 | 3/5 |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 552 | 2280 | 4008 | 4008 | 4008 | 4008 |
| For Sub-Frames 1,6 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (Note 4) |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frames 1,6 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| Binary Channel Bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 4,9 | Bits | 912 | 4032 | 7560 | 8280 | 8280 | 8280 |
| For Sub-Frames 1,6 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| Max. Throughput averaged over 1 frame | kbps | 110.4 | 456.0 | 801.6 | 801.6 | 801.6 | 801.6 |
| Note 1: For normal subframes(0,4,5,9), 4 symbols allocated to PDCCH for all channel bandwidths. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. Note 2: For 1.4MHz, no data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance. Note 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: As per Table 4.2-2 in TS 36.211 [4]. Note 6: 2 resource blocks allocated to MPDCCH. Note 7: 2 resource blocks allocated to PDSCH for 1.4MHz channel bandwidth. 2 narrowbands and 4 resource blocks in each narrowband allocated to PDSCH for 3MHz channel bandwidth. For 5MHz/10MHz/15MHz/20MHz channel bandwidth, configure 3 narrowbands and 5 resource blocks in each narrowband allocated to PDSCH. |  |  |  |  |  |  |  |

# Appendix

For PDSCH (TS36.213 7.1.7.2):

[…]

else if the UE supports ce-pdsch-pusch-maxBandwidth with value ≥5MHz

-	the UE shall first determine the TBS index () usingand Table 7.1.7.1-1

-	if the UE is configured with higher layer parameter ce-pdsch-maxBandwidth-config with value 5MHz or if the UE is configured with higher layer parameter pdsch-MaxBandwidth-SC-MTCH with value 24 PRBs

-	For CEModeA, TBS is determined by the procedure in Subclause 7.1.7.2.8 for

-	For CEModeB, TBS is determined by the procedure in Subclause 7.1.7.2.8 for

For PUSCH (TS36.213 8.6.1):

[…]

For a BL/CE UE configured with CEModeA,

-	if the UE is configured with higher layer parameter ce-pusch-maxBandwidth-config with value 5MHz, the TBS is determined by the procedure in Subclause 7.1.7.2.1, for

-	otherwise, the TBS is determined by the procedure in Subclause 7.1.7.2.1.