3GPP TSG-RAN WG4 Meeting #84	R4-1707873

Berlin, Germany, 21st - 25th August 2017

Agenda item:			9.6.4

Source:	MediaTek Inc.

Title:	Measurement Gap for CSI-RS

Document for:		Discussion

1	Introduction

In last meeting, R4 has some agreements on measurement gap, as capture below from [1].

- MGL: - For SS block based cell identification and measurement, 6ms MGL is used as baseline - The requirement shall apply provided that at least the subset of measurement gap contains the 5ms window of SS blocks; - MGRP - 40ms, 80ms could be a starting point for defining MGRP in NR - For NSA, MGRP for NR measurement shall not be smaller than 40ms. Further analysis is welcome on - Whether other MGRP value is beneficial (should not be smaller than 40ms for NSA) - Need for CSI RS based measurement in measurement gaps - Multiple layer monitoring in gaps eg - SS burst periodicity is different on different layers - SS burst offset is different on different layers - Other aspects

In this paper we discuss gap design for CSI-RS based measurements.

2	Importance of CSI-RS based measurement

It was agreed in RAN1 that CSI-RS can be used for L3 mobility reporting. [2] (for short, CSI-RS mentioned in this paper refers to CSI-RS for L3 mobility, unless specified)

Agreements: - For CONNECTED mode RRM measurement for L3 mobility, CSI-RS can be used, in addition to IDLE mode RS - Note that RAN1 will consider configuration overhead and possible inter-gNB signaling overhead - Detection of neighbor cell for measurement is based on NR-SS

In RAN2, there was similar agreement along with events A1-A6 for CSI-RS [6].

Agreements 1: CSI-RS configured for RRM purpose can be used to derive a cell level quality 2: Events A1-A6 can be configured to use CSI-RS. Events are evaluated on the cell level quality 3 Previous agreements on measurement model and cell quality derivation are also applicable for CSI-RS. 4 When the serving cell quality is above S-Measure, the UE is not required to measure the IDLR RS and CSI-RS for neighbour cells.

In our opinion the advantages of CSI-RS over SS block for measurement are

- Wider bandwidth (BW)

- Better supporting for TRP based operation

- Configurable periodicity

- Finer beam width via digital beamforming

Therefore we suggest RAN4 to put equal priority on SS block and CSI-RS when discussion corresponding RRM requirements.

Proposal 1: RAN4 to treat SS block and CSI-RS with equal priority when discussion corresponding RRM requirements.

3	Inter-frequency measurement on CSI-RS

Using CSI-RS has some fundamental differences to SS block. In the following, we try to list some of them.

- At the very first step, UE still needs to SS block for cell detection and to provide course synchronization of the timing and frequency. In our view, CSI-RS related processing is done in frequency domain (after FFT), which means the timing error should be at least within CP and the frequency error should also be controlled under a certain level compared with SCS. In other words, it is highly unlikely to have standalone CSI-RS which can be measured without any SS block. Furthermore, since UE’s clock drifts with time, the synchronization information provided by SS block has a finite valid time. In the cases of inter-frequency measurement, this means that UE prefers to measure the CSI-RS on a frequency as early as possible after UE finishes the measurement/processing on the SS block on that frequency.

Observation 1: UE needs SS block to provide course synchronization of the timing and frequency in order to measure CSI-RS.

- UE needs TTI of SS block as a reference to derive the timing of CSI-RS. This was agreed in [3], as captured below. So far, there are still some unknown detail about how the timing configuration of CSI-RS is indicated to UE, e.g., through TTI index or a fix offset to a QCL-ed SS block. In either way, UE needs to first decode the PBCH in the SS block for the information about either frame/slot of the cell or the timing index of a SS block.

Agreements: - The time synchronization reference for a CSI-RS for L3 mobility is the frame/slot/symbol timing of a cell. - Note: The frame/slot/symbol timing of the cell can be obtained from an SS block - FFS: Note: timing synchronization between CSI-RS and SS block of the cell is assured. Timing synchronization refers to frame/slot/symbol timing. - NR cell ID for time reference of CSI-RS(s) is informed to the UE

Observation 2: UE needs SS block to provide TTI reference in order to measure CSI-RS.

- CSI-RS based measurement is also possible for inter-frequency case. Although the exact signaling structure of measurement object (MO) is still under discussing in RAN2, e.g., SS block and CSI-RS on the same frequency will share the same MO or not. It is believed that corresponding information, such as slot/symbol index, measurement bandwidth and RE locations and Cell ID, should be signaled to UE in order to perform CSI-RS based measurement. Based RAN1 and RAN2 discussion so far, the slot/symbol of different CSI-RS configurations can be arbitrary values.

Observation 3: CSI-RS configuration can be provided through measurement object for the purpose of inter-frequency measurement.

- So far, RAN4 only has progress on measurement gap for SS block [7], as captured below.

- MGL: - For SS block based cell identification and measurement, 6ms MGL is used as baseline - The requirement shall apply provided that at least the subset of measurement gap contains the 5ms window of SS blocks; - CSI-RSRP measurement FFS - MGRP - 40ms, 80ms could be a starting point for defining MGRP in NR - For NSA, MGRP for NR measurement shall not be smaller than 40ms.

However, there is no agreement on the measurement gap for CSI-RS. We believe that gap is certainly needed in inter-frequency measurement cases, no matter for SS block or CSI-RS. Since it is desired to have a finite measurement gap length also for inter-frequency CSI-RS measurement, it is reasonable that those CSI-RSs are also confined in a burst structure like SS block. Otherwise, it will be very difficult to design a suitable measurement gap for CSI-RS. Two examples are provided in Figure 1, where all cells in frequency 1 have a burst structure for CSI-RS, while in frequency 2 the CSI-RSs come in distributed pattern in time.

Observation 4: It is reasonable that the CSI-RSs are also confined in a burst structure to accommodate the gap-assisted measurement for inter-frequency cases.

Figure 1. Examples for the impact of different CSI-RS pattern to the design of measurement gap.

From above discussion, we see the need to discuss the measurement gap for CSI-RS, which we expect to be confined within a burst in time domain. (Note that CSI-RS for beam management and CSI acquisition do not have such a limitation) Therefore, for a UE which is configured to perform both SS block based and CSI-RS based measurement, there are at least 3 options that can be considered for measurement gap design:

Option 1: Two independent measurement gap configurations: one for SS block and the other for CSI-RS. One example is provided in Figure 2. This options has the advantage of better flexibility for CSI-RS configuration and perhaps shorter gap, when there is not many CSI-RS configurations. One drawback is that UE anyway needs SS blocks for information like detected cell, slot/symbol reference, as well as course timing and frequency offset of a target cell before measuring CSI-RS. More complicated scheduling is required by UE to avoid those information become invalid as time goes by. Furthermore, more RF tuning time is required for two gaps which also leads to more interruption in UE reception.

Figure 2. Example of Two independent measurement gap configurations for SS block and CSI-RS based measurements.

Option 2: Same measurement gap configuration: Each gap is used for only SS block based measurement or only CSI-RS based measurement. One example is provided in Figure 3. The advantage of this options is no degradation of scheduling opportunity of the UE on serving cells and neither the need of additional RF tuning overhead. However, it shares not only the same drawbacks as Option 1, but also has the problem of longer measurement delay.

Figure 3. Example of single measurement gap configuration for SS block and CSI-RS based measurements: Each gap is used for only SS block based measurement or only CSI-RS based measurement

- Option 3. Same measurement gap configuration: Each gap is shared by both SS blocks and CSI-RSs. One example is provided in Figure 4. The advantages are that there is no additional overhead for RF re-tuning time and that the SS blocks can provide all information that are needed for CSI-RS and are absolutely valid within the same gap. However, one disadvantage of this option is less flexibility for CSI-RS configuration. Also, one concern on this option is the capacity for carrying the number of CSI-RS in the remaining slots in the same 5ms window.

Figure 4. Example of single measurement gap configuration for SS block and CSI-RS based measurements: Each gap is shared by both SS blocks and CSI-RSs

From a viewpoint of a UE vendor, Option 3 is preferred which allows measurements on both SS block and CSI-RS to be performed within the same gap. This will ease the effort of UE on handling the information like detected cell, slot/symbol reference, timing offset and frequency offset across different gaps, especially when UE is moving with high mobility.

R4 Proposal: Confine CSI-RS for L3 mobility also within the same gap with SS block.

4	Capacity of 5ms window of SS block in terms of CSI-RS configuration

The main concern of Option 3 is the capacity: how many CSI-RS configurations can be added in to the 5ms window which is already occupied by SS blocks. In this section, we investigate this issue.

In our opinion, the answer would depend on at least the following parameters:

- The number of remaining slots in 5ms window. RAN1 has agree the mapping of slots with SS blocks in a half radio frame, as provided in Appendix 1 [5]. Therefore the remaining slots left for CSI-RS is different under different scenarios, as summarized in Table 2, L is he max number of SS blocks in the 5ms window agreed in [4] and is also the max number of analog beams.

Table 1 Number of remaining slots in different scenarios

| SCS | L | # of remaining slots (14 OFDM symbols) |
| --- | --- | --- |
| 15 | 4 | 3 |
|  | 8 | 1 |
| 30 | 4 | 8 |
|  | 8 | 6 |
| 120 | 64 | 8 |
| 240 | 64 | 48 |

- CSI-RS density (D) [5]. With a larger number of D, there are less number of CSI-RS that can put in the 5ms slot. With D=1, 2, 3, 4, and 6, there are at most 12, 6, 4, 3 and 2 CSI-RS configurations that can be allocated in the same OFDM symbol, respectively.

Agreements: - CSI-RS resource with 1-port and 2-port for one OFDM symbol can be used for beam management - Value of D>=1 represents RE/RB/port within an OFDM symbol.

We also adopt the following assumptions in our analysis:

- CSI-RS transmitted from different cells can share the same time/frequency resource (RE). In LTE, ZP-CSI-RS is used in neighboring cell to protect CSI-RS from interference. However, in NR, this could be unnecessary because of the additional spatial dimension: beam. A network deployment can control the interference from neighboring cell CSI-RS via arranging proper beam directions and symbol time of individual CSI-RS.

- CSI-RSs that are transmitted in the same OFDM symbol should share the same analog beam. Each SS block is mapped to one specific analog beam. And we further assume that the (analog+digital) beam formed through CSI-RS is always within one of the analog beam formed by a SS block.

- We assumed TDM between SS block and CSI-RS. FDM is not considered here, because some of the CSI-RS REs will be punctured by SS block.

- For a slot that carries SS blocks, not all OFDM symbols are occupied by PSS, SSS and PBCH. There are symbols reserved for DL control, GP and UL control, and there are also some empty symbols neither occupied nor reserved [8]. For simplicity, here we assume those empty symbols are not used for CSI-RS.

- For the remaining slots without SS block, we assume 2 OFDM symbols are reserved for DL and 2 OFDM symbols are reserved for UL. Therefore, there 10 symbols available in a slot for allocating CSI-RS.

- We assume same numerology between SS block and CSI-RS. Therefore, SCS=240KHz is not considered.

Based on above assumptions, we can have Table 2, extended from Table 1, where

- Number of remaining symbols for CSI-RS (B) = A * S.

- Number of symbols per analog beam (C) = floor (B / L).

- max number of CSI-RS configurations (D) = C * K

- Number of CSI-RS configurations that can be allocated in the same OFDM symbol (K) = 12, 6, 4, 3 and 2 for D=1, 2, 3, 4, and 6, respectively.

- Number of symbols available in a slot for allocating CSI-RS (S) = 10.

Table 2 number of CSI-RS configurations available for remaining slots

| SCS | L | # of remaining slots (A) | # of remaining symbols for CSI-RS (B) | # of symbols per analog beam (C) | # of CSI-RS configurations per analog beam (D) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | D=1 | D=2 | D=3 | D=4 | D=6 |
| 15 | 4 | 3 | 30 | 7 | 84 | 42 | 28 | 21 | 14 |
|  | 8 | 1 | 10 | 1 | 12 | 6 | 4 | 3 | 2 |
| 30 | 4 | 8 | 80 | 20 | 240 | 120 | 80 | 60 | 40 |
|  | 8 | 6 | 60 | 7 | 84 | 42 | 28 | 21 | 14 |
| 120 | 64 | 8 | 80 | 1 | 12 | 6 | 4 | 3 | 2 |
| 240 | 64 | 48 | 480 | 7 | 84 | 42 | 28 | 21 | 14 |

From Table 2, we see that the cases with the most limited CSI-RS configurations are SCS=15KHz with L=8 and SCS=120KHz with L=64. In this cases, we can still have at least 12, 6, 4, 3 and 2 of CSI-RS configurations per beam for D = 1, 2, 3, 4 and 6, respectively. Considering that the number of CSI-RS configurations needed for L3 mobility is less than the number needed for beam management and CSI acquisition. We think it is still fine to confine CSI-RS in the same 5ms window as SS blocks.

Observation 5: When the maximum number of beam is adopted by a cell, there are at least 12, 6, 4, 3 and 2 CSI-RS configurations per beam for D =1, 2, 3, 4 and 6, respectively.

5	Summary

In this paper, we discuss the measurement gap for CSI-RS for L3 mobility and also provide an analysis on the capacity for CSI-RS in the 5ms window of SS burst. Based on above discussion, we have the following observations and proposals:

Observation 1: UE needs SS block to provide course synchronization of the timing and frequency in order to measure CSI-RS.

Observation 2: UE needs SS block to provide TTI reference in order to measure CSI-RS.

Observation 3: CSI-RS configuration can be provided through measurement object for the purpose of inter-frequency measurement.

Observation 4: It is reasonable that the CSI-RSs are also confined in a burst structure to accommodate the gap-assisted measurement for inter-frequency cases.

Observation 5: When the maximum number of beam is adopted by a cell, there are at least 12, 6, 4, 3 and 2 CSI-RS configurations per beam for D =1, 2, 3, 4 and 6, respectively.

Proposal 1: RAN4 to treat SS block and CSI-RS with equal priority when discussion corresponding RRM requirements.

6	Reference

[1] R4-1706608, “NR RRM way forward”, Ericsson, Nokia

[2] Chairman's Notes RAN1_88_final

[3] Draft_Minutes_report_RAN1#88b_v010

[4] Chairman's Notes RAN1_89_final

[5] Chairman's Notes RAN1_NRAH2_final

[6] R2-1704001, “Report of 3GPP TSG RAN2 meeting #97bis, Spokane, USA”, ETSI MCC

[7] RAN4-NR#2_Meeting report_v2

[8] R1-1711899, “Way forward on the mapping of SS blocks”, Nokia

Appendix 1:

Agreements:

- For 15k Hz, 30 kHz, and 120 kHz subcarrier spacing, the following mapping to slots in a half radio frame is agreed

  - Note: For NR and LTE coexistence, duplex direction alignment can be achieved by subframe offset