3GPP TSG-RAN WG4 Meeting #84	R4-1708190

# Berlin, Germany, 21 - 25 August, 2017

Source: 	ZTE

Title:	Proposal on the removal of EMC radiated emission requirement for BS in TS 38.113

Agenda Item:	9.5.1

Document for:	Discussion

Introduction

TS 36.113 [1] and TS 37.113 [2] have proposed the radiated emission measurement requirement for BS and auxiliary equipment. This test assesses the ability of BS and auxiliary equipment to limit unwanted emission from the enclosure port. The test site and limit for radiated emission for BS are in accordance with ITU-R SM.329 [3]. The test method and limit for radiated emission for auxiliary equipment are in accordance with CISPR22 [4].  EMC requirements for the NR BS are expected to be captured in the new specification TS 38.113.

In this contribution we compare the EMC radiated emission and RF radiated spurious emission and discuss the necessity of EMC radiated emission measurement for BS.

Discussion

From the perspective of physical origin and spatial interference emission, both radiation emission and radiation spurious emission are tested by considering the unwanted omni-directional disturbance from EUT’s enclosure. The difference is that the measurement results are field strength or power for different measured frequency range, the limit and the concerned  source of interference.

Firstly, the radiated emission test is derived from CISPR 32 [5] which replaced CISPR 22 [4]. It is the standard about the EMC emission requirement for MME (ITE). The purpose of this test is to measure and fix a limit of the omnidirectional disturbance from EUT’s enclosure. The measured frequency is from 30MHz to 6GHz. The measured metric is field strength in dBμV/m and required to test in the SAC or FSOATS. But the key point is that the  radiated emission requirements in CISPR 32 [5]  are not applicable to transmitter like BS.

Secondly, the radiated spurious emission test is derived from unwanted emissions in the spurious domain in ITU-R SM.329 [3]. The purpose of this test is also to measure and fix a limit of the omnidirectional disturbance from EUT’s enclosure. The measured frequency is from 30MHz to 300GHz. The measured metric is power in dBm and it requires to test in the SAC or OATS. But the radiated spurious emission requirements in ITU-R SM.329 [3]  are applicable to transmitter like BS.

ITU-R SM.329[3] indicates that radiated spurious emission includes harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products, therefore compared with radiated emission, radiated spurious emission is more focused on considering and limiting the interference products from out of operating frequency including the harmonic and intermodulation caused by operating frequency itself and frequency conversion products which also contain those from digital circuit. ITU-R SM.329 specifies also the limit for low frequency 30MHz~1GHz (-36dbm for 30MHz~1GHz) which covers the measurement frequency range of radiated emission.

Actually, in the ANNEX 1 chapter 3 of ITU-R.329[3] a simple relation can be established for perfect, ideal cases (which means free space, far field conditions) between E (V/m), D distance between the transmitting radio equipment and the point of measurement (m), e.i.r.p. (W) and pfd (W/m2).

In the chapter 4 of ITU-R SM.329[3] contains Category Z limits for Class A (industrial) and B (domestic) ITE. Category Z equipment is defined as that which combines ITE with a radio transmitting function. Conversion in terms of e.i.r.p. is provided for information by assuming that the maximum field strength according to the CISPR measurement method.

Based on the comparison about the radiated emission and radiated spurious emission, both of them are considered to the measurement of radiation interference. From the ETSI / ITU historical standard and the above principle analysis, radiated spurious emission requirement is more reasonable for BS. For auxiliary equipment radiated emission should be specified based on  the CISPR 32[5].

Proposal 1：We suggest not considering EMC RE any more in TS 38.113 and TR 37.843 and continue to study for RSE TRP methodology.

窗体底端

Proposal 2：For auxiliary equipment, the EMC radiated emission should be specified based on  the CISPR 32[5].

Conclusion

Based on the discussion above, the following proposals were made:

Proposal 1：We suggest not considering EMC RE any more in TS 38.113 and TR 37.843 and continue to make research for RSE TRP methodology.

Proposal 2：For auxiliary equipment, the EMC radiated emission should be required based on  the CISPR 32[5].

- References

- 3GPP TS 36.113 V14.2.0

- 3GPP TS 37.113 V14.1.0

- ITU-R SM.329	Unwanted emissions in the spurious domain

- CISPR 22 Information technology equipment –Radio disturbance characteristics – Limits and methods of measurement

- CISPR 32 Electromagnetic compatibility of multimedia equipment-Emission requirement