3GPP TSG-RAN WG4 Meeting #84											R4-1708479

Berlin, Germany 21 – 25 August 2017

Source: 	Neul, Huawei, HiSilicon

Title: 	NB-IoT Additional Spectrum Emission Mask

Agenda Item:	6.3.1

Document for:		     Approval

Introduction

In RAN4#83, it was agreed that additional spectrum emission masks for NB-IoT guard-band deployment are to be evaluated [1]. Further to the analysis in [2], two A-SEMs and the corresponding A-MPRs are proposed for the deployment of NB-IoT in the guard-band of a 10 MHz LTE channel. More details are discussed in Section 2.

Guard-band locations for NB-IoT

As pointed out in [3], the centre frequency of the NB-IoT DL channel shall meet the following constraints:

C1: The frequency gap between the NB-IoT RB and the LTE RB should be an integer multiple of 15 kHz in order to maintain orthogonality;

C2: The channel raster offset should be no more than 7.5 kHz for NB-IoT anchor carriers.

Taking into account C1 and C2 as well as the available guard-band, the possible DL channel locations are listed in Table 1.

Table 1: Possible NB-IoT DL channel locations in LTE guard-band

| LTE BW (MHz) | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- |
| NB-IoT channel centre to LTE channel centre (kHz) | ±2392.5 | ± 4597.5/4702.5 /4807.5/4897.5 | ± 6892.5/6997.5/7102.5/7207.5/7297.5/7402.5 | ± 9097.5/9202.5/9307.5/9397.5/9502.5/9607.5/9697.5/9802.5/9907.5 |
| Channel raster offset (kHz) | ± -7.5 | ± -2.5/2.5/7.5/-2.5 | ± -7.5/-2.5/2.5/7.5/-2.5/2.5 | ± -2.5/2.5/7.5/-2.5/2.5/7.5/-2.5/2.5/7.55 |
| Frequency gap between LTE and NB-IoT (kHz) | 45 | 0/105/210/300 | 45/150/255/360/450/555 | 0/105/210/300/405/510/600/705/810 |

On the other hand, the NB-IoT UL channel need not satisfy the constraint C2, but only C1. Hence there’re more choices for the UL channel locations. Assuming 15 kHz sub-carrier spacing, let N1 be the total number of LTE sub-carriers (e.g. N1=600 for 10 MHz BW), N2be the total number of NB-IoT sub-carriers (i.e. N2=12), and ∆f be the frequency gap between the NB-IoT RB and the first or last configured LTE RB. The centre-to-centre distance between the NB-IoT UL channel and the LTE UL channel may be expressed as:

fcc=N1+N22×15+∆f (kHz),

∆f=n×15 (kHz), n=0,1,2,….

Guard-band deployment and A-MPR

Guard-band deployment in a 10 MHz LTE channel

The NB-IoT channel may be deployed in the LHS or RHS guard-band of a LTE channel. For 10 MHz LTE BW, all possible locations can be determined by the frequency gap |∆f| = 0, 15, 30, …, 300 kHz, where ∆f>0 if deployed in the RHS and ∆f<0 if deployed in the LHS. Figure 1 illustrates the case of an NB-IoT carrier deployed in the RHS guard-band of a 10MHz LTE channel.

Figure 1: SEMs for NB-IoT carriers deployed in the RHS guard-band of a 10MHz LTE channel.

It can be seen that for ∆f≥90kHz, the general NB-IoT SEM needs to be tightened so that the UE emissions can also meet the general EUTRA SEM. The amount of SEM to be tightened and the starting frequency offset from the NB-IoT channel edge are listed in Table 2.

Table 2: SEM tightening for different NB-IoT channel locations

| Frequency gap ∆f (kHz) | 90 | 105 | 120 | 135 | 150 | 165 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SEM tightening (dB) | 0.2 | 2.3 | 4.4 | 6.5 | 8.6 | 10.3 | 11.2 |
| Starting offset from the channel edge (kHz) | 220 | 205 | 190 | 175 | 160 | 145 | 130 |

In order to alleviate the burden due to SEM tightening, the same amount of A-MPR may be allowed for the UE. The required A-MPR and the modified SEM are depicted in Figure 2.

Figure 2: Tightened NB-IoT SEM

It can be seen that the bigger the frequency gap ∆f is, the more the SEM should be tightened and hence the larger A-MPR is needed. As a trade-off, it is proposed to adopt the frequency gap ∆f = 105 kHz. More explicitly, when a NB-IoT channel is deployed in the RHS guard-band of a 10 MHz LTE channel with the frequency gap ∆f = 105 kHz, it should meet the additional SEM defined in Table 3.

Table 3: Additional NB-IoT spectrum emission mask when deployed in the RHS guard-band of a 10 MHz LTE channel with the frequency gap ∆f = 105 kHz

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 205 | -15.7 | 30 kHz |
| 205-225 | -18 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Furthermore, the sub-carriers that are sufficiently far away from the right edge of the channel are not affected by the SEM tightening. Hence the A-MPR may be applied depending on the sub-carrier allocation in order to minimize the impact to the system performance. As a result, Table 4 shows the proposed A-MPR when deployed in the RHS guard-band of a 10 MHz LTE channel.

Table 4: NB-IoT A-MPR when deployed in the RHS guard-band of a 10 MHz LTE channel with the frequency gap ∆f = 105 kHz

| UL Transmission |  | Single-tone |  | 3-tone |  | 6-tone |  | 12-tone |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-carrier allocation | 15 kHz spacing | 0~9 | 10~11 | 0-2, 3-5, 6-8 | 9-11 | 0-5 | 6-11 | 0-11 |
|  | 3.75 kHz spacing | 0~39 | 40~47 | N/A | N/A | N/A | N/A | N/A |
| A-MPR (dB) |  | 0 | ≤ 3 | 0 | ≤ 3 | 0 | ≤ 3 | ≤ 3 |

Correspondingly, if a NB-IoT channel is deployed in the LHS guard-band of a 10 MHz LTE channel with the frequency gap ∆f = -105 kHz, it should meet the additional SEM defined in Table 5.

Table 5: Additional NB-IoT spectrum emission mask when deployed in the LHS guard-band of a 10 MHz LTE channel with the frequency gap ∆f = -105 kHz

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 205 | -15.7 | 30 kHz |
| -(205-225) | -18 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

And the proposed A-MPR is shown in Table 6.

Table 6: NB-IoT A-MPR when deployed in the LHS guard-band of a 10 MHz LTE channel with the frequency gap ∆f = -105 kHz

| UL Transmission |  | Single-tone |  | 3-tone |  | 6-tone |  | 12-tone |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-carrier allocation | 15 kHz spacing | 0~1 | 2~11 | 0-2, | 3-5, 6-8, 9-11 | 0-5 | 6-11 | 0-11 |
|  | 3.75 kHz spacing | 0~7 | 8~47 | N/A | N/A | N/A | N/A | N/A |
| A-MPR (dB) |  | ≤ 3 | 0 | ≤ 3 | 0 | ≤ 3 | 0 | ≤ 3 |

Guard-band deployment in a 15/20 MHz LTE channel

The guard-band size of a 15/20 MHz LTE channel is much larger than that of a 10 MHz LTE channel. For example, in the guard-band of a 15 MHz LTE channel, the possible frequency gaps are |∆f| = 0, 15, 30, …, 540 kHz (i.e. n=0,1,…,36). Among them, no SEM tightening is needed for |∆f| = 0, 15, 30, …, 315 kHz (i.e. n=0,1,…,21). There are plenty of choices of locations for the NB-IoT channel, hence no A-SEM/A-MPR is needed for guard-band deployment in a 15/20 MHz LTE channel.

Conclusion

Based on the analysis in the previous section, we have the following proposals:

Proposal #1: Adopt the A-SEM in Table 3 and the corresponding A-MPR in Table 4 when deploying NB-IoT in the RHS guard-band of a 10 MHz LTE channel;

Proposal #2: Adopt the A-SEM in Table 5 and the corresponding A-MPR in Table 6 when deploying NB-IoT in the LHS guard-band of a 10 MHz LTE channel;

Proposal #3: Send a LS to RAN2 requesting the introduction of new NS messages for the two NB-IoT A-SEMs and the corresponding A-MPRs.

- References

R4-1705868, “Way forward on NB-IoT additional spectrum emission mask”, Neul, Huawei, HiSilicon, RAN4#83, Hanzhou, China, 15-19 May 2017

R4-1705579, “NB-IoT additional spectrum emission mask”, Neul, Huawei, HiSilicon, RAN4#83, Hanzhou, China, 15-19 May 2017

R1-156924, “Analysis of channel raster impact on NB-IoT”, Huawei, HiSilicon, RAN1#83, Anaheim, USA, 16-20 Nov 2015