3GPP TSG-RAN WG4 Meeting #84	R4-1708493

Berlin, Germany, 21st – 25th August 2017

Agenda item:	9.5.3.4

Source:	Nokia, Nokia Shanghai Bell

Title:	NR base station output power limit requirements for Range 2 (> 24 GHz)

Document for:	Approval

# 1	Introduction

During the last 3GPP RAN4 NR#2 ad hoc meeting, a way-forward on BS output power limit [1] was approved. It was captured in [1] that the upper output power limit for range 2 NR base stations is an open issue.

In this document, we address the open issue and conclude with our proposals.

# Discussion

Referring to TR 38.803, range 2 NR base stations are specified without antenna connectors. Therefore, the NR BS output power limit for range 2 is defined as an over the air requirement.

In TR 38.803, 3 classes are defined for NR base stations without antenna connectors, namely wide area, medium range and local area base stations.

The agreement captured in [1] concerning the output power limit for range 2 NR base stations is as follows:

- For Range 2 BS

  - TRP will be used as a metric for BS output power limit requirement

  - Upper limit value is FFS

    - Whether RAN4 introduce scaling method or not like previous page

Two options for specifying the upper output power limit requirement can be identified:

- Option 1: the upper power limit is scaled by NTXU

- Option 2: a fixed upper power limit

These two options will be discussed in detail.

Option 1: Upper power limit is scaled by NTXU

For this option, the upper limit is scaled by NTXU as shown in Table 1. NTXU is the total number of active transmitters.

| NR BS class | PRated,c,TRP |
| --- | --- |
| Wide Area BS | no limit |
| Medium Range BS | ≤ X dBm +10log(NTXU) |
| Local Area BS | ≤ Y dBm +10log(NTXU) |

Table 1: Output power limits with scaling for different range 2 NR BS classes

PRated,c,TRP is the rated total radiated power when all the active transmitters are operating at their rated output power for a single carrier.

For the TS 37.105 Rel-13 specification, NTXU is capped at 8 and 4 for E-UTRA and UTRA, respectively. These limits are based on the maximum number of layers specified in the 3GPP Rel-12 specifications. A similar approach can be adopted for specifying NTXU for range 2 NR base stations; however, NTXU should not cap at 8 as the total number of active transmitters for range 2 NR base stations is typically larger than range 1 NR base stations.

[NTXU] is for further studies.

[X] dBm and [Y] dBm are for further studies, which are the maximum radiated power per carrier per active transmitter for medium range and local area base stations, respectively.

Option 2: Fixed upper power limit

For this option, the output power limit requirement for range 2 NR base stations is fixed irrespective of NTXU as shown in Table 2.

| NR BS class | PRated,c,TRP |
| --- | --- |
| Wide Area BS | [no limit] |
| Medium Range BS | ≤ [A] dBm |
| Local Area BS | ≤ [B] dBm |

Table 2: Fixed output power limits for different range 2 NR BS classes

[A] dBm and [B] dBm are for further studies.

The pros and cons associated with each option are listed in Table 3.

|  | Pro | Con |
| --- | --- | --- |
| Option 1 | 1. Scalable upper power limit based on the total number of active transmitters | 1. Require OTA definition of NTXU |
| Option 2 | 1. Independent of the total number of active transmitters 2. No OTA definitions are required such as NTXU | 1. Not scalable upper power limit |

Table 3: Pros and cons of the considered options

From Table 3, we choose Option 2.

Proposal: Option 2: a fixed upper power limit for range 2 NR BS. The absolute limits are FFS.

# 3	Conclusions

In this document, we have discussed and compared 2 different options for specifying the upper output power limit for range 2 NR base stations. As a result, we propose the following:

Proposal: Option 2: a fixed upper power limit for range 2 NR BS. The absolute limits are FFS.

# References

- R4-1706960, “WF on BS output power limit”, NTT DOCOMO

- R4-1701761, Discussion on output power accuracy value for NR BS operating above 6 GHz, Nokia, Alcatel-Lucent Shanghai Bell