3GPP TSG-RAN WG4 Meeting #84	R4-1708500

Berlin, Germany, 21st – 25th August, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	eAAS BS spectrum emission mask requirements for below 6 GHz

Agenda item:	8.28.3.3

Document for:	Approval

# 1	Introduction

During the last 3GPP RAN4 NR#2 ad hoc meeting, the proposal to use the ΔfUEM = 40 MHz requirement for NR bands, which are equal to or wider than 100 MHz, was approved. From [1], the requirement is

Proposal: ΔfUEM = 40 MHz for NR bands with bandwidth equal to or wider than 100 MHz as a baseline for the boundary between UEM and spurious (for both Cat A and Cat B)

In this document, we propose to revisit the spectrum emission mask and adopt the above requirement for eAAS base stations that have the same operating bandwidth as the NR bands.

# 2	Discussion

Referring to TS 37.105, eAAS base stations use the same operating bands as for non-AAS BS as described in TS 37.104.

Table 1 lists those eAAS operating bands that are equal to or wider than 100 MHz bandwidth. These bands are a subset of those NR bands listed in [1] (note, band number 46 is not included in the eAAS operating bands according to TS 37.105). Thus, we propose to adopt the agreed proposal [1] for eAAS base stations; this is

Proposal: ΔfUEM = 40 MHz for eAAS bands with bandwidth equal to or wider than 100 MHz as a baseline for the boundary between UEM and spurious (for both Cat A and Cat B).

Table 1: eAAS operating bands ≥ 100 MHz

| Band number | Bandwidth [MHz] | Duplex Mode |
| --- | --- | --- |
| 40 | 100 | TDD |
| 41 | 194 | TDD |
| 42 | 200 | TDD |
| 43 | 200 | TDD |
| 44 | 100 | TDD |
| 48 | 150 | TDD |

# 3	Conclusions

In this document, we have discussed the spectrum emission mask requirement for eAAS operating bands equal to or greater than 100 MHz bandwidth. We propose the following

Proposal: ΔfUEM = 40 MHz for eAAS bands with bandwidth equal to or wider than 100 MHz as a baseline for the boundary between UEM and spurious (for both Cat A and Cat B).

# References

- R4-1706843, On NR BS boundary between UEM and spurious emission below 6 GHz, Nokia, Alcatel-Lucent Shanghai Bell