3GPP TSG-RAN WG4 Meeting #84	R4-1708504

# Berlin, Germany, 21 – 25, August, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.133 v0.0.1: Cell phase synchronization accuracy requirements

Agenda Item:	9.6.5

Document for:	Approval

# Introduction

A text proposal to specify cell phase synchronization accuracy requirements for TS 38.133 version 0.0.1.

# TP to TS 38.133 v0.0.1

Section 7.4 in TS 38.133 is updated as follows:

<<<Start of Changes>>>

## 7.4	Cell phase synchronization accuracy (TDD)

### 7.4.1	Definition

Cell phase synchronization accuracy is defined as the maximum absolute deviation in frame start timing between any pair of cells on the same frequency that have overlapping coverage areas.

### 7.4.2	Minimum requirements

For Wide Area BS, the cell phase synchronization accuracy measured at BS antenna connectors shall be better than the requirement specified in table 7.4.2-1. If a cell’s coverage area overlaps with another cell with different cell radius then the cell phase synchronization accuracy corresponding to the larger of the two cell sizes applies to the overlapping cells with different radii.

Table 7.4.2-1  Cell phase synchronization requirement for wide area BS (TDD)

| Cell Type | Cell Radius | Requirement |
| --- | --- | --- |
| Small cellNOTE 1 | 3 km | 3 s |
| NOTE 1: The requirement applies for NR cells of all sizes. |  |  |

For Home BS, the cell phase synchronization accuracy measured at BS antenna connectors shall be better than the requirement specified in table 7.4.2-2.

Table 7.4.2-2  Cell phase synchronization requirement for Home BS (TDD)

| Source Cell Type | Propagation Distance | Requirement |
| --- | --- | --- |
| Small cellNOTE 1 | 500 m | 3 s |
| NOTE 1: The requirement applies for NR cells of all sizes. |  |  |

Note 1: 	Tpropagation is the propagation delay between the Home BS and the cell selected as the network listening synchronization source. In terms of the network listening synchronization source selection, the best accurate synchronization source to GNSS should be selected.

Note 2: 	If the Home BS obtains synchronization without using network listening, the small cell requirement applies.

<<<End of Changes>>>