3GPP TSG-RAN WG4 #84	R4-1708612

Berlin, Germany, 21st-25th August 2017

Source: 	Ericsson

Title:  	TP on band arrangements for Rel-15 TR 36.790: LAA/eLAA for the “CBRS” 3.5GHz band in the United States

Agenda Item:	8.18.2

Document for:	Approval

# Introduction

In RAN#75, a new work item was agreed for licence assisted access (LAA) and enhanced LAA (eLAA) operation for the CBRS 3.5GHz band in US [1]. A new TR is agreed as TR 36.790 and a skeleton in proposed in [2].

In this contribution, we propose a text for this TR which includes texts related to band information of the report.

# Text proposal

The following text proposal is intended for TR 36.790.

<<<<< start of TP <<<<<<

# 5	Band and Channel Arrangements

<Editor’s note: Briefly describe the scope, approach and methodologies for multi-node tests design, including basic test design principles >

## 5.1	Operating band definition

E-UTRA is designed to operate in the operating bands defined in Table 5.1-1.

Table 5.1-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 49 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD1,2 |
| NOTE 1: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 2: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. |  |  |  |  |  |  |  |

## 5.2	CA operating band definition

CA_2A-49A is defined to operate in the entire frequency range of Band 2 and band 49, as defined in Table 5.2-1.

Table 5.2-1: Inter-band CA operating band definitions in Band 2 and band 49

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-49 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 49 | 3550 MHz | -- | 3700 MHz | 3550 MHz | -- | 3700 MHz | TDD |

## 5.3	Channel bandwidth per operating band

a)	The requirements in this specification apply to the combination of channel bandwidths and operating bands shown in Table 5.3-1. The transmission bandwidth configuration in Table 5.3-1 shall be supported for each of the specified channel bandwidths. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 5.3-1: E-UTRA channel bandwidth

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 49 |  |  |  | Yes |  | Yes |

## 5.4	CA channel bandwidth

Table 5.4-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (two bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-49A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 49 |  |  |  | Yes |  | Yes |  |  |

## 5.5	Channel raster

<<<<< end of TP <<<<<<

# Conclusion

We propose to adopt the above mentioned text proposal for TR 36.790.

# References

- RP- 170804, “New WI: LAA/eLAA for the CBRS 3.5GHz band in the United States”, AT&T, RAN#75, March 2017.

- R4-1708155, “Template skeleton for TR 36.790: LAA/eLAA for the “CBRS” 3.5GHz band in the United States”, Ericsson.