3GPP TSG-RAN WG4 Meeting #84	R4-1708697

# Berlin, Germany, 21 – 25, August, 2017

Source:	Nokia, Nokia Shanghai Bell, Ericsson, VODAFONE Group Plc

Title:	TP to TS 38.133 v0.0.1: Cell phase synchronization accuracy requirements

Agenda Item:	9.6.5

Document for:	Approval

# Introduction

A text proposal to specify cell phase synchronization accuracy requirements for TS 38.133 version 0.0.1 [1]. Text proposal is based on agreements made in [2] and as repeated here:

## Cell phase synchronisation

- Definition

- Cell phase synchronization accuracy is defined as the maximum absolute deviation in frame start timing between any pair of cells on the same frequency that have overlapping coverage areas.

- Minimum requirement

- The cell phase synchronization accuracy measured at BS antenna connectors shall be better than [3µs].

# TP to TS 38.133 v0.0.1

Section 7.4 in TS 38.133 is updated as follows:

<<<Start of Changes>>>

## 7.4	Cell phase synchronization accuracy

### 7.4.1	Definition

Cell phase synchronization accuracy for TDD is defined as the maximum absolute deviation in frame start timing between any pair of cells on the same frequency that have overlapping coverage areas.

### 7.4.2	Minimum requirements

The cell phase synchronization accuracy measured at BS antenna connectors shall be better than [3] µs.

<<<End of Changes>>>

# References

- 3GPP TS 38.133 V0.0.1 (2017-05)

- R4-1706608, “NR RRM way forward”, Ericsson, Nokia, 3GPP TSG-RAN WG4 Meeting #NR AH 2, Qingdao, China, May 2017.