3GPP TSG-RAN WG4 Meeting #84	    R4-1708700

Berlin, Germany, 21 – 25 Aug, 2017

Source: 	Samsung

Title: 	eFD-MIMO ad hoc Minutes

Agenda Item:	7.15.1

Document for:	Approval

Semi-open-loop transmission (10 minutes)

Related contribution list:

| R4-1707076 | Simulation results for semi-open loop transmission | Samsung |
| --- | --- | --- |
| R4-1707084 | Draft CR for introducing eFD-MIMO dedmodulation test cases | Samsung |
| R4-1707810 | Simulation result for eFD-MIMO semi-open loop | Qualcomm Incorporated |
| R4-1708652 | Demodulation performance for eFD-MIMO semi-Open-loop transimission | CATT |

Open issues:

- Issue 1: Number of Tx for rank2 test

  - Option1: 4Tx (Samsung, CATT)

  - Option2: 2Tx (QC)

- Issue 2: Doppler shift for rank2 test

  - Option 1: Previous meeting tentative agreements: 70Hz

  - Option 2: 200Hz (CATT)

Agreements:

- Issue 1: Number of Tx for rank2 test

  - 4Tx, per PRG updated  random precoding matrix according to Rel-10 4Tx codebook

- Issue 2: Doppler shift for rank2 test

  - 70Hz

CSI test cases

CSI test case for class A (10 minutes)

Related contribution list:

| R4-1707074 | Simulation results for Class A PMI test cases | Samsung |
| --- | --- | --- |
| R4-1707082 | Draft CR for introducing eFD-MIMO Class A PMI test cases | Samsung |
| R4-1707379 | E-FD-MIMO Class A PMI reporting performance requirements | Intel Corporation |
| R4-1707681 | Simulation result for eFD-MIMO class A PMI reporting test | Ericsson |
| R4-1707806 | Simulation result for eFD-MIMO Class A | Qualcomm Incorporated |
| R4-1707814 | Discussion on requirements for eFD-MIMO Class A PMI | CATT |

Open issues:

- Issue 1: MCS and rank:

  - Single PMI test case

    - Option 1: 16QAM rank-2 (Samsung, Ericsson, CATT)

    - Option 2: 64QAM rank-1 (Intel, Qualcomm, Huawei)

  - Multiple PMI test case

    - Option 1: 64QAM rank-2 (Samsung, Ericsson, CATT)

    - Option 2: 16QAM rank-2 (Qualcomm, Intel, Huawei)

- Issue 2: Reference point for Test metric:

  - Option 1: 90% (Samsung, E///?)

  - Option 2: 95% (CATT)

Agreements:

- Issue 2: Reference point for Test metric:

- 90%

Advanced codebook (10 minutes)

Related contribution list:

| R4-1707081 | Test case design for advacned codebook | Samsung |
| --- | --- | --- |
| R4-1707682 | Discussion on PMI test for advanced CSI codebook | Ericsson |
| R4-1707805 | Simulation result for eFD-MIMO Advanced Codebook Test | Qualcomm Incorporated |
| R4-1708361 | Discussion on PMI/RPI reporting test for advanced CSI | Huawei, Hisilicon |

Open issues:

- Issue 1: Beam steering model

  - Finalize the beam steering method for the advanced codebook PMI test as the dual cluster beam steering model R4-1703255:

Selection of p:

    - Option 1: 1

QC: Better discriminate performance

- Issue 2: Test metric:

  - Option 1: Throughput ratio between the followed Advanced PMI/RPI and followed Advanced PMI with fixed RPI=0.

E///: our preferred option 1.

Samsung: Test feasibility

E///: It can be implemented.

E///: UE may pass test case with class A PMI reporting.

QC: How to decide? What’s guideline to justify the gap is enough or not to discriminate UE behavior.

E///: Companies can bring analysis and results, check the performance.

  - Option 2: Throughput ratio between the followed Advanced PMI/RPI and randomized {i1,3, i2, RPI} with the Advanced CSI.  (Previous meeting tentative agreements)

  - Option 3: Throughput ratio between the followed Advanced PMI/RPI and the followed Class A PMI.

- Issue 3: MIMO correlation matrix:

  - Option 1: High correlation as existing in specs

  - Option 2:  Medium spatial correlation

|  | α1 | α2 | β | γ |
| --- | --- | --- | --- | --- |
| High spatial correlation | 0.9 | 0.9 | 0.9 | 0.3 |
| Medium spatial correlation | 0.3 | 0.3 | 0.6 | 0.2 |

Agreements:

- Issue 1: Beam steering model

  - Finalize the beam steering method for the advanced codebook PMI test as the dual cluster beam steering model R4-1703255:

Selection of p:

    - Option 1: 1

- Issue 2: Test metric:

  - Option 1: Throughput ratio between the followed Advanced PMI/RPI and followed Advanced PMI with fixed RPI=0.

  - Option 2: Throughput ratio between the followed Advanced PMI/RPI and randomized {i1,3, i2, RPI} with the Advanced CSI.  (Baseline)

- MIMO correlation matrix:

  - Option 1: High correlation as existing in specs

  - Option 2:  Medium spatial correlation

|  | α1 | α2 | β | γ |
| --- | --- | --- | --- | --- |
| High spatial correlation | 0.9 | 0.9 | 0.9 | 0.3 |
| Medium spatial correlation | 0.3 | 0.3 | 0.6 | 0.2 |

- MCS and Rank:

  - Option 1: (Baseline)-16QAM 1/2 Rank2

  - Option 2: 64QAM 1/2 Rank1

Hybrid CSI mechanism (10 minutes)

Related contribution list:

| R4-1707077 | Test case design for Hybird mechanism 1 | Samsung |
| --- | --- | --- |
| R4-1707078 | Test case design for Hybird mechanism 2 | Samsung |
| R4-1707085 | Draft CR for introducing eFD-MIMO Hybrid CSI test cases | Samsung |

Open issues:

- Issue 1: Necessity of Hybrid CSI test mechanism 1

- Issue 2: Detailed test case design for  Hybrid CSI test mechanism 2

  - Test metric : Introducing test metric as relative throughput ratio under FRC between following CRI (1st CSI reporting), PMI reporting (2nd CSI reporting) and fixed CRI, random PMI:

- is [TBD%] of the maximum throughput obtained at  using following precoder according to UE reported  PMI(2) for eMIMO-Type2 and power scaling factor according to UE reported CRI (1)  for eMIMO-Type

-	is throughput obtained at  using random precoder and fixed power scaling factor according to the one configured CSI-RS resource

  - Test parameter :

    - MIMO correlation in test: 4*2 EPA5Hz ULA Low Correlation

    - BF CSI resources configurations: Two CSI-RS resources {0,1} with Nk =4

      - Power scaling for  CSI-RS resources

, k = 0,1

    - CSI configuration:

      - For 1st eMIMO-Type , 2 (K=2) BF CSI-RS resources configured as Nk =4 ports; CSI reporting periodicity is multiple times of 2nd eMIMO-Type with additional offset

      - For 2nd eMIMO-Type, one BF CSI-RS resources configured as Nk =4 ports

    - CSI feedback mode configuration

      - PUSCH 3-1 mode

      - eMIMO-Type: Reporting interval fixed as 40ms, with aperiodic CSI request in SF#1

      - eMIMO-Type2: Reporting interval fixed as 5ms, with aperiodic CSI request in SF#1,SF#6

- Issue 3: Test case design for  Hybrid CSI test mechanism 1 (For simulation purpose)

  - Introduce hybrid CSI mechanism 1 test case based on existing Rel-13 Class A PMI reporting and Class B K=1 PMI reporting test cases with beam steering channel.

  - Test metric: Introducing test metric as relative throughput ratio under FRC between following i1 (1) (1st CSI reporting), PMI (2) reporting (2nd CSI reporting) and random i1 (1), PMI (2):

- is [TBD%] of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured atwith random precoding.

Agreements:

Issue 1: Necessity of Hybrid CSI test mechanism 1 (FFS)

- Issue 2: Detailed test case design for  Hybrid CSI test mechanism 2

  - Test metric : Introducing test metric as relative throughput ratio under FRC between following CRI (1st CSI reporting), PMI reporting (2nd CSI reporting) and fixed CRI, random PMI:

- is [TBD%] of the maximum throughput obtained at  using following precoder according to UE reported  PMI(2) for eMIMO-Type2 and power scaling factor according to UE reported CRI (1)  for eMIMO-Type

-	is throughput obtained at  using random precoder and fixed power scaling factor according to the one configured CSI-RS resource

  - Test parameter :  (baseline)

    - MIMO correlation in test: 4*2 EPA5Hz ULA Low Correlation

    - BF CSI resources configurations: Two CSI-RS resources {0,1} with Nk =4

      - Power scaling for  CSI-RS resources

, k = 0,1

    - CSI configuration:

      - For 1st eMIMO-Type , 2 (K=2) BF CSI-RS resources configured as Nk =4 ports; CSI reporting periodicity is multiple times of 2nd eMIMO-Type with additional offset

      - For 2nd eMIMO-Type, one BF CSI-RS resources configured as Nk =4 ports

    - CSI feedback mode configuration

      - PUSCH 3-1 mode

      - eMIMO-Type: Reporting interval fixed as 40ms, with aperiodic CSI request in SF#1

      - eMIMO-Type2: Reporting interval fixed as 5ms, with aperiodic CSI request in SF#1,SF#6

BF CSI-RS enhancements (10 minutes)

Related contribution list:

| R4-1707075 | Simulation results for Class B PMI test cases | Samsung |
| --- | --- | --- |
| R4-1707083 | Draft CR for introducing eFD-MIMO Class B PMI test cases | Samsung |
| R4-1707683 | Simulation result for eFD-MIMO class B K=1 PMI reporting test with CSI-RS density reduction | Ericsson |
| R4-1707808 | Simulation result for eFD-MIMO Class B CSIRS density reduction | Qualcomm Incorporated |

Open issues:

Issue 1: BF CSI-RS resources configuration for aperiodic CSI-RS test case

- For aperiodic CSI-RS test case, through RRC configured 2 CSI-RS resources, and during test through MAC CE, random selected one of CSI-RS resources activated (fixed during test)

Issue 2: BF CSI-RS resources configuration for “must-shot” CSI-RS test case

- For multi shot test case, through RRC configured 2 CSI-RS resources, and during test through MAC CE, random selected one of CSI-RS resources activated/released in an alternative way

  - The activation and deactivation command will be transmitted in sub-frame #0 per 1000 radio frames.

  - Per 1000 radio frames, the latest reported PMI in previous sub-frame is applied for the subsequent downlink sub-frames after the deactivation/activation command until a new PMI is available.

Issue 3:  Codebook configuration for aperiodic CSI-RS test case and multi-shot CSI-RS test cases

- Option 1: Class B alternative codebook (Same Rel-13 Class B K=1 PMI test case)

Note: No simulation needed, test only applicable for UE which supporting Class B alternative codebook

- Option 2: Rel-10 codebook (Same as Rel-10 CSI-RS based on PMI test case)

Note: Re-simulation may needed

Agreements:

Issue 1: BF CSI-RS resources configuration for aperiodic CSI-RS test case

- For aperiodic CSI-RS test case, through RRC configured 2 CSI-RS resources, and during test through MAC CE, random selected one of CSI-RS resources activated (fixed during test)

Issue 2: BF CSI-RS resources configuration for “must-shot” CSI-RS test case

- For multi shot test case, through RRC configured 2 CSI-RS resources, and during test through MAC CE, random selected one of CSI-RS resources activated/released in an alternative way

  - The activation and deactivation command will be transmitted in sub-frame #0 with fixed periodicity [X] ms

    - Note: X FFS

  - The corresponding Y downlink SFs after deactivation/activation commands will be skipped for PDSCH transmission

Issue 3:  Codebook configuration for aperiodic CSI-RS test case and multi-shot CSI-RS test cases

- Rel-10 codebook (Same as Rel-10 CSI-RS based on PMI test case)  based on existing Rel-10 CSI-RS single PMI test cases

  - 4*2 ULA Low for both TDD and FDD

  - Companies are encourage to bring simulation results at least for TDD case in future meetings