3GPP TSG-RAN4 Meeting #84 	R4-1708793

Berlin, Germany; 21 – 25 August 2017

Source: 	Huawei, Hisilicon

Title: 	TP for TR 36.753 Band plan of TDD operation in L-band (B50 & B51)

Agenda Item:	8.14.4

Document for:	Approval

# Introduction

A work item [1] was agreed to define a new TDD band covering 1427 – 1518 MHz.

This contribution is to provide the TP about A-MPR studies to protect EESS in the 1400-1427MHZ band, based on the proposals [10] approved during the last 3GPP RAN4 meeting#83 in Hangzhou, China, 15 - 19 May, 2017.

# Text proposal for TR

<Unchanged sections omitted>

## 4 Background

At the ITU World Radiocommunication Conference in 2015 (WRC-15), the frequency band 1 427-1 518 MHz (hereafter, L-band) was identified for International Mobile Telecommunications (IMT). In accordance with this outcome at WRC-15, ITU-R Working Party 5D (WP 5D) initiated to revise Recommendation ITU-R M.1036 “Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations”. For this revision, it has been proposed to include a new TDD arrangements.

In 3GPP, the Study Item required transmission restrictions of UE and BS to meet the unwanted emission limit (– 62dBW/27MHz with UE transmit power of 15dBm; -72dBw/27MHz at the BS antenna connector) for protection of the Earth Exploration-Satellite Service (EESS) was investigated. Also, MSS protection has been taking into account, for the BS case and EU case.

According to results of the studies, the implementation of TDD arrangement in the L-band is a realistic option while achieving the protection of the EESS and MSS.

<Unchanged sections omitted>

## 5.2		TDD band plan

Based on the work item objective, the possible band plans is:

TDD Arrangement (a)

“Up to 18 Blocks of 5 MHz + 1 MHz Block” the final number of blocks will be based on the results of compatibility studies

TDD Arrangement (b)

“1 Block of 3 MHz + and up to 17 Blocks of 5 MHz + 1 Block of 3 MHz” the final number of blocks will be based on the results of compatibility studies

Table 5-1: Possible frequency band channelization in the 1427-1518MHz band

<Unchanged sections omitted>

## 6	General issues

6.1 Frequency band arrangement

To optimize the spectrum use and to align the TDD band on the SDL band plan, the final band plan adopted by 3GP RAN4 is:

- B50

- B51 (only

UE that complies with the E-UTRA Band [50] minimum requirements in this specification shall also comply with the E-UTRA Band [51] minimum requirements

<Unchanged sections omitted>

# Conclusion

Proposal 1: Add this text above to TDD LBand TR 36.753.

.

# References

[1] RP-162557 “New E-UTRAN L-band plan 1427-1518MHz”, Etisalat, Huawei, HiSilicon

[2] R4-1704099_WF on UE and BS requirements in FDD L-band

[3] ECC Report 263, “Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1525 MHz”;

[4] ITU-R Report M.2292 “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”

[5] R4-1704098- Band plan discussion 1 5 GHz TDD

[6] TR 36.892 - 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Introduction of 1447-1467MHz Band for TD-LTE in China

[7] R4-1703068 - UE reference sensitivity

[8] R4-1703518, “Discussion on L Band FDD EU emission to protect EESS” Huawei, Hisilicon

[9] R4-1703465 - TP for TR 36.751: A-MPR summary for EESS protection for FDD operation in L-band - NTT DOCOMO, INC., Nokia, Huawei

[10] R4-1706104 “Band plan + EESS/MSS protection levels for TDD L-band” ,  Huawei, Nokia and Etisalat