3GPP TSG-RAN4 Meeting #84	R4-1709209

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.977 | CR | 0062 | rev | 2 | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MIMO OTA Measurement Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MIMO_OTA |  |  |  |  |  | Date: |  |  | 2017-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To add previously agreed measurement procedures |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define P_MODE measurement procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR requirements won’t be fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged content omitted >>

#### 12.1.6.2	Test procedure

The following steps shall be followed in order to evaluate MIMO OTA performance of the DUT:

1. Measure MIMO OTA throughput from one measurement point. MIMO OTA throughput is the minimum downlink power (or SIR) resulting in a throughput value of 70% and 95% of the maximum theoretical throughput.  The downlink power (or SIR) step size shall be no more than 0.5 dB when RF power level is near the LTE MIMO sensitivity level. Measurement duration shall be sufficient to achieve statistical significance that is TBD.

NOTE 1: 	The initial RS EPRE can be set to the user's freely selectable level. 
Recommended initial RS EPRE is found in Tables 7.1-1 and 7.1-2. For the SIR control tests, ensure that the default signal level is set such that the target SIR values can be achieved when utilizing the SIR validation procedure defined in clause 5.1.2.1.

NOTE 2: 	The throughput value target DL power level (or SIR) can be changed using user's freely selectable algorithm.

NOTE 3:	 Assuming that the default signal level meets the criteria in NOTE 1, the interference level will be adjusted to achieve the target SIR.

NOTE 4: For devices that do not reach target TP, the highest RS EPRE at which TP is measured shall be -80 dBm/15 kHz. The starting RS EPRE level of the LTE MIMO sensititivity level search does not need to be specified (NOTE 1)

NOTE 5: For devices that exhibit non-linear TP behaviour and that achieve target throughput at multiple RF power levels, LTE MIMO sensititivity level is chosen to be lowest measured DL power level which crosses the target throughput level

2. Rotate the UE around vertical axis of the test system by 30 degrees and repeat from step 1 until one complete rotation has been measured i.e. 12 different UE azimuth rotations.

3. Repeat the test from step 1 for each specified device orientation. A list of orientations is given in Annex E.

4. The postprocessing method to calculate the average MIMO Throughput is defined in 5.2.1.

## << End of changes >>