3GPP TSG-RAN WG4 #84bis	R4-1710179

Dubrovnik, Croatia, 9th – 13th Oct 2017

Agenda Item:	8.28.2

Source:	Ericsson

Title:	Further discussion on network-based CRS mitigation impact on legacy advanced UE receiver

Document for:	Discussion

# Introduction

In RAN4#84 Meeting, some analysis on the network-based CRS mitigation impact on UE performance when UE receiver performs CRS based interference mitigation were discussed. However, there is no any conclusion for this issue. In this contribution, we provide our further analysis. It should be noted that some parts have been presented in the last meeting.

# Discussion

Up to now, there are several advanced receivers which are defined in RAN4. Network-based CRS mitigation may impact the following receiver performance:

- CRS-IM receiver

- Type A and type B DL control channel IM receivers

- NAICs receiver

In the following sections, we provide our analysis on the potential impact.

## CRS-IM receiver

In one companion paper [2], extensive simulation results are provided to check the CRS-IM receiver. Based on the results, we mainly have the following observations:

The performance of legacy CRS-IC receiver in the network with the network-based CRS mitigation is slightly (at most 0.3 dB) worse than the performance in the legacy network for the worst case when the UE is in the cell edge.

The performance of legacy CRS-IC receiver in the network with network-based CRS mitigation is much better than the performance in the legacy network when the UE is in the cell center

## Type A and Type B DL control channel IM receivers

### Type A DL control IM receiver and its corresponding performance when network-based CRS-IM is employed

There are not too much details disclosed in RAN4 about proprietary CRS-IM receiver. One proprietary CRS-IM receiver is illustrated in Appendix I. The details can be found in [3]. Simulation assumption is shown in Table 1 and the performance are shown in Figure 1. For analysis, 0% load is assumed in the aggressor cells.

Table 1: Link level simulation assumption

| Parameters | value |
| --- | --- |
| Bandwidth | 10 MHz |
| Number of aggressor cells | 2 |
| Interference level (Es/N0) | 10.45 dB for aggressor cell 1 and 4.6 dB for aggressor cell 2 (PDCCH) |
| CRS-IC receiver | Assume full CRS is allocated in the aggressor cells |
| MIMO configuration | 2x2 |
| Interference modelling | When the CRS is muted, aggressor cell has no load and CRS is muted except the center 6 PRBs. When the CRS is not muted, aggressor cell has given load (0% )and CRS is transmitted across the whole bandwidth |
| CRS configuration | Non-colliding CRS is configured |
| PDCCH | 8 CCEs are used for 31 bits DCI payload (CRC is not included) |

Figure 1: Performance for PDCCH for type A DL control IM receiver with network-based CRS mitigation ON/OFF

From Figure 1, we can see that when network CRS is muting and CRS-IC receiver is enabled, the performance is very close to CRS-IC performance when neighbour CRS transmission is transmitted across the whole bandwidth and CRS-IC is enabled, which is corresponding to the legacy CRS-IC receiver under legacy network. Thus, CRS muted or not in the neighbour cell has negligible performance impact on the legacy type A DL control IM receiver.

CRS muted or not in the neighbour cell has negligible performance impact on the full blind-detection based type A DL control IM receiver

From Figure 1, we can also see that network-based CRS muting only cannot fully remove the CRS interference for control channel, since the PDCCH is scatter the whole bandwidth and the non-muted CRS of the central 6 PRB still degraded the PDCCH performance. Thus, combined with Type A receiver, additional gain can be achieved compared with non-type A receiver. The gain is about 1.4 dB.

Type A DL control IM receiver can be combined with network-based CRS-IM to improve the whole PDCCH performance

### Type B DL control IM receiver and its corresponding performance when network-based CRS-IM is employed

In Figure 2, the performance for type B receiver is shown. In this figure, colliding CRS is configured and in time domain, half subframes are full transmission and half subframes mute CRS. In frequency domain, full load PDCCH is allocated in the aggressor cell. From Figure 2, we can see that type B receiver has some performance degradation compared with full bandwidth CRS available. However, compared with traditional receiver, the type B receiver can still get significant gain even when network-based CRS-IM is employed.

Type B receiver + CRS muting have some performance degradation compared with Type B receiver + CRS non-muting, however, significant gain can still achieve compared with legacy non-IC receiver

Figure 2: Performance for PDCCH for type A DL control IM receiver with network-based CRS mitigation ON/OFF

Further, in all these simulation assumption, the CRS in control region is muted along with PDSCH. If network wishes to achieve optimal performance both for data and control channel, the CRS in control region can be kept for some subframe while CRS is muted in the data region. In this setup, it is expected the Type B receiver can achieve the optimal performance.

The type B receiver optimal performance is expected to be achieved via flexible mute neighbour cell CRS, e.g., keeping some CRS in the neighbour cell control region while muting CRS in the data region when there is no data transmission.

## NAICs receiver

For NAIC receiver, in RAN4 common understanding, the NAIC signalling shall be provided to enable NAICs feature. The signalling includes

NeighCellsInfo-r12	::=		SEQUENCE {

physCellId-r12					PhysCellId,

p-b-r12						INTEGER (0..3),

crs-PortsCount-r12				ENUMERATED {n1, n2, n4, spare},

mbsfn-SubframeConfig-r12		MBSFN-SubframeConfigList				OPTIONAL,	-- Need ON

p-aList-r12					SEQUENCE (SIZE (1..maxP-a-PerNeighCell-r12)) OF P-a,

transmissionModeList-r12		BIT STRING (SIZE(8)),

resAllocGranularity-r12			INTEGER (1..4),

...

}

In RAN4, the group has discussed a lot about whether fully blind detection for NAICs is feasible or not. RAN4 has concluded it is not feasible to performed NAICs without the signalling. The main reason is UE need to blind detect the neighbour cells p-b, transmission mode if NAICs signalling is not provided. Tremendous analysis has shown that it is not feasible to blind detection p-b and transmission mode with practical implementation and acceptable performance.

RAN4 group has common understanding NAICs signalling shall be provided to enable the NAIC feature

RAN4 group can further study the NAIC gain under network-based CRS mitigation. In case network-based CRS-IM cause some degradation of NAICs, network has the flexibility to make decision whether NAICs is enable or not to achieve the optimal system performance based on the gain of NAICs and network-based CRS muting.

Disable NAIC signalling can avoid impact on legacy NAICs UE

# Conclusion

In this paper, we provide our analysis on network-based CRS-mitigation impact on the legacy advanced receiver. We have the following observations:

Observation 1	The performance of legacy CRS-IC receiver in the network with the network-based CRS mitigation is slightly (at most 0.3 dB) worse than the performance in the legacy network for the worst case when the UE is in the cell edge.

Observation 2	The performance of legacy CRS-IC receiver in the network with network-based CRS mitigation is much better than the performance in the legacy network when the UE is in the cell center

Observation 3	CRS muted or not in the neighbour cell has negligible performance impact on the full blind-detection based type A DL control IM receiver

Observation 4	Type A DL control IM receiver can be combined with network-based CRS-IM to improve the whole PDCCH performance

Observation 5	Type B receiver + CRS muting have some performance degradation compared with Type B receiver + CRS non-muting, however, significant gain can still achieve compared with legacy non-IC receiver

Observation 6	The type B receiver optimal performance is expected to be achieved via flexible mute neighbour cell CRS, e.g., keeping some CRS in the neighbour cell control region while muting CRS in the data region when there is no data transmission.

Observation 7	RAN4 group has common understanding NAICs signalling shall be provided to enable the NAIC feature

Observation 8	Disable NAIC signalling can avoid impact on legacy NAICs UE

# Appendix I: One proprietary CRS-IM receiver

Figure 3: One Proprietary CRS-IM receiver implementation

# References

RP-171408, “New LTE WI on UE requirements for network-based CRS mitigation”, Ericsson

R4-1710180, Simulation results for CRS-IC receiver with network-based CRS interference mitigation, Ericsson, Oct. 2017.

R4-1708075, Performance analysis for legacy advanced receiver under Network-based CRS-mitigation, Ericsson, Aug. 2017