3GPP TSG-RAN WG4 Meeting #84bis	R4-1710404

Dubrovnik, Croatia, 09 - 13 October 2017

Agenda item:	6.2.4.2

Title: 	Simulation results for PDSCH in TM9 for Rel-13 non BL UE

Source: 	Intel Corporation

Document for:	Discussion

- Introduction

In RAN4# 83 it was agreed that new requirements shall be defined for UE supporting CE with 2Rx and 4Rx antennas. In RAN4#84 companies provided ideal simulation results for performance alignment to define requirements for the following tests for 2Rx and 4Rx antennas.

| Test cases | Reference in TS36.101 |
| --- | --- |
| MPDCCH CE Mode A | 8.11.2.1.1 / 8.11.2.2.1 |
| MPDCCH CE Mode B | 8.11.2.1.2 / 8.11.2.2.2 |
| PDSCH TM9 | 8.11.1.1.2 / 8.11.1.2.2 |
| PDSCH TM2 | 8.11.1.1.3 / 8.11.1.2.3 |

During the RAN4#84 meeting the simulation results from different companies were discussed and the repetition levels for 2Rx and 4Rx tests were agreed [2] as summarized below:

Repetition levels for MPDCCH tests:

- FDD

  - 2Rx

    - Mode A: Repetition number=8

    - Mode B: Repetition number=32

  - 4Rx

    - Mode A: Repetition number=2

    - Mode B: Repetition number=8

- TDD

  - 2Rx

    - Mode A: Repetition number=4

    - Mode B: Repetition number=16

  - 4Rx

    - Mode A: Repetition number=2

    - Mode B: Repetition number=4

Repetition levels for PDSCH tests:

- TM2 Mode B (FDD/TDD)

  - 2Rx

    - Repetition number=32

  - 4Rx

    - Repetition number=16

- TM9 Mode A (FDD/TDD)

  - 2Rx

    - Repetition number=4

  - 4Rx

    - Repetition number=2

With the introduction of smaller repetition levels for 2Rx and 4Rx tests, the frequency hopping interval also should to be reduced accordingly. In this contribution we present the simulation results for TM9 PDSCH performance in CE Mode A with 2Rx and 4Rx with the repetition levels and frequency hopping intervals agreed upon in [2] and [4].

# Simulation Results

The agreed simulation assumptions for TM9 tests from [4] are also listed in Appendix A.

The PDSCH performance in TM9 for 2Rx (Test2) and 4Rx (Test3) in FDD and TDD modes is captured in Figure 1 below.

Figure 1: PDSCH Performance in TM9

Based on the results we have the following observations for TM9 CE Mode A Test2 and Test 3 in FDD and TDD modes:

Observation #1: For TM9 CE Mode A Test2 in FDD, the SNR corresponding to 70% of max TP is at -6.6 dB

Observation #2: For TM9 CE Mode A Test3 in FDD, the SNR corresponding to 70% of max TP is at -6 dB

Observation #3: For TM9 CE Mode A Test2 in TDD, the SNR corresponding to 70% of max TP is at -6.7 dB

Observation #4: For TM9 CE Mode A Test3 in TDD, the SNR corresponding to 70% of max TP is at -6 dB

The SNR points for the 70% of max downlink throughput with no impairments for CE Mode A in TM9 are summarized in Table 1.

Table 1: Simulation Results for PDSCH TM9 - CE Mode A

| Duplex Type | BW | Repetition Level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| FDD | 10 MHz | 4 | R.80 FDD | EPA5 | 2 x 2 Low | 70 | -6.6 |
|  |  | 2 | R.80 FDD | EPA5 | 2 x 4 Low | 70 | -6.0 |
| TDD | 10 MHZ | 4 | R.80 TDD | EPA5 | 2 x 2 Low | 70 | -6.7 |
|  |  | 2 | R.80 TDD | EPA5 | 2 x 4 Low | 70 | -6.0 |

# Conclusions

In this contribution we have presented the ideal simulation results for PDSCH in TM9 for performance alignment for Rel-13 non-BL/CE UE requirements. Our observations are summarized below:

Observation #1: For TM9 CE Mode A Test2 in FDD, the SNR corresponding to 70% of max TP is at -6.6 dB

Observation #2: For TM9 CE Mode A Test3 in FDD, the SNR corresponding to 70% of max TP is at -6 dB

Observation #3: For TM9 CE Mode A Test2 in TDD, the SNR corresponding to 70% of max TP is at -6.7 dB

Observation #4: For TM9 CE Mode A Test3 in TDD, the SNR corresponding to 70% of max TP is at -6 dB

# References

- R4-1706196,  “WF on requirements for Rel-13 Non-BL UE”, Huawei, HiSilicon, Ericsson, Intel Corporation, Qualcomm

- R4-1708690, “Way forward on R13 non-BL UE demodulation requirements”, Huawei, HiSilicon

- R4-1709104, “Requirements alignment for  eMTC Non-BL UE demodulation”, Huawei, HiSilicon

- R4-1709039, “CR for requirements of TM9 with 2Rx and 4Rx”, Huawei, HiSilicon

# Appendix A

The simulation assumptions captured in [4] for TM9 performance requirements for FDD and half-duplex FDD are listed in the table below:

Table 1: Test Parameters for Testing CDM-multiplexed DM RS (single layer)

| parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  |  |  | dB | 0 | 0 | 0 |
|  |  |  |  | δ | dB | 3 | 3 | 3 |
|  |  |  |  | MPDCCH_RA | dB | + δ + 3 | + δ + 3 | + δ + 3 |
|  |  |  |  | MPDCCH_RB | dB | + δ + 3 | + δ + 3 | + δ + 3 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 (Note 6) | -98 (Note 6) | -98 (Note 6) |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 6 | 6 | 6 |
| PDSCH transmission mode |  |  | 9 | 9 | 9 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A | CE Mode A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 |
| Maximum number of repetitions |  |  | 8 | [4] | [2] |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Enabled | Enabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | 1 | 1 |
| Frequency hopping interval (interval-FDD) |  | ms | 8 | [2] | 1 |
| MPDCCH transmission duration (mpdcch-NumRepetition) |  | ms | 8 | [8] | [8] |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 2 | 2 | 2 |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 4 | [4] | [4] |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 24 | 24 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: For each test, DC subcarrier puncturing shall be considered. |  |  |  |  |  |

The simulation assumptions captured in [4] for TM9 performance requirements for TDD are listed in the table below:

Table 2: Test Parameters for Testing CDM-multiplexed DM RS (single layer)

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 |
|  | MPDCCH_RA | dB | + δ + 3 | + δ + 3 | + δ + 3 |
|  | MPDCCH_RB | dB | + δ + 3 | + δ + 3 | + δ + 3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.1 | Annex B.4.1 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 6 | 6 | 6 |
| Simultaneous transmission |  |  | No | No | No |
| PDSCH transmission mode |  |  | 9 | 9 | 9 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A | CE Mode A |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 |
| Maximum number of repetitions |  |  | 8 | [4] | [2] |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Enabled | Enabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | 1 | 1 |
| Frequency hopping interval (interval-TDD) |  | ms | 10 | 1 | 1 |
| MPDCCH transmission duration (mpdcch-NumRepetition) |  | ms | 8 | [8] | [8] |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 2 | 2 | 2 |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 1 | [1] | [1] |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 24 | 24 |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 4: For each test, DC subcarrier puncturing shall be considered. |  |  |  |  |  |