3GPP TSG-RAN WG4 Meeting  #84bis                                                                         R4-1710467

Dubrovnik, Croatia, 9 - 13 Oct., 2017

Source:	CATT

Title:	Draft CR to TS 37.105 for OTA Transmit ON/OFF power in section 9.5

Agenda Item:	8.27.2

Document for:	  Approval

Introduction

In RAN4#83 meeting, the WF on TS 37.105 spec drafting was approved [1].

In this contribution, draft specification text for TS 37.105 sub-clause 9.5 on OTA Transmit ON/OFF power is created for approval.

Discussion

The technical text for OTA transmit ON/OFF power is modified from the conducted Transmit ON/OFF power in Rel-13 TS 37.105 section 6.4, the changes are listed below:

1)  TAB connector is replaced with RIB.

2)  The rquirement is a co-location requirement and specified as the the spectral density from AAS BS coupling to the co-location reference antenna.

3) The OTA transmitter OFF power spectral density shall be less than -[85] dBm/MHz.

Conclusion

In this contribution, the draft specification for TS 37.105 sub-clause 9.5 on OTA Transmit ON/OFF power is created in the annex for approval.

Reference

- R4-1706126, WF on spec drafting, Huawei, RAN4#83, 15-19 May, 2017

Annex

<Text Proposal>

<Unchanged texts omitted>

- 9.5	OTA Transmit ON/OFF power

- 9.5.1	General

OTA transmitter ON/OFF power requirements apply only to TDD operation of UTRA and E-UTRA.

- 9.5.2	OTA Transmitter OFF power

- 9.5.2.1	General

OTA transmitter OFF power is defined as the mean power measured over 70 s filtered with a square filter of bandwidth equal to the Base Station RF Bandwidth (s) centred on the central frequency of the Base Station RF Bandwidth (s) during the transmitter OFF period.

The requirement applies at each RIB supporting transmission in the operating band.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

For single band RIBs supporting transmission in multiple operating bands, the requirement is applicable per supported operating band.

The requirement is a co-location requirement and specified as the spectral density from AAS BS coupling to the co-location reference antenna.

- 9.5.2.2	Minimum requirement for MSR operation

The OTA transmitter OFF power spectral density shall be less than [-85+10log10(N)] dBm/MHz, where N is the scaling factor.

- 9.5.2.3	Minimum requirement for single RAT UTRA operation

The minimum requirement for UTRA operation is the same as that defined in subclause 9.5.2.2.

- 9.5.2.4	Minimum requirement for single RAT E-UTRA operation

The minimum requirement for UTRA operation is the same as that defined in subclause 9.5.2.2.

- 9.5.3	OTA Transmitter transient period

- 9.5.3.1	General

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 9.5.3.1-1.

Figure 9.5.3.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at each RIB supporting transmission in the operating band.

- 9.5.3.2	Minimum requirement for MSR operation

The minimum requirements for MSR AAS BS OTA transmitter transient period are the same as in 3GPP TS 37.104 [9], subclause 6.4.2.1.

- 9.5.3.3	Minimum requirement for single RAT UTRA operation

The minimum requirements for single RAT AAS BS OTA transmitter transient period are the same as in 3GPP TS 25.105 [7], subclause 6.5.2.1.2.

- 9.5.3.4	Minimum requirement for single RAT E-UTRA operation

The minimum requirements for single RAT AAS BS OTA transmitter transient period are the same as in 3GPP TS 36.104 [8], subclause 6.4.2.1.

<Unchanged texts omitted>

<End of Text Proposal>