3GPP TSG-RAN WG4 Meeting #84bis                 	R4-1710717

Dubrovnik, Croatia, 9 – 13, October 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Number of cells and beams to monitor in NR

Agenda item:	9.7.3

Document for:	Discussion

# 1	Introduction

Discussions related to the UE measurement capability continued. Discussions progressed and there were many companies proposing re-using LTE requirements concerning number of cells to monitor for sub-6. It was also discussed that in addition to monitoring cells, the UE will also need to be able to monitor a number of beams per cells. Therefore, some companies raised concerns on UE complexity and how to balance the cell vs beams monitoring requirements, and whether beam monitoring requirements should be per cell or per carrier.

In this paper, we discuss this aspect and present our view.

# 2	Discussion

## One cell is represented by a physical cell Id which is decoded from the SS-Block. I.e. cell detection is done by detecting an SS-Block. Network can choose to cover the cell by use of one or more beams. I.e. the network can transmit the SS-Block with same physical cell Id using one or multiple beams if this is necessary. E.g. in lower carrier frequencies a cell could cover the whole sector using a single beam, while at higher carrier frequencies the network would need to be using multiple beams in order to cover the sector and ensure cell coverage.

Which approach is applied will be agnostic to the UE and UE procedures. In order to provide a full physical cell identification of a cell, the physical cell identity obtained from the SS-Block together with the SS-Block Index is needed.

## 2.1	Number of cells to monitor

RAN1 agreed, that there can be up to ‘L’ SS-Blocks per SS-Block burst set per cell – i.e. up to ‘L’ different beams. A cell transmits SS-Blocks with same physical cell ID by using one beam or more beams. This means, one beam could be transmitting one SS-block for covering one cell, or the same one cell could be covered by using more beams - each beam SS-Block having same physical cell ID.

Observation 1: A cell is identified by its Physical Cell Id

L is a maximum limit of SS-Blocks which can be present in an SS-Block burst set in a cell. all SS-Blocks would have one and same physical cell ID, and each SS-Block distinguished by the SS-Block Index.

Observation 2: There can be up to L SS-Blocks per SS-Block burst set per cell

Observation 3: Each SS-Block is distinguished by its Index

The UE will likely not be able to detect all SS-Blocks transmitted from a cell, except if the number of SS-blocks is low. If the network on the other hand is using narrow cell and many beams to ensure cell coverage, the UE could potentially detect a very high number of cell and/or SS-Blocks. Additionally, the UE will in many situations be able to detect SS-Blocks transmitted from other cells. How many SS-Blocks the UE can detect and the exact number of cells/SS-Blocks the UE would need to be able monitor, would need to be investigated using simulation support.

Based on results shown in [1] we propose:

Proposal 1: For below 6GHz, re-use E-UTRAN requirements concerning number of intra-frequency cells the UE shall be able to monitor.

We propose also to re-use LTE requirements for sub-6GHz inter-frequency number of cells in NR:

Proposal 2: For below 6GHz, re-use E-UTRAN requirements concerning number of inter-frequency cells the UE shall be able to monitor.

Many companies have provided simulation results. Based on the results in our results [1] we observe following:

Observation 4: At higher frequencies, the UE is able to detect more cells than at lower frequencies.

Which also means that the UE should be able to monitor more beams at higher carrier frequencies than at lower. The actual number of cells per carrier for higher carrier frequencies would need more study in RAN4

## 2.2	Number of beams to monitor

If considering below 6GHz we see that L can be either 4 or 8 depending on whether the carrier frequency is below or above 3GHz. If UE is required to be able to monitor e.g. 8 cells on serving carrier for carrier frequencies below 6GHz (as in LTE) it would mean that in worst case the UE would need to be able to monitor 32 or 64 SS-Blocks depending on the serving carrier frequency. For inter-frequency requirements, the number of cells is 4 per carrier which wold lead to beam monitoring requirement of 16 and 32 per carrier depending on the carrier frequency.

Such requirements could be regarded as being a bit stringent and one could question if under realistic assumptions such requirements would even be needed.

In our simulation paper [1] we have collected one set of results related total number of cells and beams per cell (and total number of beams) the UE can detect for 30GHz. And there we see that under those specific simulation assumptions, the total number of beams the UE can detect is not higher than 14. I.e. it is not as high as e.g. 8x64=512. The actual number however, depend heavily on the assumptions. If we e.g. consider 30GHz it could under some deployments be necessary to use more cell sectors and narrow beam forming in order to ensure the necessary planned cell coverage. Under such conditions it is highly likely that the UE will be able to detect more cells and more beam per cell – and therefore also total number of beams.

In last meeting, there was a discussion whether beam monitoring requirements should be per cell or per frequency. When defining monitoring requirements for number of beams (beams and index) it seems reasonable to define this requirements as a per carrier frequency requirement.

Proposal 3: Define beam monitoring requirements per carrier frequency.

Simulation results seems to indicate that the UE can detect more cells at higher carrier frequencies than at lower carrier frequencies. This means UE should also be able to monitor more beams as well at higher carrier frequencies than at lower.

Observation 5: At higher frequencies, the UE is able to detect more beams than at lower frequencies.

Still to be discussed is the actual number of beams to monitor for different carrier frequency ranges, which would need further study in RAN4.

## 2.2	Text proposal for TR

We recognize that more simulation work is likely needed in order to determine the exact numbers. However, it would already now be possible to capture the requirements text related to intra-frequency monitoring requirements for SS-Block cell based measurements. Below we propose text proposal regarding such requirements – i.e. the UE monitoring requirements for below 6GHz and above 6GHz:

### 2.2.1	Intra-frequency monitoring requirement

Below 6GHz:

In the RRC_CONNECTED state the measurement period for intra frequency measurements is [FFS] ms. When no measurement gaps are activated, the UE shall be capable of performing [SS-RSRP], [SS-RSRQ], and [SS-SINR] measurements for [8] identified-intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of [FFS] ms.

Above 6GHz:

In the RRC_CONNECTED state the measurement period for intra frequency measurements is [FFS] ms. When no measurement gaps are activated, the UE shall be capable of performing [SS-RSRP], [SS-RSRQ], and [SS-SINR] measurements for [FFS] identified-intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of [FFS] ms.

### 2.2.2	Inter-frequency monitoring requirement

TP for inter-frequency is provided in [2].

# 3	Conclusion

In this paper, we continued the discussions related to the UE measurement capability continued. Based on the discussions and simulation results we propose following:

Proposal 1: For below 6GHz, re-use E-UTRAN requirements concerning number of intra-frequency cells the UE shall be able to monitor.

Proposal 2: For below 6GHz, re-use E-UTRAN requirements concerning number of inter-frequency cells the UE shall be able to monitor.

Proposal 3: Define beam monitoring requirements per carrier frequency.

We also note that the actual number however, depend heavily on the assumptions. If we e.g. consider 30GHz it could under some deployments be necessary to use more cell sectors and narrow beam forming in order to ensure the necessary planned cell coverage. Under such conditions it is highly likely that the UE will be able to detect more cells and more beam per cell – and therefore also total number of beams.

# References

- R4-1710720, Further input on SLS results, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1710722, TP for TR on inter-frequency measurement requirements, Nokia, Nokia Shanghai Bell