3GPP TSG-RAN WG4 Meeting #84bis                                         R4-1710727

Dubrovnik, Croatia, 9 -13 October, 2017

Agenda Item:	9.4.2.1

Source:	Apple Inc.

Title:	Performance Evaluation of LTE NR DC Dual UL and Single UL

Document for:	Discussion/Decision

# Introduction

It was endorsed in RP #75 ([1][2]) that NR Rel.15 will prioritize the option 3 non-standalone (NSA) mode as the first phase of NR specification, in order to expedite the initial NR deployment. The option 3 NSA mode, as shown in Figure 1, operates in inter-RAT dual connectivity, where the control plane signaling goes through LTE eNB and user plane payload can be sent through NR gNB [3].

Figure 1  5G NSA RAN architecture 3 and 3A

As explained in [4][5][6], depending on the band combinations, sending two concurrent uplinks in two different bands can produce undesirable inter-modulations that degrades UE’s RX sensitivity. This creates challenges in UE RF implementations. In order to avoid serious receiver sensitivity degradations on any agreed LTE/NR band combinations, it is imperative that some other mechanisms to be specified in Rel-15 NR work item phase to avoid simultaneous LTE and NR uplink transmissions from a single UE perspective when such issue occurs.

Specifically, an agreement was made in RAN1 #89 [7] to support an optional mode so that the NSA mode UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers to avoid simultaneously transmitting on two carriers when such self-interference exists:

Agreements:

- For NR NSA for a UE, NR supports the case that when the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers

  - FFS whether or not there is specification impact

    - If there is RAN1 specification impact, aim to minimize the specification impact for NR

    - Note: this feature by itself is not intended to have any LTE RAN1 specification impact

  - Note: the other case of allowing simultaneous operation on two or more UL carriers is already agreed to be supported

To provide specific mechanism/solution to avoid UE simultaneous uplink transmission, in RAN1 NR#2 in June, following agreement [8] was made to provide two mechanisms for single UL transmission:

Agreements:

- Support the following solution to single UL transmission where NW synchronization between eNodeB and gNodeB is assumed (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency)

  - When UE is activated with multiple UL carriers on different frequencies, time-switching of LTE UL carrier and NR UL carrier is used

    - UL transmission timing pattern of LTE carrier and NR carrier is semi-statically shared between eNodeB and gNodeB

    - FFS: Signaling to UE of UL transmission timing pattern

  - UE simultaneously receives signals/channels from both NR DL carrier and LTE DL carrier

    - For scheduling/HARQ timing of LTE FDD carrier, the following timing can be considered, e.g., for LTE:

      - DL-reference UL/DL configuration for TDD

      - DL-reference UL/DL configuration defined for FDD-SCell in TDD-FDD CA with TDD-PCell

      - Up to NW implementation (i.e., no RAN1 spec. impact)

    - For scheduling/HARQ timing of NR carrier, no special handling would be necessary

  - Other solutions are not precluded

In RAN1#90 in August [9], specific TDM patterns were selected and further agreed:

Agreements:

- When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers

  - For LTE carrier, UE can be configured with

    - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell

      - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied

      - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration

      - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact

    - Case 2: Release 15 LTE-FDD HARQ timing

      - No impact on LTE RAN1 specifications

    - Note: it doesn’t necessarily imply that UE has to support both cases

In addition, in RAN plenary meeting in Sept, the following agreements [10][11][12] was reached

- Define in RAN4 specs for which band combinations and channel allocations (to the extent specified by RAN4) within that band combination the UE is allowed to indicate that it does not support 2 simultaneous UL tx (this is not implying the granularity of the signalling)

- Signalling to be defined to support the 'red text' from RAN4 part of the single tx discussion (i.e. as in RP-172064):

  - UE capability indicates that the UE does not allow 2 simultaneous UL transmission for the RAN4 specified channel allocations in a given band combination. If the network chooses to operate the UE in a way that is not consistent with this capability indication then the UE behavior is not specified and the UE might not meet the performance criteria.

- The categorization of band combinations is up to RAN4

  - RAN4 is to define a rule for categorization (of band combinations including channel assignments) in order to apply them

    - A possible rule could be based on presence of resulting IMD[2]…[7] (including harmonics as long as the cause includes simultaneous Tx) falling in Rx with the given channel assignment

In this contribution, we provide the numerical results comparing both the DL and UL performance of dual UL and single UL.

# UL Performance Comparison between Dual and Single UL

In this session, we provide the numerical results to compare the UL performance between dual LTE/NR UL and single UL in NSA DC mode.

We summarize the simulation settings in the following Table

Table 1 Evaluation Settings for Dual and Single UL performance evaluation

|  | LTE | NR |
| --- | --- | --- |
| Carrier Frequency | 1.8GHz | above 3GHz |
| Aggregate System Bandwidth | 10MHz | 80MHz |
| Thermal Noise Level | -174dBm/Hz |  |
| LTE Path Loss (d in meter) | 36.873, Assume bore side direction PL = 22.0log10(d) + 28.0 + 20log10(fc); 10m<d<288m PL = 40log10(d)+28.0+20log10(fc) -44.3; d>288m |  |
| LTE Penetration Loss | 20dB |  |
| NR Path Loss | 10dB worse than LTE |  |
| UL Power Control | Target = 25dB over Thermal, Alpha = 0.9 |  |
| UE Tx Power | 23dBm |  |
| UE Tx Power Split in DC | 20dBm | 20dBm |
| UE Receiver Noise figure | 9dB |  |
| UE Antenna Config. | 1Tx/2Rx | 2Tx/4Rx |
| UE Antenna Gain | 0dBi |  |
| BS Tx Power | 49dBm |  |
| BS Receiver Noise Figure | 5dB |  |
| BS Antenna Elements Gain | 8dBi |  |
| #BS Antenna Elements | 64 | 256 |
| BS Antenna Configuration | 2Tx/2Rx | 32Tx/32Rx |

Next, we discuss the assumption on the DL and UL transmission configuration for LTE and NR, we assume

- LTE is FDD deployment with dedicated UL and DL carrier

- NR is TDD deployment, with DL and UL slot ratio of 6:4, i.e. 60% slots for DL transmission and 40% slots for UL transmission

Given this assumption, we have the following settings for dual UL transmission and single UL transmission as illustrated in Figure 2

- Dual Uplink: For 40% of slots, it is simultaneous LTE and NR uplink transmission. For the rest 60% slots, it is LTE only uplink transmission as NR is in TDD DL operation

- Single Uplink: For 40% of slots, it is NR uplink transmission. For the rest 60% slots, it is LTE uplink transmission.

Figure 2 LTE and NR UL Transmission Configuration for Dual and Single UL

Before going into the detailed simulation results, there are a few highlight we would like to point out regarding the LTE and NR deployment difference

- NR carrier experiences higher path loss compared to LTE carrier due to the fact that it is at higher frequency band.

- NR deployment can have wider bandwidth compared to LTE deployment

- NR deployment can also support larger number of antenna elements, higher order spatial multiplexing (MIMO), etc. compared to LTE deployment

At high level, the performance different between dual UL and single UL depends on the UE operation region (or the location of the UE).

- When UE is severely power limited (UE is at the cell edge), the additional bandwidth would not improve the UL performance. The best strategy to improve the coverage is for UE to transmit all its power on the carrier that has the smallest path loss, i.e. LTE carrier.

- When UE moves closer to the cell center, UE UL performance starts to become dimension limited, such that wider bandwidth starts to improve the UL performance.

- When UE becomes fully dimension limited, i.e. UE transmit at below maximum power limitation due to the power control constraint, NR carrier could provide majority of the UL throughput due to the fact that NR carrier typically has wider bandwidth, and support higher order spatial multiplexing.

Figure 3 shows the UL transmit power on LTE and NR carrier as well as achievable UL geometry on each carrier as a function of path loss for dual UL transmission assuming fixed power split of 20dBm + 20dBm between LTE and NR. It is notable that LTE carrier requires much less transmit power compared to NR mainly due to two reasons (1) LTE carrier has smaller path loss compared to NR carrier (2) LTE carrier has narrower bandwidth compared to NR carrier

Figure 3 UE Tx power and UL geometry for dual UL transmission

Figure 4 provides UL throughput comparison between dual uplink and single uplink transmission. It is interesting to note that

- At high path loss (cell edge), single UL performs the best as expected, due to the fact that LTE carrier has smaller path loss at lower frequency band

- At low path loss (cell center), dual UL performs the best as system becomes dimension limited.

We also try to intuitively explain why at cell edge, when UE is power limited, the best strategy is to transmit all the power on the carrier with the smallest path loss. According to Shannon capacity, the achievable data rate is r=W∙log(1+PW∙n0) where W is the bandwidth, P is the total received power and n0 is noise spectral density. At the cell edge when UE is power limited, the SNR on UL is very low, log operation can be approximated by the linear operation, which suggests that capacity grows linearly as the SNR grows, i.e. r≈W∙PW∙n0=Pn0. This shows that when UE is power limited, increasing bandwidth will not increase the throughput, instead, it is more important to increase the received power. To increase the receive power, UE should transmit on the carrier with the smallest path loss.

Figure 4 UL throughput comparison between dual UL and single UL

Figure 5 provides the gain of dual UL with respect to single UL.

- At very cell edge, e.g. path loss of 140dB, dual UL performs worse than single LTE UL by around 20%

- As the path loss reduces, dual UL starts to perform better than single UL for up to 4-5%

Figure 5 UL throughput comparison dual UL gain over single UL: Path Loss

From the performance comparison, we can make the following observations

Observation 1: for LTE-NR DC UE’s that are power limited in the uplink, there is negligible impact, or even performance gain, on uplink throughput using non-simultaneous UL compared to simultaneous UL.

Observation 2: for LTE-NR DC UE’s that are not power limited in the uplink, there is limited performance gain on uplink throughput using simultaneous UL compared to non-simultaneous UL.

Figure 5 shows the UL throughput gain of dual UL as a function of path loss on LTE carrier. To be more representative, it is useful to show the gain as a function of the coverage as well. To show the coverage, each path loss point is converted to the distance, i.e. dPL. Assuming a cell has the coverage of 140dB, then for each path loss, the normalized coverage is computed as dPL/d140dB2. Figure 6 shows the gain of dual UL as a function of the coverage. As shown in the figure, UEs in only a small percentage of the cells benefit from dual UL with limited gain.

Figure 6 UL throughput comparison dual UL gain over single UL: Coverage

# DL IMD issue with Dual UL

As explained in [4][5][6], depending on the band combinations, sending two concurrent uplinks in two different bands can produce undesirable inter-modulations that degrades UE’s RX sensitivity. This creates challenges in UE RF implementations. To quantitatively understand the severity of the impact on UE Rx sensitivity, we use the following table which summarizes the Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations in 36.101

|  | MSE Values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Average MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IM2 | 24 | 26 | 29.8 | 32.5 | 26 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.05 |
| IM3 | 23 | 25.7 | 5 | 7.7 | 12 | 12 | 14.7 | 12.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 14.03 |
| IM4 | 6 | 8.7 | 4 | 13 | 15.7 | 8 | 4 | 6.7 | 9 | 11.7 | 4 | 6.7 | 9 | 11.7 | 4 | 6.7 | 8.2 | 8 | 10.7 | 15 | 17.7 | 8.98 |
| IM5 | 5 | 6.4 | 2.5 | 5.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.85 |

From the Table, the 5th order IM, IM5, can still cause close to 5dB Rx sensitivity degradation at the UE side. From the UL perspective, Figure 6 shows that, compared to single UL, for majority of the UEs, dual UL suffers performance loss. Even for small percentile of UEs that dual UL performs better, the gain is very limited. Therefore, we have the following proposals

Proposal 1: At least for “difficult” band/channel combinations, introduce 1-bit indicator as part of the UE RF capability message, to indicate that the UE does not support simultaneous UL transmission.

Proposal 2: To define “difficult” band/channel combinations (including at least channel assignments), using the rule based on at least the presence of IM5

# Conclusions

In this contribution, we provide the numerical results comparing the performance of dual UL and single UL performance from both the UL and DL perspective. Based on the evaluation results, we can make the following observations

Observation 1: for LTE-NR DC UE’s that are power limited in the uplink, there is negligible impact, or even performance gain, on uplink throughput using non-simultaneous UL compared to simultaneous UL.

Observation 2: for LTE-NR DC UE’s that are not power limited in the uplink, there is limited performance gain on uplink throughput using simultaneous UL compared to non-simultaneous UL.

We also propose the following

Proposal 1: At least for “difficult” band/channel combinations, introduce 1-bit indicator as part of the UE RF capability message, to indicate that the UE does not support simultaneous UL transmission.

Proposal 2: To define “difficult” band/channel combinations (including at least channel assignments), using the rule based on at least the presence of IM5

# References

- RP-170857, “Draft Report of 3GPP TSG RAN meeting #75”, 3GPP TSG RAN meeting #75, Dubrovnik, Croatia, March 6th-9th, 2017.

- RP-170741, “Way Forward on the overall 5G-NR eMBB workplan”, 3GPP TSG RAN meeting #75, Dubrovnik, Croatia, March 6th – 9th, 2017.

- 3GPP TR 38.801, “Study on new radio access technology: Radio access architecture and interfaces”, V14.0.0, 2017-03.

- R1-1708276, “Uplink sharing in NSA mode”, 3GPP RAN1#89, Hangzhou, China, May 15th-19th, 2017.

- R4-1704759, “Co-existence analysis on LTE-NR band combinations”, 3GPP RAN4 #83, Hangzhou, China, May 15th-19th, 2017.

- R4-1704411, “Proposal of frequency ranges for LTE-NR coexistence with UL sharing”, RAN4#82bis, Spokane, US, April 3-7, 2017

- 3GPP RAN1 #89 Chairman Notes, “Chairman’s Notes RAN1 #89 final”, Hangzhou, China, May 15th-19th, 2017.

- 3GPP RAN1 NR AdHoc #2 Chairman Notes, “Chairman’s Notes RAN1_NRAH2_final”, Qingdao, China, June 27th-30th, 2017

- 3GPP RAN1 #90 Chairman Notes, “Chairman’s Notes RAN1 #90 final”, Prague, Czech, Aug. 21th-25th, 2017.

- RP-172100, “LS on single Tx”, RAN, Sapporo, Sept. 11 – Sept.14, 2017

- RP-172064, “Single Tx switched UL”, RAN, Sapporo, Sept. 11 – Sept.14, 2017

- RP-172085, “Signalling for Single/Dual UL Tx”, RAN, Sapporo, Sept. 11 – Sept.14, 2017