3GPP TSG-RAN WG4 Meeting #84bis	R4-1710786

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	5.4.5.2

Source:	Ericsson

Title:	Simulation results of eMTC PDSCH for non-BL CE UE

Document for:	Discussion

# 1	Introduction

RAN4#84 agreed with the PDSCH parameters for Rel-13 Non-BL CE UEs [1][2]:

This contribution presents our simulation results for PDSCH considering 2Rx/4Rx UE supporting coverage enhancements. We also discuss the MPDCCH repetition number used for PDSCH test.

# 2	Simulation results

## 2.1	TM9 CE Mode A

Table 1	Parameters for TM9 CE Mode A test.

| Test case | Duplex | Rx | Repetition | Interval |
| --- | --- | --- | --- | --- |
| TM9 CEModeA | FDD | 2 | 4 | 1 |
| TM9 CEModeA | FDD | 4 | 2 | 1 |
| TM9 CEModeA | TDD | 2 | 4 | 1 |
| TM9 CEModeA | TDD | 4 | 2 | 1 |

Figure 1 shows the PDSCH TM9 CE Mode A simulation results with 2Rx/4Rx based on the simulation parameters specified in [2]. Table 1 shows the summary of the parameters. Table 2 gives the required SNR [dB] to achieve 70% of relative throughput.

Figure 1	PDSCH TM9 CE Mode A FDD (left) and TDD (right).

Table 2	Required SNR [dB] for 70% of relative throughput.

| Duplex mode | Number of receive antennas | Ideal results | Impairment results |
| --- | --- | --- | --- |
| FDD | 2Rx | -5.8 | -3.8 |
|  | 4Rx | -6.2 | -4.2 |
| TDD | 2Rx | -5.4 | -3.4 |
|  | 4Rx | -5.6 | -3.6 |

## 2.2	TM2 CE Mode B

Table 3	Parameters for TM2 CE Mode B test.

| Test case | Duplex | Rx | Repetition | Interval |
| --- | --- | --- | --- | --- |
| TM2 CEModeB | FDD | 2 | 32 | 16 => 8 |
| TM2 CEModeB | FDD | 4 | 16 | 8 => 4 |
| TM2 CEModeB | TDD | 2 | 32 | 20 => 10 |
| TM2 CEModeB | TDD | 4 | 16 | 10 => 5 |

Figure 2 shows the PDSCH TM2 CE Mode A simulation results with 2Rx/4Rx based on the simulation parameters specified in [1]. Table 3 shows the summary of parameters. Note we change the frequency hopping interval parameters in the simulation. This is because the interval parameter agreed in [1] can uses only 2 narrowbands although the number of narrowbands for CE Mode B uses 4. Table 4 gives the required SNR [dB] to achieve 70% of relative throughput.

Figure 2	PDSCH TM2 CE Mode B FDD (left) and TDD (right).

Table 4	Required SNR [dB] for 70% of relative throughput.

| Duplex mode | Number of receive antennas | Ideal results | Impairment results |
| --- | --- | --- | --- |
| FDD | 2Rx | -17.2 | -15.2 |
|  | 4Rx | -17.4 | -15.4 |
| TDD | 2Rx | -17.1 | -15.1 |
|  | 4Rx | -17.4 | -15.4 |

# 3	MPDCCH repetition numbers

This section discusses the MPDCCH repetition numbers during the PDSCH test. PDSCH demodulation needs the MPDCCH decoding in advance, and the decoding success rate should be almost 100% to avoid the impact to the PDSCH performance. One of the easiest way to ensure 100% decoding success rate is to set the largest repetition level, 256, for MPDCCH, but it makes the test time longer. Therefore, we need to choose the minimum MPDDCH repetition number which satisfy 100% decoding rate.

## 3.1	CE Mode A

Table 5 shows our proposal for MPDCCH repetition number used for PDSCH CE Mode A test. We choose these numbers assuming 3dB MPDCCH power boosting as same as the existing Rel-13 BL/CE UE demodulation requirements.

For reference, we show our ideal MPDCCH simulation results for different repetition level in Figure 3 and Figure 4. The simulation parameters are same in [3].

Table 5	Proposed MPDDCH repetition number for PDSCH CE Mode A test.

|  |  | PDSCH repetition | PDSCH SNR (ideal) | Proposal MPDCCH repetition | Required SNR for 99% MPDDCH decoding success rate (ideal) |
| --- | --- | --- | --- | --- | --- |
| FDD | 2Rx | 4 | -5.8 | 2 | -5.0 |
|  | 4Rx | 2 | -6.2 | 1 | -7.4 |
| TDD | 2Rx | 4 | -5.4 | 2 | -5.4 |
|  | 4Rx | 2 | -5.6 | 1 | -7.5 |

Figure 3	MPDCCH CE Mode A simulation results for 2Rx UE.

Figure 4	MPDCCH CE Mode A simulation results for 4Rx UE.

## 3.2	CE Mode B

Table 6 shows our proposal for MPDCCH repetition number used for PDSCH CE Mode B test. We choose these numbers assuming 3dB MPDCCH power boosting.

For reference, we show our ideal MPDCCH simulation results for different repetition level in Figure 5 and Figure 6. The simulation parameters are same in [3].

Table 6	Proposed MPDDCH repetition number for PDSCH CE Mode B test.

|  |  | PDSCH repetition | PDSCH SNR (ideal) | Proposal MPDCCH repetition | Required SNR for 99% MPDDCH decoding success rate (ideal) |
| --- | --- | --- | --- | --- | --- |
| FDD | 2Rx | 32 | -17.2 | 32 | -16.8 |
|  | 4Rx | 16 | -17.4 | 8 | -16.0 |
| TDD | 2Rx | 32 | -17.1 | 32 | -17.0 |
|  | 4Rx | 16 | -17.4 | 8 | -16.0 |

Figure 5	MPDCCH CE Mode B simulation results for 2Rx UE.

Figure 6	MPDCCH CE Mode B simulation results for 4Rx UE.

# 4	Conclusion

The table below summarizes our simulation results for non-BL UE PDSCH demodulation requirements.

Proposal 1: We propose to take our results into account for the PDSCH demodulation requirements.

| CE Mode | Duplex mode | Number of receive antennas | PDSCH repetition number (Same as [1][2]) | Impairment results | MPDCCH repetition number (assuming 3dB power boosting) | Frequency hopping interval |
| --- | --- | --- | --- | --- | --- | --- |
| CE Mode A | FDD | 2Rx | 4 | -3.8 | 2 | 1 |
|  |  | 4Rx | 2 | -4.2 | 1 | 1 |
|  | TDD | 2Rx | 4 | -3.4 | 2 | 1 |
|  |  | 4Rx | 2 | -3.6 | 1 | 1 |
| CE Mode B | FDD | 2Rx | 32 | -15.2 | 32 | 8 |
|  |  | 4Rx | 16 | -15.4 | 8 | 4 |
|  | TDD | 2Rx | 32 | -15.1 | 32 | 10 |
|  |  | 4Rx | 16 | -15.4 | 8 | 5 |

# 5	References

R4-1709038, “CR for requirements of TM2 with 2Rx and 4Rx”, Huawei, HiSilicon.

R4-1709039, “CR for requirements of TM9 with 2Rx and 4Rx”, Huawei, HiSilicon.

R4-1710787, “Simulation results of MPDCCH for non-BL CE UE” Ericsson.