3GPP TSG-RAN WG4 Meeting #84bis	R4-1710794

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	6.2.4.2

Source:	Ericsson

Title:	Simulation results of FeMTC PDSCH for non-BL CE UE

Document for:	Discussion

# 1	Introduction

RAN4#84 agreed to introduce the new PDSCH demodulation requirements for non-BL UE supporting 96PRB [1]. This contribution gives out ideal simulation results and discuss the PDSCH repetition number.

# 2	Discussion

Table 1 summarizes the parameters assumed for Rel-14 non-BL UE PDSCH demodulation requirements. Figure 1 is the ideal simulation results.

Table 1	Parameters for Rel-14 non-BL UE PDSCH demodulation requirements.

| Test number | System bandwidth and MCS | Propagation channel | Correlation Matrix and Antenna Configuration | Transmission mode | Coverage enhancement mode | Allocated PRBs for PDSCH | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20MHz QPSK 1/3 | EPA5 | 2x2 Low | TM2 | CE Mode A | 87 | 7736 bits |
| 2 | 20MHz QPSK 1/3 | EPA5 | 2x4 Low | TM2 | CE Mode A | 87 | 7736 bits |

Figure 1	Simulation results for Rel-14 Non-BL UE PDSCH demodulation for 2Rx UE (left) and 4Rx UE (right).

Table 2 lists the required SNR values to achieve 70% of the maximum throughput. Considering the target SNR = -6dB, we propose to set the repetition number to 4 for 2Rx UE and to 2 for 4Rx UE. With these repetition numbers, the required SNR seems lower, i.e., about -9.5dB. However, if we consider the lower repetition level, it becomes 1 (no repetition) for 4Rx UE. In this case, there is no meaning to introduce the test case for coverage enhancement. Therefore we propose to set these repetition numbers although the final requirement would become 1-2dB lower than the target SNR.

Table 2	Summary of PDSCH simulation results.

| RX antennas | Repetition number | Required SNR [dB] to achieve 70% of the maximum throughput |
| --- | --- | --- |
| 2 | 2 | -6.7 |
|  | 4 | -9.5 |
|  | 8 | -12.0 |
| 4 | 2 | -9.6 |
|  | 4 | -12.1 |
|  | 8 | -15.5 |

# 3	Conclusion

Proposal: For Rel-14 non-BL CE PDSCH demodulation requirements, set the repetition numbers as follows:

| RX antennas | PDSCH Repetition number |
| --- | --- |
| 2 | 4 |
| 4 | 2 |

# 4	References

R4-1708702, “Way forward on UE demodulation requirement for Rel-14 FeMTC”, Ericsson, Intel, Qualcomm, Samsung.

R4-1707671, “PDSCH demodulation requirements for UE DL Cat-M2”, Ericsson.