3GPP TSG-RAN WG4 Meeting #84bis	R4-1710803

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	9.7.11.1

Source:	Ericsson

Title:	Initial simulation results for PBCH demodulation

Document for:	Discussion

# 1	Introduction

RAN4 NR AH3 agreed with the simulation assumption for NR PBCH to derive the basic SI reading time [1]. The simulation assumption is used to derive the required time for 1) PBCH decoding and 2) PBCH-DMRS time index acquisition. This contribution results our initial simulation results.

# 2	Simulation results

Table 1 lists the parameters we used for evaluation.

Table 1	Simulation parameters

| Parameters | Values |  |
| --- | --- | --- |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier spacing | 15kHz, 30kHz | 120kHz, 240kHz |
| Number of Tx antennas | 1 |  |
| Number of Rx antennas | 2 (w/o receiver beam-forming) |  |
| Number of transmitted SS/PBCH blocks within an SS burst set period (K) | 1 |  |
| SS burst set periodicity | 20ms |  |
| PBCH-DMRS power offset with respect to PSS/SSS | 0dB |  |
| PBCH Payload (including CRC) | 56 bits (including 19 bits CRC) |  |
| PBCH SNR | -10dB, -8dB, -6dB, -4dB, -2dB, 0dB |  |
| Propagation condition | EPA5, ETU70 | CDL-C, TDL-C RMS delay spread = 100ns |
| Doppler frequency | See above | 83Hz (3km/h@30GHz) |
| Detection method | One shot detection |  |

## 2.1	PBCH-DMRS time index acquisition

According to RAN1 agreements, PBCH-DMRS sequence initialization is based on Cell ID and 3 LSB of SS/PBCH block time index. Since the number of SS/BPCH blocks per SS burst can be set up to 8 in case sub sub-6GHz operation, 3 LSB for PBCH-DMRS sequence initialization can be used to identify the OFDM symbol location (time index). In the case of above 6GHz, the number of SS/PBCH blocks can be transmitted up to 64. This means 3 more bits are needed to identify the time index and this information is included in the NR-MIB. According to the simulation assumption, PBCH-DMRS time index reading evaluation does not include PBCH decoding. This means this scenario is applicable for sub-6GHz operation.

Table 2 shows the simulation results for PBCH-DMRS time index acquisition. For this simulation we assume UE knows the Cell ID.

Table 2	Number of attempts for PBCH-DMRS time index acquisition to achieve 99% decoding success rate (4GHz, 2Rx)

| SCS | Fading channel | SNR=-10dB | SNR=-8dB | SNR=-6dB | SNR=-4dB | SNR=-2dB | SNR=0dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | EPA5 | 2 | 2 | 1 | 1 | 1 | 1 |
|  | ETU70 | 3 | 2 | 2 | 2 | 2 | 2 |
| 30kHz | EPA5 | 2 | 2 | 1 | 1 | 1 | 1 |
|  | ETU70 | 4 | 3 | 3 | 2 | 2 | 2 |

## 2.2	PBCH decoding

PBCH decoding performance is used to derive the NB-MIB acquisition time requirements. Simulation needs the detailed design for channel coding (i.e., Polar coding) and PBCH-DMRS sequence. However, RAN1 is still discussing the channel coding design, therefore our results are based on one of the candidate methods discussing in RAN1.

PBCH soft-combining across the different SS burst is possible for the PBCH with the same time index. For sub-6GHz, we assume UE will perform soft-combining after UE acquire the time index of each SS/PBCH block from the PBCH-DMRS. For mmWave, however, a part of time index information (3 MSBs) is transmitted in NR-MIB. This means UE needs some hypothesis testing for 3 MSBs for soft-combining. In our understanding RAN1 is discussing the PBCH channel coding method and some proposals help to perform PBCH soft-combining. In this contribution, we provide the one-shot detection result only.

Another aspect is the channel coding. RAN1 agreed to use the same polar code construction as for control channel, however RAN1 has not completed the exact coding design for BPCH. Our contribution uses the same method as DCI, that is, distributed CRC bits with associated interleaver and 24 bits CRC polynomial length.

Table 3 and Table 4 show the simulation results for the number of attempts for NR-MIB acquisition with 99% success rate. Note the results also consider the PBCH-DMRS time index acquisition, therefore the number of attempts are equal or larger than PBCH-DMRS results in the case of sub-6GHz scenario.

Table 3	Number of attempts for PBCH to achieve 99% decoding success rate (4GHz, 2Rx)

| SCS | Fading channel | SNR=-10dB | SNR=-8dB | SNR=-6dB | SNR=-4dB | SNR=-2dB | SNR=0dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | EPA5 | 8 | 4 | 3 | 2 | 2 | 1 |
|  | EVA70 | 20 | 7 | 4 | 2 | 2 | 2 |
| 30kHz | EPA5 | 8 | 4 | 3 | 2 | 2 | 1 |
|  | EVA70 | 35 | 9 | 4 | 3 | 2 | 2 |

Table 4	Number of attempts for PBCH to achieve 99% decoding success rate (30GHz, 2Rx)

| SCS | Fading channel | SNR=-10dB | SNR=-8dB | SNR=-6dB | SNR=-4dB | SNR=-2dB | SNR=0dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120kHz | TDL-C, 100ns, Doppler=83Hz | 19 | 7 | 4 | 2 | 2 | 2 |
| 240kHz | TDL-C, 100ns, Doppler=83Hz | 54 | 16 | 8 | 5 | 4 | 3 |

# 3	Summary

We provided our initial simulation results. As we discussed, RAN1 is still discussing the detailed design for PBCH and it affects to our evaluation. RAN4 need further evaluation according to the RAN1 decision, and also more discussion on the detailed simulation assumption for the SI reading procedure.

# 4	References

R4-1709908, “Updated PBCH Simulation assumption”, Huawei, HiSilicon.

# Appendix

Simulation assumption [1].

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 4GHz; 30GHz |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2, 4 (uncorrelated. Option 1: With receiver beamforming Option 2: Without receiver beamforming) |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 0 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | To be indicated |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (Range: 40 ~ 72 bits) |
| PBCH SNR | dB | -12 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition | - | EPA5, ETU70 for 4GHz CDL/TDL-C with desired RMS delay spread 100ns Note1 for 30GHz |
| UE speed and Doppler Shift | - | 3km/h speed with 83Hz Doppler at 30GHz |
| Detection Method |  | Baseline: on one shot detection (No combination for different PBCHs,) Soft- combining across PBCHs in different SS burst sets (2,4 combining) |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.900. |  |  |