3GPP TSG-RAN WG4 Meeting #84bis	R4-1710952

Dubrovnik, Croatia, 9 – 13 October 2017

Source: 	Nokia

Title: 					TP for TR 36.715-02-01: CA_2DL_14A-30A_1UL_BCS0

Agenda Item: 			8.2.2

Document for:	Approval

# 1	Introduction

This contribution provides a TP for TR 36.715-02-01 to include the requirements for the 2DL/1UL CA combination CA_2DL_14A-30A_1UL_BCS0as defined in RP-171026 [1].

# 2	References

[1]	RP-171026, “New WID on LTE Advanced inter-band CA Rel-14 for 2DL/1UL”, RAN#76, Qualcomm

******************************* Start of the TP ******************************************

## 5.x	CA_2DL_14A-30A_1UL_BCS0

### 5.x.1		Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_14A-30A | - | 14 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |

### 5.x.2	Co-existence studies

As shown in Table 5.x.2-1, the UL harmonic frequencies of Band 14 and Band 30 are away from the receive bands of interest in the DL and we can conclude that there is no issue on UL harmonic interference.

Table 5.x.2-1: UL harmonics frequency limits for CA of Band 2, Band 14 and Band 30

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 14 | 788 | 798 | 1576 | 1596 | 2364 | 2394 |
| 30 | 2305 | 2315 | 4610 | 4630 | 6915 | 6945 |

### 5.x.3	ΔTIB,c and ΔRIB,c values

Table 5.x.3-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_14A-30A | 14 | 0.3 |
|  | 30 | 0.3 |

Table 5.x.3-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_14A-30A | 14 | 0 |
|  | 30 | 0 |

### 5.x.4	 MSD

No MSD values need to be defined for this combination.

******************************* End of the TP ******************************************