3GPP TSG-RAN WG4 Meeting #84Bis                                   R4-1711028

Dubrovnik , Croatia, 9 - 13 Oct, 2017

Agenda item:			9.7.2.1

Source:			Samsung

Title:	System Level Simulation Results for FR1

Document for:			Discussion

# Introduction

In RAN4 NR#3 meeting, the updated assumptions for system level simulations are agreed in [1], while the initial discussion has begun in RAN4 #84, with the common understanding that the following discussion may be dependent on the system-level evaluation:

The following aspects are expected to be evaluated from system level simulation. - Number of SS block beams to be detected - Number of cells to be detected Note: A cell is called a detected cell to a UE if one of its beams is detected by that UE. - Number of SS block beams to be detected for a best RSRP cell - Number of SS block beams to be detected for a neighbour cell (any other detected cell than best RSRP cell) - Mobility performance - Interested companies are encouraged to provide results at 20th percentile, 50th percentile and 90th percentile of CDF for number of cells to be detected and number of beams to be detected

In this paper, we would like to provide our updated system level simulation results for 4GHz of FR1 (sub 6GHz) according to the updated simulation assumptions, and accordingly the analysis and views on the related topics.

# Simulation assumptions

- SS beam configuration

  - For 4GHz carrier frequency, beam sweeping is not applied, i.e. the number of SS beam is 1.

- UE Rx beamforming and antenna modelling

  - For 4GHz carrier frequency, UE Rx beamforming and antenna modelling are not applied, i.e. UE use omnidirectional with 0dBi gain.

- UE cell measurements

  - A cell is considered detected if it has an SINR≥-6dB.

- Cell edge UE

  - A cell edge UE is defined as a UE with cell RSRP in the lowest 5th percentile.

- Just like all UE results, 20%-tile, 50%-tile, 90%-tile of CDF for number of detectable cells/beams is collected for cell edge UE.

The detailed simulation assumptions are listed in the tables in the Annex.

# Simulation Results

In this paper, we would provide the simulation results for Urban Macro (UMa) deployment with ISD = 500m for 4GHz carrier frequency.

## Results for all UE

In system level simulation, multiple drops are evaluated. There are totally 21000 points (21000 UE) for drawing the CDF curves.

Below figures show all UE’s CDF of number of cells to be detected and the CDF of SINR for best 4 cells. The table shows the 20th percentile, 50th percentile and 90th percentile of the CDF of detected cells.

| Fig. 3.1-1. CDF of detected cells for all UE | Fig. 3.1-2. CDF of SINR for best 4 cells for all UE |
| --- | --- |

Table 3.1: X%-tile of CDF of detected beams/cells

| Item | 20%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- |
| Detected cells | 1 | 1 | 2 |

Based on the above results, below observation can be made:

- The 20%-tile of CDF is 1 cell, 50%-tile of CDF is 1 cell, and 90%-tile of CDF is 2 cells.

Observation 1: For 4GHz UMa (ISD=500m) scenario, the 20/50/90%-tile of CDF is 1/1/2 detected cells respectively.

## Results for cell edge UE

The cell edge UE is defined as the UE with cell RSRP in the lowest 5th percentile. So, the in this section, there are 1050 points (1050 cell edge UE) for drawing the CDF curves, since all UE number is 21000.

Below figures show the cell edge UE’s CDF of number of cells to be detected and the CDF of SINR for best 4 cells. The table shows the 20th percentile, 50th percentile and 90th percentile of the CDF of detected cells.

| Fig. 3.2-1. CDF of detected cells for cell edge UE | Fig. 3.2-2. CDF of SINR for best 4 cells for cell edge UE |
| --- | --- |

Table 3.2: X%-tile of CDF of detected beams/cells

| Item | 20%-tile | 50%-tile | 90%-tile |
| --- | --- | --- | --- |
| Detected cells | 0 | 1 | 1 |

Based on the above results, below observations can be made:

- For cell edge UE, the 20%-tile of CDF is 0 cell, 50%-tile of CDF is 1 cell, and 90%-tile of CDF is 1 cell..

Observation 2: For 4GHz UMa (ISD=500m) scenario, for cell edge UE, the 20/50/90%-tile of CDF is 0/1/1 detected cells respectively.

# Conclusion

In this contribution, we provide our initial evaluation results for 4GHz carrier frequency according to the system level simulation assumptions. Based on the above evaluation results, the following observations could be obtained.

Observation 1: For 4GHz UMa (ISD=500m) scenario, the 20/50/90%-tile of CDF is 1/1/2 detected cells respectively.

Observation 2: For 4GHz UMa (ISD=500m) scenario, for cell edge UE, the 20/50/90%-tile of CDF is 0/1/1 detected cells respectively.

# Reference

- R4-1709903, “System level simulation assumptions in NR RRM”, Ericsson.

- 3GPP TR 38.802, Study on New Radio Access Technology, Physical Layer Aspects

- 3GPP TR 36.873, Study on 3D channel model for LTE (below 6GHz)

# Annex A: Detailed Simulation Assumptions

Table A-1: System level evaluation assumptions for Urban macro (simplified based on Table A.2.1-1 in TR 36.802)

| Parameters | Urban macro |
| --- | --- |
| Layout | Single layer Macro layer: Hex. Grid, 7 Site 21 Sector |
| Inter-BS distance | 500m (for 4GHz) |
| Carrier frequency | 4GHz |
| Aggregated system bandwidth | No Carrier Aggregation |
| Simulation bandwidth | Downlink 20MHz for below 6GHz |
| Channel model | Below 6GHz: 3D UMa |
| BS Tx power | Below 6GHz: 49dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 49dBm |
| UE Tx power | Not used in DL. |
| BS antenna configurations | See Table A.2.1-4. |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | See Table A.2.1-4 |
| BS receiver noise figure | Not used in DL. |
| UE antenna configuration | See Table A.2.1-4. |
| UE antenna height | Follow TR36.873 |
| UE antenna gain | Follow the modeling of TR36.873 |
| UE receiver noise figure | Below 6GHz: 9dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 100 users per TRP for full buffer traffic Mix of O2I penetration loss models for higher carrier frequency - Option1 - Low loss model – 80% - High-loss model – 20% |

Table A-2: Antenna configurations for below 6GHz (simplified based on Table A.2.1-4 in TR 36.802)

|  | Below 6GHz (4GHz) |
| --- | --- |
| TXRU mapping | Per panel, reuse models in TR 36.897. TXRU mapping to antenna elements for 4GHz: 2D DFT based beam per polarization BS TXRU mapping: all the elements for each polarization are mapped to a single antenna port; UE TXRU mapping: omni-directional antenna |
| TXRU mapping weights | 2D DFT per polarization |
| Number of BS antenna elements across all panels | 4GHz: 64 Tx antenna elements |
| Number of UE antenna elements | 4GHz: 8 Rx antenna elements |
| BS (M,N,P,Mg,Ng) | 4GHz: Dense urban and Urban macro: - (M,N,P,Mg,Ng) = (4,8,2,1,1). |
| BS (dH,dV,dH,g,dV,g) | 4GHz: Dense urban and Urban macro: - Baseline: (dH,dV) = (0.5, 0.5)λ. |
| UE antenna model parameters | One panel (M,N,P,Mg,Ng) = (2,2,2,1,1). dH= dV=0.5λ |
| BS antenna element gain pattern | See Table A-3 |
| UE antenna element gain pattern | See Table A-4 |

Table A-3: 3-Sector BS antenna radiation pattern for above 6GHz (Table A.2.1-6 in TR 38.802)

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |