3GPP TSG-RAN WG4 Meeting 84bis	    R4-1711032

Dubrovnik, Croatia, 9 – 13 Oct., 2017

Agenda Item:	9.7.12.1

Source: 	Samsung

Title: 	Simulation results for SSB based RSRP measurement accuracy

Document for:	Discussion

Introduction

In RAN4 #NR Ad Hoc-2 meeting, link level simulation assumptions for SS block measurement were agreed in [1]. In RAN4#84 and NR Ad Hoc-3 meetings, companies provided simulation results [2]-[6] accordingly.

In this contribution, we provide the simulation results of SSB based RSRP measurement accuracy. Then an analysis on the results is presented.

Simulation Assumptions

The detailed simulation assumptions are listed in Table A.1, and A.2 in Annex A, with some downselections highlighted by red. In addition, L1 sampling interval is 40 ms.

The metric is Delta RSRP=Measured RSRP-Ideal RSRP, where Measured RSRP assumes real channel estimation.

Simulation results

SCS 15 kHz

Figure 3.1-1~Figure 3.1-6 are the CDF curves of Delta RSRP under AWGN, EPA, ETU, TDL-A, TDL-B, and TDL-C channels, respectively for SCS 15 kHz. For each channel, both synchronous and asynchronous networks are considered. Besides, since the CDF curves of Delta RSRP for different SCSs are quite similar, the CDF curves for SCS 30kHz, 120 kHz, 240 kHz will not be presented in later subsections.

| Figure 3.1-1 (a) AWGN, sync, 15 kHz, -6 dB | Figure 3.1-1 (b) AWGN, async, 15 kHz, -6 dB |
| --- | --- |

| Figure 3.1-2 (a) EPA5, sync, 15kHz, -6 dB | Figure 3.1-2 (b) EPA5, async, 15kHz, -6 dB |
| --- | --- |

| Figure 3.1-3 (a) ETU30, sync, 15kHz, -6 dB | Figure 3.1-3 (b) ETU30, async, 15kHz, -6 dB |
| --- | --- |

| Figure 3.1-4 (a) TDL-A, sync, 15kHz, -6 dB | Figure 3.1-4 (b) TDL-A, async, 15kHz, -6 dB |
| --- | --- |

| Figure 3.1-5 (a) TDL-B, sync, 15kHz, -6 dB | Figure 3.1-5 (b) TDL-B, async, 15kHz, -6 dB |
| --- | --- |

| Figure 3.1-6 (a) TDL-C, sync, 15kHz, -6 dB | Figure 3.1-6 (b) TDL-C, async, 15kHz, -6 dB |
| --- | --- |

As seen from the above figures, the accuracy improves significantly when the number of samples increases from 1 to 3, but the difference of accuracies for 3 samples and 5 samples is limited.

Table 3.1-1 summarizes the 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions. Definitely, when the number of samples used for averaging increases, the absolute value of either the 5% point or 95% point decreases. The maximum absolute value of the 5% and 95% points among all channel conditions with SCS 15 kHz is 3.75 dB, as highlighted in the table.

Table 3.1-1: Delta RSRP, SINR -6 dB, 15 kHz

| CDF Channel | Samples | Sync |  |  | Async |  |  | Min 5% | Max 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |  |  |
| AWGN | 1 | -1.10 | 0.21 | 1.26 | -2.21 | 0.51 | 2.31 | -2.21 | 2.31 |
|  | 3 | -0.37 | 0.07 | 0.42 | -0.75 | 0.17 | 0.77 | -0.75 | 0.77 |
|  | 5 | -0.23 | 0.04 | 0.25 | -0.45 | 0.10 | 0.46 | -0.45 | 0.46 |
| EPA5 | 1 | -3.16 | 0.01 | 2.25 | -1.89 | 0.19 | 2.91 | -3.16 | 2.91 |
|  | 3 | -1.03 | 0 | 0.73 | -0.64 | 0.06 | 0.97 | -1.03 | 0.97 |
|  | 5 | -0.64 | 0 | 0.45 | -0.38 | 0.04 | 0.57 | -0.64 | 0.57 |
| ETU30 | 1 | -3.75 | 0.06 | 2.38 | -1.93 | 0.35 | 2.67 | -3.75 | 2.67 |
|  | 3 | -1.26 | 0.02 | 0.80 | -0.64 | 0.12 | 0.88 | -1.26 | 0.88 |
|  | 5 | -0.76 | 0.01 | 0.47 | -0.41 | 0.07 | 0.54 | -0.76 | 0.54 |
| TDL-A | 1 | -3.18 | 0.05 | 2.32 | -1.96 | 0.20 | 2.83 | -3.18 | 2.83 |
|  | 3 | -1.06 | 0.02 | 0.78 | -0.65 | 0.07 | 0.94 | -1.06 | 0..94 |
|  | 5 | -0.64 | 0.01 | 0.46 | -0.40 | 0.04 | 0.58 | -0.64 | 0.58 |
| TDL-B | 1 | -3.17 | 0.03 | 2.35 | -1.95 | 0.20 | 2.86 | -3.17 | 2.86 |
|  | 3 | -1.05 | 0.01 | 0.79 | -0.66 | 0.06 | 0.95 | -1.05 | 0.95 |
|  | 5 | -0.63 | 0.01 | 0.47 | -0.38 | 0.04 | 0.57 | -0.63 | 0.57 |
| TDL-C | 1 | -3.17 | 0.01 | 2.29 | -1.90 | 0.20 | 2.87 | -3.17 | 2.87 |
|  | 3 | -1.07 | 0.01 | 0.76 | -0.62 | 0.07 | 0.97 | -1.07 | 0.97 |
|  | 5 | -0.62 | 0 | 0.46 | -0.38 | 0.04 | 0.57 | -0.62 | 0.57 |

SCS 30 kHz

Table 3.2-1 summarizes the values of 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions with SCS 30 kHz. The maximum absolute value of the 5% and 95% points for SCS 30 kHz is 3.13 dB, and it not too different from 3.75 dB with 15 kHz SCS.

Table 3.2-1: Delta RSRP, SINR -6 dB, 30 kHz

| CDF Channel | Samples | Sync |  |  | Async |  |  | Min 5% | Max 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |  |  |
| AWGN | 1 | -1.10 | 0.21 | 1.25 | -2.22 | 0.51 | 2.33 | -2.22 | 2.33 |
|  | 3 | -0.36 | 0.07 | 0.41 | -0.74 | 0.17 | 0.78 | -0.74 | 0.78 |
|  | 5 | -0.21 | 0.04 | 0.25 | -0.43 | 0.10 | 0.46 | -0.43 | 0.46 |
| EPA5 | 1 | -3.13 | 0.06 | 2.41 | -1.92 | 0.20 | 2.83 | -3.13 | 2.83 |
|  | 3 | -1.04 | 0.02 | 0.80 | -0.65 | 0.07 | 0.93 | -1.04 | 0.93 |
|  | 5 | -0.61 | 0.01 | 0.48 | -0.39 | 0.04 | 0.58 | -0.61 | 0.58 |
| ETU30 | 1 | -2.85 | 0.10 | 2.24 | -2.05 | 0.30 | 2.53 | -2.85 | 2.53 |
|  | 3 | -0.94 | 0.03 | 0.75 | -0.68 | 0.10 | 0.85 | -0.94 | 0.85 |
|  | 5 | -0.56 | 0.02 | 0.45 | -0.42 | 0.06 | 0.50 | -0.56 | 0.50 |
| TDL-A | 1 | -3.04 | 0.07 | 2.22 | -1.99 | 0.22 | 2.85 | -3.04 | 2.85 |
|  | 3 | -1.00 | 0.03 | 0.73 | -0.68 | 0.07 | 0.92 | -1.00 | 0.92 |
|  | 5 | -0.61 | 0.02 | 0.45 | -0.39 | 0.05 | 0.56 | -0.61 | 0.56 |
| TDL-B | 1 | -3.06 | 0.10 | 2.42 | -1.99 | 0.24 | 2.74 | -3.06 | 2.74 |
|  | 3 | -1.03 | 0.03 | 0.81 | -0.66 | 0.08 | 0.92 | -1.03 | 0.92 |
|  | 5 | -0.60 | 0.02 | 0.48 | -0.38 | 0.05 | 0.55 | -0.60 | 0.55 |
| TDL-C | 1 | -3.12 | 0.04 | 2.40 | -1.93 | 0.20 | 2.78 | -3.12 | 2.78 |
|  | 3 | -1.04 | 0.01 | 0.78 | -0.62 | 0.07 | 0.93 | -1.04 | 0.93 |
|  | 5 | -0.61 | 0 | 0.48 | -0.37 | 0.04 | 0.57 | -0.61 | 0.57 |

SCS 120 kHz

Table 3.3-1 summarizes the values of 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions with SCS 120 kHz. The maximum absolute value of the 5% and 95% points with SCS 120 kHz is 3.24 dB.

Table 3.3-1: Delta RSRP, SINR -6 dB, 120 kHz

| CDF Channel | Samples | Sync |  |  | Async |  |  | Min 5% | Max 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |  |  |
| AWGN | 1 | -1.08 | 0.21 | 1.26 | -2.22 | 0.52 | 2.32 | -2.22 | 2.32 |
|  | 3 | -0.37 | 0.07 | 0.42 | -0.76 | 0.18 | 0.78 | -0.76 | 0.78 |
|  | 5 | -0.21 | 0.04 | 0.25 | -0.43 | 0.11 | 0.46 | -0.43 | 0.46 |
| EPA5 | 1 | -2.99 | 0.05 | 2.31 | -2.09 | 0.27 | 2.60 | -2.99 | 2.60 |
|  | 3 | -0.98 | 0.01 | 0.77 | -0.69 | 0.08 | 0.86 | -0.98 | 0.86 |
|  | 5 | -0.60 | 0 | 0.47 | -0.42 | 0.06 | 0.52 | -0.60 | 0.52 |
| TDL-A | 1 | -3.24 | 0 | 2.18 | -1.94 | 0.21 | 2.75 | -3.24 | 2.75 |
|  | 3 | -1.08 | 0 | 0.72 | -0.65 | 0.07 | 0.92 | -1.08 | 0.92 |
|  | 5 | -0.66 | 0 | 0.44 | -0.40 | 0.04 | 0.56 | -0.66 | 0.56 |
| TDL-B | 1 | -3.15 | 0.07 | 2.36 | -1.98 | 0.23 | 2.75 | -3.15 | 2.75 |
|  | 3 | -1.07 | 0.02 | 0.78 | -0.65 | 0.08 | 0.92 | -1.07 | 0.92 |
|  | 5 | -0.63 | 0.01 | 0.47 | -0.40 | 0.04 | 0.56 | -0.63 | 0.56 |
| TDL-C | 1 | -3.15 | 0.04 | 2.28 | -1.90 | 0.21 | 2.79 | -3.15 | 2.79 |
|  | 3 | -1.04 | 0.01 | 0.76 | -0.63 | 0.07 | 0.93 | -1.04 | 0.93 |
|  | 5 | -0.63 | 0.01 | 0.45 | -0.37 | 0.04 | 0.56 | -0.63 | 0.56 |

SCS 240 kHz

Table 3.4-1 summarizes the values of 5%, 50%, 95% points on the CDF curves of Delta RSRP under different channel conditions with SCS 240 kHz. The maximum absolute value of the 5% and 95% points for SCS 240 kHz is 3.75 dB.

Table 3.4-1: Delta RSRP, SINR -6 dB, 240 kHz

| CDF Channel | Samples | Sync |  |  | Async |  |  | Min 5% | Max 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |  |  |
| AWGN | 1 | -1.10 | 0.20 | 1.24 | -2.21 | 0.51 | 2.33 | -2.21 | 2.33 |
|  | 3 | -0.37 | 0.07 | 0.42 | -0.74 | 0.17 | 0.78 | -0.74 | 0.78 |
|  | 5 | -0.22 | 0.04 | 0.25 | -0.42 | 0.10 | 0.46 | -0.42 | 0.46 |
| EPA5 | 1 | -2.83 | 0.04 | 2.05 | -2.06 | 0.29 | 2.48 | -2.83 | 2.48 |
|  | 3 | -0.95 | 0.02 | 0.70 | -0.70 | 0.10 | 0.82 | -0.95 | 0.82 |
|  | 5 | -0.56 | 0.01 | 0.41 | -0.41 | 0.06 | 0.49 | -0.56 | 0.49 |
| TDL-A | 1 | -3.75 | -0.14 | 2.08 | -2.14 | 0.21 | 2.64 | -3.75 | 2.64 |
|  | 3 | -1.27 | -0.05 | 0.68 | -0.72 | 0.07 | 0.88 | -1.27 | 0.88 |
|  | 5 | -0.73 | -0.03 | 0.41 | -0.43 | 0.04 | 0.52 | -0.73 | 0.52 |
| TDL-B | 1 | -2.98 | 0.03 | 2.17 | -2.00 | 0.25 | 2.62 | -2.98 | 2.62 |
|  | 3 | -1.01 | 0.01 | 0.72 | -0.68 | 0.08 | 0.88 | -1.01 | 0.88 |
|  | 5 | -0.62 | 0 | 0.43 | -0.40 | 0.05 | 0.52 | -0.62 | 0.52 |
| TDL-C | 1 | -3.27 | 0.04 | 2.29 | -2.00 | 0.22 | 2.68 | -3.27 | 2.68 |
|  | 3 | -1.08 | 0.01 | 0.75 | -0.67 | 0.08 | 0.89 | -1.08 | 0.89 |
|  | 5 | -0.65 | 0.01 | 0.45 | -0.39 | 0.05 | 0.54 | -0.65 | 0.54 |

Summary across different SCSs

Based on previous simulation results, the maximum absolute value of 5% and 95% points on the CDF curve of Delta RSRP with different SCSs is summarized in Table 3.5-1.

Table 3.5-1 Maximum absolute value of Delta RSRP (5%~95% on CDF), SINR -6 dB

| SCS | 15 kHz | 30 kHz | 120 kHz | 240 kHz |
| --- | --- | --- | --- | --- |
| Maximum absolute value of Delta RSRP | 3.75 dB | 3.13 dB | 3.24 dB | 3.75 dB |

From Table 3.5-1, it can be seen that

Observation 1: The difference of maximum absolute value of Delta RSRP among different SCSs is limited.

Observation 2: The maximum absolute value of Delta RSRP in the range 5%~95% on the CDF curve for all SCSs is 3~4 dB.

Conclusion

In this paper, we provided the SSB based RSRP measurement accuracy simulation results. The results covered different SCSs, channel conditions, network synchronization conditions, and sample numbers. From the simulation results, we obtain the following observations:

Observation 1: The difference of maximum absolute value of Delta RSRP among different SCSs is limited.

Observation 2: The maximum absolute value of Delta RSRP in the range 5%~95% on the CDF curve for all SCSs is 3~4 dB.

Reference

- R4-1706904, Link level simulation assumptions for SS block measurements, Ericsson

- R4-1709301, SS block RSRP link level simulation results, Intel

- R4-1709332, Simulation results for RSRP measurement accuracy of NR, MediaTek

- R4-1709442, Link level evaluation results on SS-RSRP, NTT Docomo

- R4-1709498, Updated simulation results for SS block measurement, CMCC

- R4-1709623, Updated simulation results of SS block measurement accuracy in NR

- Annex A: Simulation Assumptions

Table A.1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | Case A Note2: 2 rx and 4 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx and 4 rx with rx beamforming Note1 | Case A Note2: 2 rx and 4 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx and 4 rx with rx beamforming Note1 |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 1, 3, 5; other number of samples may also be studied upon a need | 1, 3, 5; other numbers of samples may also be studied upon a need |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set, K | 1 Note2 and 4 | 1 Note2 and 4 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - SS block BW | 127 subcarriers for SS, 288 subcarriers for PBCH | 127 subcarriers for SS, 288 subcarriers for PBCH |
| - Actual SS block transmissions | always transmitted | always transmitted |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results NOTE2: prioritized for RAN4#84 |  |  |

Table A.2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | (,)=(112, 1) | (,)=(224, 1) |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR for Case A | dB | K=1: 6 dB K=4: (2, 6,4, -2) dB | K=1: 1 dB K=4: (2, 1, 1, 1) dB and (-1, 5, -3, 2) dB |
| Es/IoT for Case A (calculated from SNR) | dB | N/A | K=1: -5.97 dB K=4: (-2.12, -5.97, -4.45, -1.12) dB and (-5.12, 1.46, -8.45, -0.12) |
| SNR for Case B, including beamforming gain | dB | K=1: 6 dB K=4: (2, 6,4, -2) dB | K=1: 1 dB K=4: (2, 1, 1, 1) dB and (-1, 5, -3, 2) dB |
| Es/IoT for Case B (calculated from SNR), including beamforming gain | dB | N/A | K=1: -5.97 dB K=4: (-2.12, -5.97, -4.45, -1.12) dB and (-5.12, 1.46, -8.45, -0.12) |
| Propagation conditions | - | AWGN, EPA5, ETU30, ETU70, CDL-A/B/C Note, TDL-A/B/C Note |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |