3GPP TSG-RAN WG4 Meeting #84bis	R4-1711046

Dubrovnik, Croatia, 9 - 13 Oct, 2017

Agenda item:	9.4.2.1

Source:	Dish Network, Nokia

Title:	Categorization for single switched 1 UL in NSA NR-LTE

Document for:	Approval

# 1	Introduction

This contribution builds categorization rules on how to define a difficult band/channel combination in NSA LTE-NR.

# 2	Discussion

We begin by reviewing what RAN#77 decided.

RP-1702064:

- The categorization of band combinations is up to RAN4

  - RAN4 is to define a rule for categorization (of band combinations including channel assignments) in order to apply them

    - A possible rule could be based on presence of resulting IMD[2]…[7] (including harmonics as long as the cause includes simultaneous Tx) falling in Rx with the given channel assignment

- RP-1702085:

- Define in RAN4 specs for which band combinations and channel allocations (to the extent specified by RAN4) within that band combination the UE is allowed to indicate that it does not support 2 simultaneous UL tx (this is not implying the granularity of the signalling)

- Signalling to be defined to support the 'red text' from RAN4 part of the single tx discussion (i.e. as in RP-172064):

- UE capability indicates that the UE does not allow 2 simultaneous UL transmission for the RAN4 specified channel allocations in a given band combination. If the network chooses to operate the UE in a way that is not consistent with this capability indication then the UE behavior is not specified and the UE might not meet the performance criteria.

According to RAN#77, simultaneous 2 UL is mandatory for easy and intermediate band/channel combinations and optional for difficult band/channel combinations. This is stated in RP-1702064, but due to editorial difficulties the figures were not added here.

The first thing RAN4 should do is to define a rule that is used to identify which band/channel combinations are difficult.

Intermodulation orders 2-7 (IM2-IM7) have the following absolute value coefficients. Please note that the sign in between the numbers is not minus, it is there just to separate absolute value coefficients.

IM2: 1-1

IM3: 2-1, 1-2

IM4: 3-1, 2-2, 1-3

IM5: 4-1, 3-2, 2-3, 1-4

IM6: 5-1, 4-2, 3-3, 2-4, 1-5

IM7: 6-1, 5-2, 4-3, 3-4, 2-5, 1-6

IM6 and IM7 have not been defined for any LTE 2UL/2DL band combination because the group basically has decided that IMD5 is the highest that can result as MSD above 0dB. For many IMD5 case, the requirement is not specified because it is so small (<2dB).

Observation 1: IMD’s have been defined up to IMD5 in LTE, because IM6 and IM7 are seen negligible for inter-band CA.

We did a deep dive into currently specified MSD numbers. Please note that one column is added that characterizes IM coefficients:

Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD | Type of IMD (victim band first) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |
| CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 | 2-1 |
|  |  |  |  |  |  | 25.75 |  |  | 2-1 |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A |  | N/A | N/A |
| CA_1A-8A | 1 | 1965 | 5 | 25 | 2155 | 6 | FDD | IMD4 | 2-2 |
|  |  |  |  |  |  | 8.75 |  |  | 2-2 |
|  | 8 | 887.5 | 5 | 25 | 932.5 | N/A |  | N/A | N/A |
| CA_2A-4A | 2 | 1860 | 20 | 502 | 1940 | 5 | FDD | IMD3 | 2-1 |
|  |  |  |  |  |  | 7.75 |  |  | 2-1 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A |  | N/A | N-A |
| CA_2A-4A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | FDD | N/A | N-A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 |  | IMD5 | 2-3 |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 | 2-2 |
|  |  |  |  |  |  | 6.75 |  |  | 2-2 |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  | N/A | N/A |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 24 |  | IMD24 | 1-1 |
| CA_3A-7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 |  | IMD4 | 1-3 |
|  |  |  |  |  |  | 15.75 |  |  | 1-3 |
| CA_3A-8A | 3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 |  | IMD44 | 3-1 |
| CA_3A-8A | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 | 1-4 |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A |  | N/A | N/A |
| CA_3A-19A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 | 2-2 |
|  |  |  |  |  |  | 6.75 |  |  | 2-2 |
|  | 19 | 838 | 5 | 25 | 883 | N/A |  | N/A | N/A |
| CA_3A-19A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A | N/A |
|  | 19 | 838 | 5 | 25 | 883 | 27 |  | IMD24 | 1-1 |
| CA_3A-20A | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 | 2-2 |
|  |  |  |  |  |  | 6.75 |  |  | 2-2 |
|  | 20 | 840 | 5 | 25 | 799 | N/A |  | N/A | N/A |
| CA_3A-20A | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 |  | IMD4 | 3-1 |
|  |  |  |  |  |  | 11.75 |  |  | 3-1 |
| CA_3A-26A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 | 2-2 |
|  |  |  |  |  |  | 6.75 |  |  | 2-2 |
|  | 26 | 838 | 5 | 25 | 883 | N/A |  | N/A | N/A |
| CA_3A-26A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A | N/A |
|  | 26 | 838 | 5 | 25 | 883 | 26 |  | IMD24 | 1-1 |
| CA_3A-41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 | 2-2 |
|  |  |  |  |  |  | 10.95 |  |  | 2-2 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | TDD | N/A | N/A |
| CA_3A-42A | 3 | 1740 | 5 | 25 | 1835 | 29.8 | FDD | IMD24 | 1-1 |
|  |  |  |  |  |  | 32.55 |  |  | 1-1 |
|  | 42 | 3575 | 5 | 25 | 3575 | N/A | TDD | N/A | N/A |
| CA_3A-42A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 | 3-1 |
|  |  |  |  |  |  | 10.75 |  |  | 3-1 |
|  | 42 | 3435 | 5 | 25 | 3435 | N/A | TDD | N/A | N/A |
| CA_4A-5A | 4 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 26 |  | IMD24 | 1-1 |
| CA_4A-7A | 4 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A | N/A |
|  | 7 | 2535 | 5 | 25 | 2655 | 15 |  | IMD4 | 1-3 |
|  |  |  |  |  |  | 17.75 |  |  | 1-3 |
| CA_5A-7A | 5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 | 2-1 |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A |  | N/A | N/A |
| CA_7A-20A | 7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 12 |  | IMD34 | 2-1 |
|  |  |  |  |  |  | 14.75 |  |  | 2-1 |
| CA_8A-41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD3 | 2-1 |
|  | 41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A | N/A |
| CA_8A-41A | 8 | 900 | 5 | 25 | 945 | 5.7 | FDD | IMD5 | 4-1 |
|  | 41 | 2655 | 10 | 50 | 2655 | N/A | TDD | N/A | N/A |
| CA_19A-42A3 | 19 | N/A | N/A | N/A | N/A | N/A | FDD | N/A | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A | N/A |
| CA_21A-28A | 21 | 1450.4 | 5 | 25 | 1498.4 | [2.5] | FDD | IMD5 | 1-4 |
|  | 28 | 735.5 | 5 | 25 | 790.5 | N/A | TDD | N/A | N/A |
| CA_21A-42A3 | 21 | N/A | N/A | N/A | N/A | N/A | FDD | N/A | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A | N/A |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 | 4-1 |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | N/A | TDD | N/A | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |

The results can be collected as follows.

| IM order/type | 1-1 | 2-1 | 1-2 | 3-1 | 1-3 | 2-2 | 3-2 | 2-3 | 4-1 | 1-4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IM2 [dB] | 24, 27, 26, 29.8, 26 |  |  |  |  |  |  |  |  |  |
| IM3 [dB] |  | 23, 5*, 12, 12, 12.1 |  |  |  |  |  |  |  |  |
| IM4 [dB] |  |  |  | 8, 9, 8 | 13, 15 | 6, 4, 4, 4, 8.2 |  |  |  |  |
| IM5 [dB] |  |  |  |  |  |  |  | 5 | 5.7, [5.5] | 6.4, [2.5] |
| Worst [dB] | 29.8 | 23 |  | 9 | 15 | 8.2 |  | 5 | 5.7 | 6.4 |
| Average [dB] | 26.6 | 12.8* |  | 8.3 | 14 | 5.2 |  | 5 | 5.6 | 4.5 |

* indicates that for 5dB IM3 the correction factor significantly impacted the MSD because the IMD only partially hits in this case.

IM2 and IM5 are quite constant while IM3 and IM4 vary significantly. There are many reasons why the variation amongst IMD3 and IMD4 is so big. One of the main explanations are RF architecture differences due to different kind of combinations; L-H combos are done using a diplexer while some L-L and H-H combos are done using a quadplexer. PA mixing impact varies a lot based on type of the IMD. In general cases like 2-2, 2-3, and 3-2 are easier from PA mixing perspective than those with 1-X or X-1. Type 1-3 IM4 combos in table above are B3+B7 and B4+B7. In these combinations, the most dominating component in MSD definition was quadplexer.

Observation 2: Out of currently specified LTE inter-band CA MSD, IM2 and IM5 are quite constant while IM3 and IM4 vary significantly.

Observation 3: RF architecture differences due to different type of CA combinations (L-H or H-H for instance) explain much of the differences

We recognize that it might not be very fruitful to start discussing whether RAN4 should redo all existing MSD analyses using more realistic component assumptions. There is no time for that. We need to find categorization rule using current numbers as references. However, we encourage companies to provide updated assumptions for new cases that will be needed for NR specification.

Observation 4: Even updating some of the generic assumptions would result as lower MSD numbers, time is too limited to redo old analysis

One practical issue that was not discussed very deeply when UL inter-band CA MSD process was developed is the fact that in 2UL REFSENS test both uplinks are transmitting maximum UL allocation with 3dB reduced power and still the MSD is calculated from REFSENS when, in our understanding, the UE cannot be close to cell edge. Hence assessing the impact of MSD when it is calculated from cell edge value is difficult.

Observation 5: Current method to define MSD with 3dB reduced TX power per CC and maximum UL allocation is not fully relevant because it does not represent cell edge scenario.

Next we consider which IM’s are difficult. We underline that in this context word “difficult” means just high MSD, not that it would be more difficult to achieve specified MSD when the number is high. We have the following view:

- IM2 is justifiably difficult as all numbers are above 20dB

- IM3 can be either intermediate or difficult. Some band combinations which are close to each other in frequency such as bands 1 and 3 are difficult

- IM4 is not difficult either, besides maybe in 1-3 case for H-H combos or for non-existing L-L combos. However, because the dominating component is quadplexer, we genuinely think that using current technology it is quite easy to reach MSD’s similar to other IM4 types hence making it intermediate

- IM5 is always easy

RAN4 could use coarse classification for the frequency separation for IM3. Basically, cases when both bands are <1GHz or within 1.695GHz<2.2GHz could be considered difficult for IM3.

Proposal 1: IM2 is difficult and IM3 for band combinations when both bands are <1GHz or 1.695GHz<2.2GHz is difficult. Other IM’s are either intermediate or easy.

Much more difficult issue that to find suitable rule for the difficult band combinations is to define how to distinguish not easy/intermediate channel combinations from the difficult bands combinations without doing overly extensive analysis for every possible frequency combination.

We pick a few options. There are likely many other options as well not listed here:

- Define only difficult band combinations and assume scheduler to schedule simultaneous 2TX when channel allocations are such that difficult IMD does not hit on top of own DL channel.

- Define only difficult channel combinations

- Define difficult band combinations and difficult channel combinations

| Option | Pro’s | Con’s |
| --- | --- | --- |
| 1 | -Would be easy from RAN4 perspective -Would be extendable to be even transmission BW specific depending on scheduler -Could be based on simple formula | -Definition would not be rigorous |
| 2 | -Very rigorously defined | -Huge amount of work in RAN4 |
| 3 | -Very rigorously defined | -Huge amount of work in RAN4 |

The capability in option 1 could be understood as follows.

-For every channel combination with IM2/IM3 when both bands are <1GHz or 1.695GHz<2.2GHz, UE does not support concurrent 2 UL transmission if UE has indicated it by (single) capability bit for this band combination”.

-IM2/ IM3 when boths bands are <1GHz or 1.695GHz<2.2GHz formulas are defined to avoid both partial and direct hit. We think that it would be too complex to consider some partial hit IM2/close-proximity IM3’s as easy ones even in practice it would be so.

Proposal2: UE is allowed to indicate that it does not support concurrent 2 UL transmission for LTE-NR for band combinations that have MSD due to IM2 and/or IM3 when both bands are <1GHz or 1.695GHz<2.2GHz. UE shall meet REFSENS requirements and BS scheduler is assumed to schedule concurrent 2 UL transmission for all channel combinations within that band combination that do not have IM2 and/or close-proximity IM3.

MSD should be defined similar to LTE, i.e worst case MSD is defined for each band combination, up to IMD5. For band combinations without IMD issues, REFSENS is verified. For band combinations with not difficult IMD’s, the REFSENS is verified at least in worst case spot like in LTE. For band combinations with IM2 and/or IM3 when both bands are <1GHz or 1.695GHz<2.2GHz MSD is defined but not verified.

Proposal3: MSD shall be defined similar to LTE, i.e. worst case MSD is defined for each band combination up to IMD5.

- REFSENS with 0dB MSD is verified for band combinations without IMD’s

- REFSENS with at least non-zero MSD is verified for band combinations having easy/intermediate IMD’s. It’s up to RAN5 to define testing for 0dB MSD channel combinations

- REFSENS with MSD is defined for all band combinations with difficult IMD’s but verified only with UE’s supporting simultaneous 2 UL transmission for that band combination. It’s up to RAN5 to define testing for 0dB MSD channel combinations

It’s very important to know when REFSENS degradation i.e non-zero MSD occurs. Assuming that also partial hit would be counted as hit, it is straightforward to build formulas based on the following parameters:

- UL/DL CC center frequencies

- CC bandwidths

- IM order

Having formulas in TS in questionable because they would be for informative purposes only. We suggest RAN4 to consider if IM formulas should be defined, and if yes, whether to have those in TR or TS.

Proposal4: RAN4 should consider if IM formulas should be defined, and if yes, whether to have those in TR or TS.

# 3	Conclusions

Categorization rules for single switched UL in LTE-NR were considered with the following proposals.

Proposal 1: IM2 is difficult and IM3 for band combinations when both bands are <1GHz or 1.695GHz<2.2GHz is difficult. Other IM’s are either intermediate or easy.

Proposal2: UE is allowed to indicate that it does not support concurrent 2 UL transmission for LTE-NR for band combinations that have MSD due to IM2 and/or IM3 when both bands are <1GHz or 1.695GHz<2.2GHz. UE shall meet REFSENS requirements and BS scheduler is assumed to schedule concurrent 2 UL transmission for all channel combinations within that band combination that do not have IM2 and/or close-proximity IM3.

Proposal3: MSD shall be defined similar to LTE, i.e. worst case MSD is defined for each band combination up to IMD5.

- REFSENS with 0dB MSD is verified for band combinations without IMD’s

- REFSENS with at least non-zero MSD is verified for band combinations having easy/intermediate IMD’s. It’s up to RAN5 to define testing for 0dB MSD channel combinations

- REFSENS with MSD is defined for all band combinations with difficult IMD’s but verified only with UE’s supporting simultaneous 2 UL transmission for that band combination. It’s up to RAN5 to define testing for 0dB MSD channel combinations

Proposal4: RAN4 should consider if IM formulas should be defined, and if yes, whether to have those in TR or TS.

# 4	References