3GPP TSG-RAN WG4 Meeting 84-bis	R4-1711096

# Dubrovnik, Croatia, 9-13 October 2017

Source:	Ericsson

Title:	TP to TS 38.133 v0.2.0; UE timing advance adjustment accuracy

Agenda item:	9.7.6.1

Document for:	Approval

## 1 Background

A Way Forward was approved in [1]. The WF states two options for UE Timing Advance adjustment accuracy scaling:

- Option 1: with UL SCS

- Option 2: with UL BW and UL SCS

This is a relative requirement, so option 1 will work.

When we scale UE Timing Advance adjustment accuracy, using Ts = 1/(15000*248) based on LTE, to maintain UE demodulation performance and UL capacity for larger SCS, we get:

| Sub Carrier Spacing, SCS kHz |  |  |  |
| --- | --- | --- | --- |
| SCS | 15 | 30 | 60 | 120 |
| UE Timing Advance adjustment accuracy | ±4 Ts | ±2 Ts | ±1 Ts | ±0.5 Ts |
| TA command step size | 16 Ts | 8 Ts | 4 Ts | 2 Ts |

However, in TS 38.211 RAN1 has specified a new Time unit, for NR, they are expressed in terms of NR basic timing parameter Tc. According to the latest version of TS 38.211:

where  Hz and . The constant  where ,  and .

In summary 1 Ts = 64 Tc where 1 Ts = 32.55 ns (LTE basic time unit) e.g. Te = ±768 Tc = ±12 Ts.

If we use the NR Tc unit we get:

| Sub Carrier Spacing, SCS kHz |  |  |  |
| --- | --- | --- | --- |
| SCS | 15 | 30 | 60 | 120 |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±128 Tc | ±64 Tc | ±32 Tc |
| TA command step size | 1024 Tc | 512 Tc | 256 Tc | 128 Ts |

## 2 Summary

We define UE Timing Advance adjustment accuracy in NR Ts as:

| Sub Carrier Spacing, SCS kHz |  |  |  |
| --- | --- | --- | --- |
| SCS | 15 | 30 | 60 | 120 |
| UE Timing Advance adjustment accuracy | ±256 Ts | ±128 Ts | ±64 Ts | ±32 Ts |
| TA command step size | 1024 Ts | 512 Ts | 256 Ts | 128 Ts |

## 3 Proposal

It is proposed that the attached text proposal is included in TS 38.133.

## 4 References

[1] R4-1709902, Way forward on UE transmit timing, Huawei, HiSilicon

================================== start of TP =======================================

## 7.3	Timing advance

### 7.3.1	Introduction

The timing advance is initiated from NG-RAN with MAC message that implies and adjustment of the timing advance,

### 7.3.2	Requirements

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 7.3.2-1 seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command is expressed in multiples of [FFS] as stated in TA command step size requirement in Table 7.3.2-1 and is relative to the current uplink timing.

Table 7.3.2-1: UE Timing Advance adjustment accuracy and TA command step size

| Sub Carrier Spacing, SCS kHz | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±128 Tc | ±64 Tc | ±32 Tc |
| TA command step size | [FFS] | [FFS] | [FFS] | [FFS] |

================================== end of TP =========================================