3GPP TSG-RAN WG4 Meeting #84Bis	R4-1711153

Dubrovnik, Croatia, 9 – 13 October 2017

Agenda Item:	9.5.3.3.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on mmWave NR BS ACLR

Document for:	Approval

# 1.	Introduction

mmWave NR BS ACLR requirements were further discussed in RAN4-NR#2, and the way forward was agreed in [1], where certain aspects are FFS. These FFS aspects were further discussed in RAN4#84 [2-4] and RAN4-NR#3 [5, 6], but no conclusion could be reached.

This contribution provides a proposal to conclude on the FFS aspects to specify the mmWave NR BS ACLR requirements in the RAN4 specifications.

# 2.	Discussion

The following aspect for NR BS ACLR requirements were FFS in the agreed way forward [1]:

- Whether to specify the absolute limit of the ACLR requirements for each NR BS class.

It was proposed in [2] to specify the absolute limit of the ACLR requirements as -20dBm/MHz for all mmWave NR BS classes. But it was commented that the proposed value would be less than the spurious emission limit, and the absolute limit of the ACLR requirements should guarantee no degradation. While it was proposed in [3-6] to specify the absolute limit of the ACLR requirements to be equal to the spurious emission limit.

The proposal in [3-6] is agreeable based on the following reasons:

1)	In general, the mmWave NR BS should be able to achieve a lower unwanted emission limit at frequency range that is further away from the transmission bandwidth, resulting from both the lower transmission unwanted emission level and the higher achievable transmit filter rejection at the further away frequency range. Therefore, it should not be expected that the absolute emission limit at the adjacent channel is lower than the spurious emission limit.

2)	In terms of degradation to the coexisting system, the proposal in [3-6] guarantees that the mmWave NR BS could at least provide the same protection to the adjacent channel system as those coexisting system that are further away in frequency from the BS transmission bandwidth, which are protected by the BS spurious emission limit.

In view of the above, it is proposed to agree the proposal in [3-6] and specify the absolute limit of the ACLR requirements to be equal to the spurious emission limit for mmWave NR BS.

# 3.	Conclusion and proposals

This contribution has provided a proposal to conclude on the FFS aspects to specify the mmWave NR BS ACLR requirements in the RAN4 specifications.

Proposal:

To specify specify the absolute limit of the ACLR requirements to be equal to the spurious emission limit for mmWave NR BS.

# References

[1]	R4-1706954, “WF on BS ACLR and ACS for mmWave”, Nokia, Alcatel-Lucent Shanghai Bell.

[2]	R4-1707151, “Proposals on mmWave NR BS ACLR and ACS”, Nokia, Nokia Shanghai Bell.

[3]	R4-1707961, “	Discussion on ACLR and ACS requirement for mmWave NR BS”, 	ZTE Corporation.

[4]	R4-1708123, “On absolute level ACLR for mm-wave bands”, Ericsson.

[5]	R4-1709704, “Proposals on mmWave 6GHz NR BS ACLR”, Nokia, Nokia Shanghai Bell.

[6]	R4-1709844, “Proposals on absolute level ACLR for mm-wave NR base station”, Ericsson.