3GPP TSG-RAN WG4 Meeting #84bis 											      R4- 17114113

Dubrovnik, Crotia, 9 – 13 Oct, 2017

Source: 	Huawei, HiSilicon

Title: 	TP for TR 36.758: Specific issue for TDD operation in 3300-3400MHz band – UE sensitivity

Agenda Item:	8.12.1

Document for:	Approval

# Introduction

During last 3GPP TSG RAN Meeting #76 in West Palm Beach, USA June 5 - 8, 2017, a new WI about the study of the TDD in the C-band (3300-3400 MHz) has been approved.

This contribution provides the TP for the TR 36.758, about the UE sensitivity.

# Discussion

## Background

The text below is to define the UE sensitivity in the 3300-3400MHz band for Africa countries.

# Text proposal for TR

<Unchanged sections omitted>

# 8	Study of E-UTRA specific issues

## 8.2    UE sensitivity for E-UTRA

The reference sensitivity is specified in Table 8.6-1 for Band 52 for general case:

Table 8.6-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 52 |  |  | -99 | -96 | -94.2 | -93 | TDD |

For specific case, UE(s) equipped with 4 antenna ports, the following additional requirements described in the table 8.6-2 is applied:

Table 8.6-2: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 52 |  |  | -101.2 | -98.2 | -96.4 | -95.2 | TDD |

<Unchanged sections omitted>

# Conclusion

It is proposed:

Proposal: Add the text as proposed in the paragraph “Text proposal for TR” in the TR [4]

# References

[1] RP-171506 “New LTE band for 3.3-3.4 GHz for Africa”, Huawei, HiSilicon

[2] R4-1708452 “Regulation background for TDD C-Band (3300-3400MHz) for Africa”; Huawei, HiSilicon

[3] R4-1709188 “LS to African Telecommunications Union about Regulation background to protect incumbent users in the band: 3.3-3.4GHz”, 3GPP RAN WG4

[4] TR 36.756 “3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; TDD operating in 3300-3400MHz band in Africa for LTE (Release 15)”

[5] R4-1710124 “Response to the LS R4-1709188 from 3GPP to ATU about regulations background to protect incumbent users in the band”, African Telecommunications Union