3GPP TSG RAN WG4 Meeting #84bis	R4-1711454

Dubrovnik, Croatia, October 9 – 13, 2017

Agenda Item:	5.4.3

Source: 	Ericsson

Title:	Measurement accuracy of non-BL/CE in CE Mode B

Document for:	Discussion

# Introduction

At RAN4#83 improvement of measurement accuracy of non-BL UEs operating in CE Mode B was discussed. In a WF [1] it was agreed that companies would look into whether the measurement accuracy can be improved by having the non-BL UE using both receiver branches when operating in CE Mode B. Several companies provided input to RAN4#84 in Berlin, but the meeting arrangement was too tight to allow discussions about the results.

In this contribution we are compiling the results provided by companies at RAN4#84, and proposing tightened requirements for non-BL UEs operating in CE Mode B.

For justifications on why it would be desirable to improve the measurement accuracy, see [1].

Since RSRQ measurement accuracy requirements for BL UE in CE Mode B are to be agreed at RAN4#84bis, it is suggested that non-BL/CE RSRQ measurement accuracy requirements are revisited at RAN4#85.

# Discussion

## Simulation results

Simulation results were provided by Huawei [3], Intel [4], and Ericsson [5], and are summarized in the tables below. The columns maximum absolute deviation (Max abs deviation) refers to the largest deviation of the 5th and 95th percentiles to the theoretical value.

Table 1: RSRP Huawei results

| Ês/Iot | Channel | 1Rx [dB] |  | 2Rx [dB] |  | Max abs deviation [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5th | 95th | 5th | 95th | 1Rx | 2Rx |
| -15 | AWGN | -7.46 | 3.17 | -5.26 | 2.45 | 7.46 | 5.26 |
|  | ETU 1Hz | -7.15 | 3.44 | -5.42 | 2.47 | 7.15 | 5.42 |
|  | EPA 1Hz | -5.89 | 4.47 | -4.68 | 2.83 | 5.89 | 4.68 |
|  |  |  |  |  |  | 7.46 | 5.42 |
| -12 | AWGN | -3.35 | 1.99 | -2.32 | 1.51 | 3.35 | 2.32 |
|  | ETU 1Hz | -4.15 | 1.92 | -2.98 | 1.32 | 4.15 | 2.98 |
|  | EPA 1Hz | -4.19 | 2.68 | -2.46 | 1.63 | 4.19 | 2.46 |
|  |  |  |  |  |  | 4.19 | 2.98 |
| -6 | AWGN | -0.83 | 0.67 | -0.56 | 0.52 | 0.83 | 0.56 |
|  | ETU 1Hz | -1.21 | 0.53 | -0.93 | 0.31 | 1.21 | 0.93 |
|  | EPA 1Hz | -1.08 | 0.87 | -0.61 | 0.52 | 1.08 | 0.61 |
|  |  |  |  |  |  | 1.21 | 0.93 |

Table 2: RSRP Intel results

| Ês/Iot | Channel | 1Rx [dB] |  | 2Rx [dB] |  | Max abs deviation [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5th | 95th | 5th | 95th | 1Rx | 2Rx |
| -15 to -12 | AWGN |  |  |  |  |  | 2.6 |
|  | ETU 1Hz |  |  |  |  |  | 3.5 |
|  | EPA 1Hz |  |  |  |  |  | 4.2 |
|  |  |  |  |  |  |  | 4.2 |
| -12 to -6 | AWGN |  |  |  |  |  | 1.2 |
|  | ETU 1Hz |  |  |  |  |  | 1.6 |
|  | EPA 1Hz |  |  |  |  |  | 2.1 |
|  |  |  |  |  |  |  | 2.1 |

Table 3: RSRP Ericsson results

| Ês/Iot | Channel | 1Rx [dB] |  | 2Rx [dB] |  | Max abs deviation [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5th | 95th | 5th | 95th | 1Rx | 2Rx |
| -15 to -12 | AWGN |  |  |  |  | 2.2 | 2.4 |
|  | ETU 1Hz |  |  |  |  | 4.1 | 3.1 |
|  | EPA 1Hz |  |  |  |  | 3.4 | 3.9 |
|  |  |  |  |  |  | 4.1 | 3.9 |
| -12 to -6 | AWGN |  |  |  |  | 1.3 | 1.5 |
|  | ETU 1Hz |  |  |  |  | 5.5 | 3.3 |
|  | EPA 1Hz |  |  |  |  | 4.2 | 3.8 |
|  |  |  |  |  |  | 5.5 | 3.8 |

Table 4: RSRQ Huawei results

| Ês/Iot | Channel | 1Rx [dB] |  | 2Rx [dB] |  | Max abs deviation [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5th | 95th | 5th | 95th | 1Rx | 2Rx |
| -15 | AWGN | -7.71 | 3.11 | -5.38 | 2.47 | 7.71 | 5.38 |
|  | ETU 1Hz | -7.31 | 3.43 | -5.34 | 2.49 | 7.31 | 5.34 |
|  | EPA 1Hz | -6.05 | 4.46 | -5.12 | 2.8 | 6.05 | 5.12 |
|  |  |  |  |  |  | 7.71 | 5.38 |
| -12 | AWGN | -3.38 | 1.98 | -2.41 | 1.46 | 3.38 | 2.41 |
|  | ETU 1Hz | -4.06 | 1.96 | -3.08 | 1.35 | 4.06 | 3.08 |
|  | EPA 1Hz | -4.58 | 2.74 | -2.57 | 1.65 | 4.58 | 2.57 |
|  |  |  |  |  |  | 4.58 | 3.08 |
| -6 | AWGN | -0.84 | 0.66 | -0.54 | 0.49 | 0.84 | 0.54 |
|  | ETU 1Hz | -1.27 | 0.52 | -1.01 | 0.31 | 1.27 | 1.01 |
|  | EPA 1Hz | -1.18 | 0.93 | -0.68 | 0.56 | 1.18 | 0.68 |
|  |  |  |  |  |  | 1.27 | 1.01 |

Table 5: RSRQ Ericsson results

| Ês/Iot | Channel | 1Rx [dB] |  | 2Rx [dB] |  | Max abs deviation [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5th | 95th | 5th | 95th | 1Rx | 2Rx |
| -15 to -12 | AWGN |  |  |  |  | 2.2 | 2.3 |
|  | ETU 1Hz |  |  |  |  | 4.1 | 3.0 |
|  | EPA 1Hz |  |  |  |  | 3.2 | 3.7 |
|  |  |  |  |  |  | 4.1 | 3.7 |
| -12 to -6 | AWGN |  |  |  |  | 1.3 | 1.5 |
|  | ETU 1Hz |  |  |  |  | 5.0 | 3.1 |
|  | EPA 1Hz |  |  |  |  | 3.8 | 3.5 |
|  |  |  |  |  |  | 5.0 | 3.5 |

## Proposed RSRP requirements for non-BL/CE UE in CE Mode B

The existing RSRP measurement accuracy requirements for BL UE in CE Mode B are as follows below.

Table 9.1.21.3-1: RSRP Intra frequency absolute accuracy for UE category M1 with CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 8 | 11 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
| 7 | 10 | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 10 | 13 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| 9 | 12 | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Studying the simulation results provided by the companies, one can see that for 1 Rx the result provided by Huawei (Table 1) is very close to the existing requirement in the range -15 ≤ Ês/Iot < -12dB, however there is a gain in accuracy of 1dB when using 2 Rx branches. The results provided for 2 Rx by Intel (Table 2) and Ericsson (Table 3) are on par with each other and have more margin to the existing requirements.

For the range -12 ≤ Ês/Iot < -6dB, the result provided by Ericsson (Table 3) has the least margin to the requirements both for 1 Rx and 2 Rx. The gain in accuracy when using 2 Rx is about 1.7dB, suggesting that the RSRP measurement accuracy can be tightened by at least 1dB.

Based on the simulation results, we therefore propose to tighten the RSRP intra-frequency absolute accuracy requirements over the range -15 ≤ Ês/Iot < -6dB by 1dB, whereby a tentative requirement may look as follows.

Table 9.X.Y.Z-1: RSRP Intra frequency absolute accuracy for non-BL/CE UE operating in CE mode B for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 7 | 10 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
| 6 | 9 | -12 dB | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 9 | 12 | -15≤Ês/Iot≤-12 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| 8 | 11 | -12 dB |  |  |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Proposal 1: In CE Mode B, a non-BL/CE UE shall fulfil an RSRP intra-frequency absolute requirement that is 1dB tighter then the corresponding requirement for a BL/CE UE.

# Summary and Conclusion

In this contribution we have studied the simulation results provided by companies at RAN4#84, and see an opportunity of tightening the RSRP intra-frequency accuracy requirement for a non-BL/CE UE that operates in CE Mode B, compared to the corresponding requirements for a BL/CE UE operating under same conditions. We make the following proposal.

Proposal 1: In CE Mode B, a non-BL/CE UE shall fulfil an RSRP intra-frequency absolute requirement that is 1dB tighter then the corresponding requirement for a BL/CE UE.

Remaining RSRP requirements under CE Mode B can be derived from this value in similar fashion to how the new requirements for efeMTC were derived.

Due to that RSRQ measurement accuracy requirements for BL/CE UE in CE Mode B have not been agreed yet, we suggest that RSRQ is revisited at RAN4#85.

# References

- R4-1706185 “WF on RSRP and RSRQ Requirements for Rel-14 non-BL/CE UE”, Ericsson, Intel, Huawei, HiSilicon

- 36.133

- R4-1708262 “Discussion on RSRP and RSRQ requirements for non-BL/CE UE in R14”, Huawei, HiSilicon

- R4-1707351 “Discussion on the RSRP accuracy for R14 non-BL/CE UE”, Intel

- R4-1708319 “Measurement accuracy of non-BL/CE in CE Mode B”, Ericsson