3GPP TSG-RAN WG4 Meeting #84bis	R4-1711481

Dubrovnik, Croatia, 9th-13th October 2017

Source: 	Ericsson

Title:  	Inband receiver blocking for mmWave NR BS with 55dBm CPE FWA deployment

Agenda Item:	9.5.4.3.2

Document for:	Discussion

# Introduction

During RAN4#84, we have agreed on a WF related to 55dBm CPE in mmWave spectrum [6]. RAN4 has agreed on simulation assumptions in [7]. Based on these agreed simulation assumptions, we provide our simulation results for FWA scenario in a companion contribution [8].

BS receiver blocking has been studied in RAN4 in the Rel-14 NR SI phase. In one of companion contributions [3], we provided a methodology based on joint probability of receiver blocking where probability of both wanted signal and blocking signal above a level are considered.

In this contribution, we provide simulation results for receiver blocking in NR BS when 55dBm CPE deployment is considered.

# UMa assumptions

We use the same assumptions as agreed in TR 38.803 [1]. We mention several them below:

- ISD of 300m

- UL transmission BW of 200MHz

- Carrier frequency: 30 GHz

- For non-collocated case,

  - The correlation of shadowing fading between base stations is 0.5.

  - LOS/NLOS is assigned to each link independently with respect to different base stations.

- We assumed following antenna parameters:

  - BS antenna array: 16x8

  - CPE antenna array: 8x4

# Blocking results when array gain is considered

## Collocated UMa deployment

In this section, we provide the simulation results when wanted signal and blocker signals are measured after the array gain is included.

On the left of Figure 1, we show the cumulative distribution of wanted and blocking signals when resource utilization is as high as 90% in collocated UMa deployment. Based on the methodology proposed in ‘Joint probability for receiver blocking in NR BS’, R4-1705094, the blocking levels for joint probability P1, ranging from stringent requirement 1e-4 to less stringent requirement 0.01 are shown on the right side of Figure 1. Moreover, as the blocking level is highly depending on the resource utilization in the BS, we show also how the blocking level increases when resource utilization increases.

Figure 1	On the left: Distribution of wanted and blocking signals in collocated Uma deployment. One the right: blocking levels and resource utilization for different joint probability requirements

| Wanted signal [dBm] | -75 |
| --- | --- |
| Blocking level [dBm] | [-70 -64] |

## Non-collocated UMa deployment

Similar, we show in Figure 2, on the left side, the cumulative distribution of wanted and blocking signals when resource utilization is as high as 90% in non-collocated UMa deployment. On the right side of Figure 2, blocking level vs. resource utilization in BS from more stringent requirement 1e-4 to less stringent requirement 0.01 are given.

Figure 2	On the left: Distribution of wanted and blocking signals in non-collocated with a grid-shift 100 m Uma deployment. One the right: blocking levels and resource utilization for different joint probability requirements

| Wanted signal [dBm] | -75 |
| --- | --- |
| Blocking level [dBm] | [-62 -57] |

# Summary

It is worth noting here that the blocking levels in this contribution are considered for conducted levels.

Observation: The results in this contribution are strictly valid for conducted blocking levels.

The actual blocking level in OTA will depend on the method that we use for calculation of OTA level. It is important to consider whether average gain can be used to calculate the OTA level or we need to consider maximum gain for OTA blocking levels.

The “conducted” blocking levels are very similar when array gain is considered, when we compare the results in this contribution to our previous results in [5].

# References

TR 38.803

R4-1700224, BS OTA sensitivity for mm-wave frequencies, Ericsson

R4-1706811, The importance for considering joint probability for receiver blocking in NR BS, Ericsson

R4-1708145, Consideration of joint probability for wanted and interference signals in mmWave NR BS receiver blocking investigation

R4-1709859, Conducted blocker levels for NR BS In-band blocking, Ericsson

R4-1708853, WF on Requirement for 55dBm EIRP Transportable Stations, Verizon, Ericsson, Qualcomm, Qorvo, Huawei, ZTE, Nokia

R4-1709987, Simulation assumptions of Co-existence study for 55dBm CPE in mmWave spectrum, Ericsson, Nokia.

R4-1711483, Simulations Results on Coexistence Studies in Urban Macro Scenario for 55dBm CPE, Ericsson