3GPP TSG-RAN WG4 Meeting #84bis	R4-1711488

Dubrovnik, Croatia, 9th-13th October 2017

Source: 	Ericsson

Title:  	TP to TR 38.817-02 v0.2.0: Methodology for mmWave NR BS receiver blocking investigation

Agenda Item:	9.5.4.3.2

Document for:	Approval

# Background

A skeleton for TR 38.817-02 has been approved in [1].

In the past in RAN4, extensive discussions have been done in RAN4 with respect to inband receiver blocking for FR2 NR BS. We have proposed a methodology which considers joint probability of wanted and interferer while considering the receiver blocking for FR2 NR BS. We presented our analysis in [2] and [3].

# Proposal

We propose to adopt the text in this contribution to TR 38.817-02.

# References

R4-1710085, TR 38.817-02 v0.2.0: General aspects for BS RF for NR

R4-1706811, The importance for considering joint probability for receiver blocking in NR BS, Ericsson

R4-1708145, Consideration of joint probability for wanted and interference signals in mmWave NR BS receiver blocking investigation

# Text proposal for TR 38.817-02

# <Start of Text proposal>

## 10.5	OTA In-band selectivity and blocking

Detailed structure of the subclause is TBD.

### 10.5.1 OTA In-band blocking for FR2

#### 10.5.1.1 General methodology

Regarding methods to determine OTA blocking levels, applying the same framework as it is used in eAAS work is adopted. This framework defines a two-step approach:

- A single “conducted” level for the wanted signal and blocking should be decided using the coexistence simulations

- The OTA requirement should be based on the “conducted” level and the eAAS declarations framework.

In NR, beamforming will play an important role in terms of directivity between wanted signals and blocker signals. The directivity of the signals and the positions of the UEs will actually determine the blocker signal levels. In the following figures, we provide joint distribution of relative angles and relative distances of being both the victim UE and blocker UE received at one of base station, in addition to joint distribution of being in certain distance with respect to the victim BS. We provide distributions for both collocated and non-collocated case when urban macro scenario in 30GHz is considered.

In case of NR, beamforming at the transmitter side will ensure that the blocker will affect the receiver only for short while, e.g. for a subframe or a few subframes depending on the scheduling of the blocker and will only impact one or a subset of users during that TTI. This impact will only be seen only when both the victim and blocker UE transmit in similar directions towards the victim BS.

The figures below provide a good indication on how the victim and blocker UE locations with respect to the NR BS impact the actual interference level seen in the BS receiver.

Figure 10.5.1-1	Joint distribution of angle and distances between victims and blockers in UMa collocated deployment

Figure 10.5.1-2	Joint distribution of angle and distances between victims and blockers in UMa non-collocated deployment

From the above figures, we can see very clearly that, the potential stronger blockers occur only at certain distance and certain directivity to a level below 1%. It is worth mentioning here that, the above plots do not indicate the blocker levels, however only indicates potential blockers at certain distance and angle distribution.

Agreement: 99% joint blocker probability is used for blocking investigations.

Both wanted and blocker signals are simulated at once by considering signals coming from every direction, however the calculation of blocker level is taken care of by considering blocker and wanted signal probabilities. These two probabilities are connected via the join probability definition as described below. The following steps for determining the joint probability of receiver blocking levels are followed:

- Use a joint blocker probability (P1) of 1% considering agreed probability of blocking as 99%

- Find out the probability that the wanted signal being below REFSENS+6dB (P2)

- Find the interference level for which P3 is satisfied, where P3=P1/P2.

#### 10.5.1.2 “Conducted” simulation results

#### 10.5.1.3 Conversion of “conducted” simulation results to OTA levels

### 10.5.2 OTA In-band selectivity for FR2

# <End of Text proposal>